Miner 2049er Commented

A commented disassembly of Bill Hogue’s 1982 cartridge for the Atari 400/800

The game’s own credits screen, at the end of its opening demo.

Big Five Software, 1982  ·  16K cartridge  ·  6502 machine language

October 2026

# Contents

Introduction	4

What is in this package	4

How the work was checked	5

The Atari 400	6

Origins	6

Inside the machine	6

The Atari 5200	7

Why the hardware mattered to Miner 2049er	7

Miner 2049er and Big Five Software	8

The ten stations	8

Reading the listing	10

Architecture	11

Memory map	11

One ROM, two machines	11

Program flow	12

The vertical blank, step by step	13

The screen	14

The display list	14

Pixels	14

Colours	15

Text in the bitmap	15

Building a station	16

Girders and sections	16

Station machinery	16

Overlapping data	17

Bounty Bob	18

Three players, three colours	18

The movement state machine	19

Claiming the floor	20

Mutants	22

Two sprites, eight mutants	22

Behaviour	23

Items, scoring and time	24

Items	24

Score	25

The bonus timer	25

The end of a station	25

Station machinery in detail	26

Transporters (stations 3 and 7)	26

The lift (station 8)	26

Pulverizers (station 9)	26

The cannon (station 10)	26

Title screens and the attract sequence	28

The credits	28

The high-score screen	28

Other screens	31

The phone-number cheat	31

Sound and music	33

Tricks lost to time	34

Oddities and bugs	35

Graphics gallery	36

Sources	40

Appendix A: the complete listing	41

# Introduction

Source, corrections and issues: https://github.com/jeffadickson/BigFive-Miner2049

Miner 2049er came out for the Atari 400 and 800 at the end of 1982 on a 16-kilobyte cartridge, twice the size of a standard Atari cartridge of the time. It was written by one programmer, Bill Hogue of Big Five Software, entirely in 6502 machine language. This document is a guided tour of that cartridge, reconstructed by disassembling the ROM.

The game itself is simple to describe. Bounty Bob, a Canadian Mountie, must walk over every section of every girder in an abandoned uranium mine while radioactive mutants patrol the levels. Picking up the prospecting tools left behind turns the mutants green, and for a short time he can eat them. There are ten "stations", each with its own gadget: slides, transporters, moving "lillipads", a lift, pulverizers and finally a cannon fired with TNT.

What makes the program remarkable is how much it fits into 16K, and how hard it pushes the hardware. Among the things described in the chapters that follow:

- One ROM, two machines. The same 16K image boots on the Atari 400/800 and on the Atari 5200 SuperSystem. At power-up it checks whether the 5200's character set is at $F800, then builds a table of pointers to the graphics and sound chips, which sit at different addresses on the two machines. Every later access to those chips goes through the table. I confirmed this by booting the unmodified file as a 5200 cartridge in an emulator.

- Ten colours on a scan line. The high-score screen's "chasing lights" use a cycle-counted routine that runs from RAM. It rewrites all four playfield colour registers twice on every scan line and patches its own opcodes to stay in step with the fine scroll.

- Eight mutants, two sprites. Display-list interrupts cut the screen into four bands and reuse player 3 and the four missiles in each band. The hardware collision registers are read band by band, so the game knows which mutant touched Bob.

- A game that runs in the interrupt. Almost all of the game, from Bob's movement to the mutants and the timer, runs in the vertical-blank interrupt. The main program only waits.

- Data packed into data. Tables overlap their neighbours wherever their bytes happen to agree. A display list's jump address doubles as two characters of text, the phone number printed in the credits is also the cheat code, and the lift's wheels are also a bonus value.

None of the original source survives. Every name, comment and explanation here was worked out from the machine code. Where the evidence is indirect, the text says so.

## What is in this package

- Miner 2049er Commented (this document). It covers the Atari 400 and its background, the program's architecture, its algorithms and its graphics, and ends with the complete listing in an appendix.

- miner2049er.asm, the commented source: about 9,200 lines for the ca65 assembler (part of the free cc65 suite). It assembles to an image that is byte-for-byte identical to the original cartridge. Every routine has a name and a header describing what it does, and every subroutine call is commented with what the called routine does.

- cart.cfg, the linker configuration needed to rebuild the ROM.

## How the work was checked

Three kinds of checking were used. First, the commented source is reassembled after every change and compared with the original ROM, which proves that no byte was misread. Second, the cartridge was run in the Atari800 emulator, both as a 400/800 cartridge and as a 5200 cartridge. Third, a small 6502 harness written for this project runs the ROM so that memory can be inspected, stations can be built on demand and claims can be tested. For example, Bob jumping at the start of station 5 touches the goblet and is killed; that was confirmed this way. The station screenshots were taken in Atari800 using the game's own cheat code. Atari's original ROMs were not used: the screenshots show the emulator's built-in replacement firmware (AltirraOS), so the letter shapes differ slightly from a real Atari's.

# The Atari 400

## Origins

Atari announced its home computers in December 1978, showed the Atari 400 and 800 at the Winter Consumer Electronics Show in January 1979, and shipped them in November 1979. The 400 started life as "Candy", a low-cost design. Early in 1979 it became a full computer aimed at children and families, which is why it has a flat, spill-proof membrane keyboard instead of the 800's full-travel keys. Joe Decuir, one of its designers, later said the keyboard "didn't feel great, but it worked." Sources give its launch price as $495, $549.95 or $550.

Strict FCC limits on radio interference shaped the design. Both machines are wrapped in cast-aluminium shielding, and rather than expansion slots they talk to their peripherals over a single serial bus (SIO). The 400 has one cartridge slot under a lid on top; the 800 has two. Cartridges, keyed connectors and a sealed case made the machine friendly to people who had never owned a computer.

The 400 was planned with 4K of RAM (hence the name), shipped with 8K, and by 1981-82 was sold with 16K. Miner 2049er needs 16K of RAM. Prices fell steeply: about $400 with BASIC and a cassette recorder in 1981, $349 in 1982, and under $200 after rebates by the end of 1982. The 400 outsold the 800 by roughly two to one. About two million Atari 8-bit computers were sold between 1979 and 1985. Their main rivals were the Apple II, the TRS-80 and, from August 1982, the Commodore 64.

## Inside the machine

The 400 and 800 share their electronics. The processor is a MOS 6502 running at 1.79 MHz (1.77 MHz on PAL machines). What made the Ataris different from their rivals was a set of three custom chips, designed under Jay Miner, who later designed the Amiga's chips:

- ANTIC (Joe Decuir, Francois Michel and Steve Smith) is a processor in its own right. It fetches screen data from RAM by DMA and follows a *display list*, a little program in which each instruction describes one horizontal band of the screen: its graphics mode, where its data comes from, whether it scrolls, and whether it should interrupt the 6502 when it is drawn (a *display list interrupt*, or DLI). Once started it runs with no help from the CPU, stealing memory cycles from it as it goes.

- CTIA/GTIA (George McLeod) turns ANTIC's data into colour. It also provides *player/missile graphics*, Atari's name for sprites: four 8-pixel-wide "players" and four 2-pixel "missiles", each a vertical strip the full height of the screen with its own colour and horizontal position. GTIA records which players, missiles and playfield colours touched each other in *collision registers*. Machines built from late 1981 have GTIA; earlier ones have CTIA.

- POKEY (Doug Neubauer) provides four sound voices, the keyboard scanner, eight paddle (potentiometer) inputs, a 17-bit random-number generator, timers and serial I/O.

A fourth chip, a 6520 PIA, reads the joysticks. The operating system lives in a 10K ROM at the top of memory.

## The Atari 5200

In 1982 Atari repackaged the same chipset as a games console, the 5200 SuperSystem. It shipped in October 1982 at $299.95 and sold just over a million units before it was discontinued in May 1984. David Ahl called it "a 400 computer in disguise". The differences matter to this program:

| Item | Atari 400/800 | Atari 5200 |
| --- | --- | --- |
| ANTIC | $D400 | $D400 |
| GTIA | $D000 | $C000 |
| POKEY | $D200 | $E800 |
| PIA (joysticks) | $D300 | none |
| Joysticks | digital switches | analog pots (POKEY), non-centring |
| Keys | keyboard + START/SELECT/OPTION | 12-key keypad + START/PAUSE/RESET |
| Firmware | 10K OS, font at $E000 | 2K BIOS, font at $F800 |
| Cartridge start | vector at $BFFA, init at $BFFE | vector at $BFFE |

Miner 2049er was the first game from an independent publisher for the 5200, in July 1983. As the next chapters show, the 5200 support is already in the 400/800 cartridge.

## Why the hardware mattered to Miner 2049er

The design of the game follows the hardware closely. Each element maps onto a feature:

- The mine is a bitmap in ANTIC mode E: 160 pixels across, four colours, two bits per pixel, 40 bytes per line.

- Bounty Bob is three players laid over each other, which gives him three colours.

- The mutants are player 3 and the missiles, reused down the screen.

- The cannon is drawn with player 3 and the missiles, and the lift with player 3.

- Collisions with scenery come from GTIA's collision registers.

- The sound effects use all four POKEY voices.

# Miner 2049er and Big Five Software

Big Five Software was founded in 1980 in Van Nuys, California, by Bill Hogue and Jeff Konyu. Hogue had been a Radio Shack salesman and Heathkit builder. Their first products were arcade-style games for the TRS-80, among them Super Nova, Galaxy Invasion, Attack Force, Meteor Mission II and Robot Attack. Big Five also sold the TRISSTICK, an adapter that let TRS-80 owners use Atari joysticks. Defense Command was Hogue's last TRS-80 game. In 1983 he told 80 Micro that Big Five had "officially left" the TRS-80 market: "if we hadn't come out with our Atari game at the time we did, we probably would have gone out of business."

Miner 2049er was that Atari game, released at the end of 1982. Sources say November or December. Hogue built the game play first and added the mining theme and story afterwards. He combined Donkey Kong-style climbing with the Pac-Man ideas of covering the whole maze and of enemies that become vulnerable. The title is a pun on the "forty-niners" of the 1849 California Gold Rush, moved a century into the future. The folk song "Oh My Darling, Clementine" has the line "dwelt a miner, forty-niner", and the cartridge plays that tune after a game ends (see the chapter on sound and music).

The game was a hit. Reviewers praised it: Tom Hudson of ANALOG wrote that it "looks as if it were designed, not just thrown together". It won Electronic Game of the Year at the 1984 Arkie Awards. Licensing was handled by Barry Friedman's International Computer Group. Ports followed for about twenty systems, including the 5200, ColecoVision, Apple II, IBM PC, Commodore 64, VIC-20, TI-99/4A and a two-cartridge Atari 2600 version by Tigervision. It also inspired Matthew Smith's Manic Miner. Hogue's sequel, Bounty Bob Strikes Back!, followed in 1985 on a bank-switched cartridge.

## The ten stations

| Station | Name | What is special | Sections | Bonus (zone 1) |
| --- | --- | --- | --- | --- |
| 1 | The Mine Shaft | ladders and sloping girders | 175 | 3000 |
| 2 | The Slides | chutes that carry Bob down | 144 | 3500 |
| 3 | The Transporters | four numbered booths; press 1-4 | 124 | 4000 |
| 4 | The Lillipads | many small floating girders | 154 | 4500 |
| 5 | Advanced Lillipads | slides, moving platforms, a poisoned goblet | 131 | 5000 |
| 6 | Radioactive Waste | a deadly glowing tank | 83 | 5500 |
| 7 | Advanced Transporters | transporters and long slides | 90 | 6000 |
| 8 | The Lift | a scissor lift Bob drives himself | 114 | 4500 |
| 9 | The Pulverizers | pistons, platforms that wait for Bob | 78 | 6500 |
| 10 | The Cannon | load TNT, fire yourself upwards | 134 | 7000 |

The station names are from Creative Computing's 1983 guide. The section counts and bonuses come from the station data in the ROM. A "zone" is one pass through all ten stations; each new zone adds 500 to every starting bonus and speeds up the mutants.

# Reading the listing

The listing follows a few conventions:

- Names. Routines have upper-case names (for example CLAIM_FLOOR or GAME_VBI). Local labels inside a routine share a short prefix (CF_, MU_ ...). Zero-page variables have names such as BOB_X or SECTIONS. The hardware pointers are named after the register they reach, with a "p" in front: pAUDC3 is the zero-page word that holds the address of POKEY's AUDC3 register.

- Headers. Each routine starts with a header that says what it does, what it takes and what it returns.

- Comments. Inline comments explain the line or block they sit on. Lines whose comment begins with "if", "loop until" or "X = Y + Z" were summarised automatically from short instruction patterns; the rest were written by hand.

- Data. Tables are shown in whichever form is clearest: sprites as bit pictures (# for a set bit, or 0-3 for two-bit pixels), text as screen codes with the decoded string, display lists one instruction per line.

Positions use the game's own conventions:

- Horizontal positions are in player/missile coordinates, where the playfield's left edge is 48 ($30). Bitmap routines subtract $30 to get the pixel column 0-159.

- Vertical positions are bitmap lines 8-191, counted so that bitmap line y is drawn on scan line y + 32, which is also player/missile row y + 32. Bob's BOB_Y is the top of his sprite; his feet are 21 ($15) lines lower.

To rebuild the cartridge:

ca65 miner2049er.asm -o miner.o

ld65 -C cart.cfg -o miner.rom miner.o

The result is identical to the original 16,384-byte image.

# Architecture

## Memory map

| Address | Contents |
| --- | --- |
| $0000-$00FF | zero page: hardware pointer table $10-$6F; game variables $70-$FF |
| $0200-$0223 | interrupt vectors (both the 400/800 and the 5200 layouts are written) |
| $0300-$07FF | player/missile graphics: missiles at $0300, Bob in players 0-2, mutants in player 3 |
| $0800-$08EF | high-score table and the text of the title screens |
| $0917-$0A24 | screen text, item table, mutant records, pulverizers |
| $1C00-$1FFF | second player/missile buffer for the high-score screen; the 5200's font copy |
| $2000-$20FF | game display list and status line |
| $2100-$3DBF | the mine: 184 lines x 40 bytes |
| $3DC0-$3F2F | bitmap line-address tables (LINE_LO/LINE_HI, indexed from y = 8) |
| $3F30-$3F5F | moving platforms |
| $8000-$91F0 | cartridge data: screens, text, sprites, items, station descriptions |
| $91F1-$BFBC | cartridge code |
| $BFBD-$BFFF | tables and the cartridge trailer |

The 16K of RAM the game needs is used almost entirely. The display list, status line and bitmap alone take 7.6K.

## One ROM, two machines

The last six bytes of the cartridge serve both machines:

$BFFA  .word RESTART      400/800: start address

$BFFC  .byte $00          400/800: "a cartridge is present"

$BFFD  .byte $FF          400/800: option byte;  5200: "skip the title screen"

$BFFE  .word COLD_START   400/800: init address;  5200: start address

On the 400/800, bit 7 of the option byte marks a *diagnostic* cartridge. The OS jumps to the init address straight after reset, before it has initialised anything, so the game owns the machine from the first instruction. On the 5200, the BIOS reads $FF at $BFFD as "no title screen" and jumps through the word at $BFFE, which is the same address.

COLD_START then works out which machine it is on. On the 5200, $F800 is the start of the BIOS character set, whose first two characters (space and "!") are made only of the bytes $00 and $18. On the 400/800, $F800 is in the middle of OS code. If all sixteen bytes from $F800 are $00 or $18, the game is on a 5200.

It then builds the hardware pointer table in zero page:

$10 + 2r  ->  GTIA register r     ($D000 + r on the 400/800, $C000 + r on the 5200)

$50 + 2r  ->  POKEY register r    ($D200 + r,               $E800 + r)

From then on, nearly every access to GTIA and POKEY is indirect through the table, STA (pAUDC1),Y with Y = 0, so the same code works on both machines. Only ANTIC, which is at $D400 on both, is addressed directly. The cost is small: an indirect store takes 6 cycles instead of 4 and needs Y = 0. A few places use a well-chosen X with (zp,X) to reach a different register from a nearby pointer. For example, STA ($3A,X) with X = $1E writes AUDF3, because $3A + $1E = $58, which is pAUDF3.

The other differences are handled where they arise:

- Input. On the 5200 the analog stick is read through POKEY's pot inputs and converted into the four direction bits of the 400/800's PIA joystick port (READ_STICK).

- Keypad. POLL_KEYS translates the 5200's keypad codes into the 400/800 key codes the game expects. The # key becomes the space bar, PAUSE becomes ESC and RESET restarts the cartridge.

- Fonts. The text renderer draws from whichever ROM font the machine has (FONTPAGE).

- High scores. On the 400/800 they survive the RESET key, because the OS's warm-start flag tells the game it was not a power-up. On the 5200 there is no such flag, so COLD_START clears them at power-up; the keypad's RESET key goes straight to RESTART, so they survive that too.

The SET_VBI routine shows how far this goes. The 400/800 OS saves A, X and Y before it calls the game's vertical-blank handler; the 5200 BIOS does not. So on the 5200 the game points the BIOS vector at a six-instruction stub, VBI_5200, which saves the registers itself and continues through the handler. Every handler can then end with the same NMI_EXIT on either machine.

## Program flow

The cartridge cycles through these screens:

power-up / RESET

COLD_START  ->  RESTART

credits (station 11, recorded demo)   --START-->  game

game-over tune (Clementine)

high-score screen   ---- 30 s --->  demo of station 1

--START-->  game

game:  START_GAME -> PLAY_STATION:  prepare screen -> station -> next station / next player

out of lives -> "Game Over" -> name entry -> high-score screen

PLAY_STATION draws a station behind the "MINER 1 / PREPARE FOR / STATION 1 / ZONE 1" screen. The station is drawn pixel by pixel and takes a while, so the three-second prepare screen hides it. When the countdown ends, START_STATION switches the display to the game screen and installs two handlers:

- GAME_VBI, the vertical-blank handler. It runs the whole game once per frame.

- DLI_MUTANTS, the display-list interrupt. It runs four times per frame and manages the mutants.

The main loop, GAME_LOOP, has almost nothing to do. It polls the console keys (OPTION, START, and digit keys during the demo), keeps the TNT readout up to date on station 10, and waits for one of two events:

- the turn ends (ALIVE reaches 0), after which TURN_OVER decides what comes next;

- the end-of-station wipe finishes (WIPE_LINE reaches 0), after which the next station is built.

Running everything in the interrupt keeps the game's speed steady whatever the station looks like. The game drops no frames, because there is no frame loop to fall behind.

## The vertical blank, step by step

GAME_VBI is a long routine that does the following, in order:

- Read the keyboard (POLL_KEYS), count the frame, reload the display list pointer, and place the bonus box (the four missiles, double width, behind the bonus digits).

- Read the collision registers for the bottom band, set the status-line colours, and read the 5200 stick.

- If an item was just picked up, turn every living mutant edible.

- If the bonus is under 700, flash the bonus box, make Bob's face flicker and play a ticking sound that speeds up as time runs out.

- Handle pause.

- Make the current player's score blink (it is written with bit 7 set, and CHACTL bit 0 blanks such characters).

- Check whether Bob touched a deadly colour-1 object: the waste on station 6, the moving bar on station 8, a pulverizer on station 9.

- If Bob is dying, make him throb (the players change width) and play noise.

- Play the extra-life jingle.

- Move Bob (MOVE_BOB) and the moving platforms (UPDATE_LIFTS), set Bob's sprite position and size, claim girder sections under his feet (CLAIM_FLOOR), then move the pulverizers.

- Count down to the end of the turn after a death.

- On station 11 only: run the credits reveal and skip straight to step 16.

- Redraw the status line, update the items, run the bonus timer, and count the bonus into the score and wipe the screen once the station is clear.

- Add points for an eaten mutant.

- Check the phone-number cheat.

- In VBI_TAIL: advance the demo recording, run the cannon or the lift, and return.

# The screen

## The display list

BUILD_DLIST writes the game's display list at $2000:

$2000  3 x "8 blank lines"

$2003  mode 2, LMS $20D8, DLI       status line, 40 text characters

$2006  184 x mode E                 the mine, 160 x 184, four colours

lines 46, 93, 140 set DLI    band boundaries for the mutants

line 96 has LMS $3000        ANTIC cannot cross a 4K boundary

$20C0  JVB $2000

ANTIC's address counter wraps within 4K. A bitmap that starts at $2100 would run past $3000 on its 97th line, so that line reloads the address explicitly. It is needed only because the bitmap is 7,360 bytes long.

The game screen with its display-list interrupts. Each band shows two mutants: one in player 3, one in the four missiles.

## Pixels

In ANTIC mode E each byte holds four pixels, two bits each, leftmost pixel in the top bits. A pixel value of 0 is the background; 1, 2 and 3 select the colour registers COLPF0, COLPF1 and COLPF2. Miner uses them consistently:

| Value | Register | Used for |
| --- | --- | --- |
| 0 | COLBK | background (black) |
| 1 | COLPF0 | items, text, deadly objects (waste, crusher bar, pulverizers) |
| 2 | COLPF1 | ladders, slides, points numbers |
| 3 | COLPF2 | girders, platforms |

How a mode E byte maps to pixels, and the tables used to set them.

PLOT and its variants find the byte as LINE_LO/HI[y] + x/4, where the two 184-entry tables give the address of each bitmap line. They then use the pixel's position within the byte, x AND 3, to index two small tables. PIXEL_MASK ($C0 $30 $0C $03) isolates a pixel's two bits. PIXEL_SET supplies the bits for a colour, and is indexed by PLOT_COLOR + (x AND 3), where PLOT_COLOR is 0, 4 or 8 for colours 1-3 and 12 for the background. The tables overlap: colour 3's entries are PIXEL_MASK itself, and the four zero bytes after it give the background. PLOT_IF_EMPTY draws only on background pixels, which is how ladders and slides avoid painting over girders. READ_PIXEL returns a pixel's colour and sets the Z flag if it is 3, which makes "is there a girder here?" a single subroutine call.

## Colours

Each turn, START_STATION picks the three playfield colours at random, with two rules: the three hues must differ, and so must the three brightness values, which are taken from {4, 6, 8}. The second rule keeps items, ladders and girders apart on a black-and-white television. Every play of a station therefore looks different, as the screenshots show.

The status line takes its colours from GAME_VBI. The first display-list interrupt then sets colours 2 and 3 for the mine below it (and, on the transporter stations, colour 1 as well).

## Text in the bitmap

The game uses two kinds of text. Fixed screens such as the title, prepare, name-entry and high-score screens use ANTIC character modes. Text inside the mine ("TNT Charge= 0 Tons", "DANGER", the credits, "Game Over") is drawn into the bitmap pixel by pixel by DRAW_CHAR. DRAW_CHAR reads the machine's ROM font, either at $E000 or at $F800 on the 5200, and can double the height. It paints 1 bits in the ink colour and clears 0 bits, so text erases whatever is behind it.

# Building a station

A station is described entirely by data. STATION_TABLE gives the address of four lists for each station, and four more tables give the items, the mutants, Bob's starting position and the transporter column:

| List | Entry | Meaning |
| --- | --- | --- |
| girders | sections, x, y | three sub-lists: sloping down, sloping up, flat |
| ladders | height, x, y, style | 8 pixels wide, a rung every 4 lines |
| slides | rows, x, y, step | an 8-pixel band shifted "step" pixels per line |
| moving platforms | timer, width, x, y, travel, speed | a 3-line bar that moves back and forth |
| items | shape+$80, y, x | a picture copied byte by byte into the bitmap |
| mutants | 8 records of 11 bytes | start position, patrol, speed, colour timing |

## Girders and sections

The unit of the game is the girder *section*, 4 pixels wide. DRAW_GIRDERS draws each section as three thin stripes of colour 3, two lines apart, and counts it into SECTIONS. On sloping girders each section is one line higher or lower than the one before. On flat girders, sections alternate between two stripe patterns, which gives them a brickwork look. That alternation turns out to be essential to how claiming works (see "Claiming the floor").

Part of station 1, magnified: unclaimed sections (striped brickwork) on the left, sections Bob has claimed (solid) on the right.

## Station machinery

Station-specific set-up code adds the rest:

- Stations 3 and 7: transporters. DRAW_TRANSPORTER draws the column of four booths from an 8-bit run-length pattern (BOOTH_SHAPE), each bit two pixels wide, and writes the digits 1-4 in them with the font renderer.

- Station 6: the waste tank. DRAW_WASTE draws a solid colour-1 box and cuts "DANGER / Radioactive / Waste" into it in the background colour.

- Station 8: the lift. The lift's platform, scissor arms and cart are drawn into player 3, and its track into the bitmap.

- Station 9: the pulverizers. SETUP_PULVERIZERS draws the beam and posts and gives the four heads random starting delays.

- Station 10: the cannon. The cannon goes into player 3 plus the missiles. The TNT store is drawn with its shelves and sign.

The machinery, drawn from the ROM's data: a transporter booth, a pulverizer head, the lift and the cannon.

## Overlapping data

The data area is full of sharing:

- A list's 0 terminator is often the first byte of the next list.

- Station 1's item list ends with the 0 that also serves as "no ladders, no slides, no platforms" for every station that lacks them.

- Entry 0 of STATION_TABLE is also ITEM_SHAPES 33-36, the goblet and the three TNT bundles.

- Station 10's eighth mutant record runs ten bytes into station 1's girder list. That is harmless, because the record's first byte, 0, means "no mutant".

- Several girder lists start inside the slide list before them.

Because the generated listing follows the ROM byte by byte, these overlaps show up as labels that sit in the middle of what looks like another table.

# Bounty Bob

## Three players, three colours

Bob is drawn with players 0, 1 and 2 laid over each other: blue ($8A) for his coat, orange ($3A) for his face and hands, white ($0F) for his legs and the stripe down his front. PRIOR is set to $31: players in front of the playfield, with *multicolour* mode on. In multicolour mode, where players 0 and 1 overlap, GTIA ORs their colours, so the hat comes out a fourth colour, $BA (green). Player 2 shows only where the other two are empty.

Frame 1 taken apart: the three players and the combined sprite.

Each animation frame stores the three layers back to back, 20, 17 and 17 lines, with each layer's first line shared with the previous layer's last: 52 bytes, stored 51 bytes apart, so a frame's last byte is also the next frame's first. DRAW_BOB copies them into player memory from the bottom up, and the blank lines at the ends erase his previous position as he moves.

Bob's twelve animation frames (frame 0, standing, is the same data as frame 1).

## The movement state machine

MOVE_BOB is called once per frame and branches on Bob's state:

| State | Condition | Routine |
| --- | --- | --- |
| dying | SPECIAL = 1 | MELT_BOB |
| dead | SPECIAL = 2 | nothing; GAME_VBI ends the turn |
| transporting | SPECIAL = 3-7 | TRANSPORT |
| falling | BOB_FALL | FALLING |
| sliding | SLIDE_LEN | SLIDING |
| jumping | frame 11-12 | JUMPING |
| climbing | frame 7-10 | ON_LADDER |
| on a girder | otherwise | button, stick, ladders, walking, booths |

Walking moves Bob one pixel every other frame and cycles through three frames. After each step, the routine tests the bitmap under his feet with READ_PIXEL:

- girder one line above his feet: the girder rises, so he steps up a line;

- girder under his feet: nothing to do;

- girder one line below: the girder falls, so he steps down a line;

- no girder at all: he has walked off the edge and starts to fall.

Bob follows sloping girders this way with no slope data at all: the bitmap is the collision map.

Ladders. TRY_CLIMB_UP looks for a ladder whose foot is level with Bob's feet or one line below them and within two pixels of his x, lines him up with it, and starts the climb. MOVE_A and MOVE_B count the lines to the top and to the bottom, and reaching either end puts him back on his feet. Frames 9 and 10 show him stepping on or off at the top.

Jumping. START_JUMP takes the direction from the stick. Each frame JUMPING moves Bob vertically by the next entry of JUMP_ARC: 11 lines up, a pause, 3 up, a pause, 2 up, then down. Sideways he moves one pixel every other frame, unless a wall stops him. After the table runs out he falls one line per frame until LAND_CHECK finds a girder.

Height of Bob's jump frame by frame, from JUMP_ARC.

Falling is counted in FALL_LEN, one per frame. Forty frames of falling kill him; because a jump also counts its descent, a drop of about 18 lines below the take-off point is already fatal. When he lands, LAND_CHECK makes sure his feet are on the top stripe of the girder, not inside it.

Slides. If Bob stands exactly at the top of a slide, he rides it: one line down and "step" pixels sideways per frame, with a falling whistle.

Dying. Bob throbs, his players switching between normal, double and quadruple width with an offset that keeps him centred, and at the same time he melts (if he dies in the air, he melts when he lands): MELT_BOB feeds random bytes in at his feet and shifts each player's column down a line per frame until he is a puddle of noise.

## Claiming the floor

CLAIM_FLOOR is the heart of the game. It runs every frame while Bob walks or stands:

- FEET_OFFSETS gives the x positions of his two feet for the current animation frame. For each foot, the routine reads the pixel at the feet line. It must be girder (colour 3).

- It reads the next two lines down. An unclaimed section has gaps between its stripes. If the line under the feet is a gap, the gaps are at +1, +3 and +5; if not, they are at +2 and +4. If both lines are solid, the section is already claimed and there is nothing to do.

- It finds the section's left edge by scanning along a gap row. A neighbouring section's stripes are at different heights, either because it belongs to the other brickwork pattern or because it is a step higher or lower on a slope, so where the gap row turns solid is the boundary.

- It fills the gap rows of those four pixels with colour 3. The section turns solid, SECTIONS decreases by one and the score goes up by 5.

So the bitmap is not just the picture of the mine. It is also the record of what has been claimed. There is no array of section flags, which saves both memory and time. The brickwork pattern of the flat girders is what makes the boundaries findable. The station is complete when SECTIONS reaches zero.

# Mutants

## Two sprites, eight mutants

Each station has up to eight radioactive mutants, but the hardware has only player 3 and the missiles free once Bob has used three players. Miner divides the screen into four horizontal bands with display-list interrupts: one at the end of the status line and three in the bitmap. Each band shows two mutants: one drawn with player 3, the other with the four missiles placed side by side in "fifth player" mode (PRIOR bit 4), which makes them an 8-pixel sprite coloured by COLPF3. At each band boundary, DLI_MUTANTS re-positions the two sprites and sets their colours for the band below. The player and missile memory holds all eight shapes at once, each at its own height, so only positions and colours need to change mid-screen.

DLI_MUTANTS also does three other jobs:

- Collisions. At the start of each band it saves the collision registers for the band that just ended, then clears them (READ_HITS). Bob is players 0-2, so a collision bit 0-2 in the missile-to-player or player-3-to-player registers means that band's mutant touched him. Reading and clearing per band is what lets two sprites stand in for eight mutants and still tell which one hit Bob.

- Movement. It moves and animates two mutants per interrupt, spreading the work over the frame. The mutant it moves is not the one it is currently positioning; the indexing is arranged so that each record is updated once per frame.

- Band effects. It sets the transporter booths' colour (green, or red while the transporter recharges), positions the cannon (station 10) or the lift (station 8), which borrow the same sprites in their own band, and makes colour 1 flicker in the bottom band on stations 5, 6, 8 and 9. On station 6 that makes the radioactive waste glow.

The ten mutant shapes. The "edible" pair raise their arms; an eaten mutant becomes the number of points it was worth.

## Behaviour

Each mutant patrols back and forth along a girder. Its station record gives:

- its height and starting position;

- the length of its patrol and its speed (frames per step);

- the timing of its shimmer (its brightness pulses up and down by 2).

The step delay is reduced by the zone number, so mutants get faster as the game goes on.

When Bob picks up an item, ITEM_GOT is set, and on the next frame GAME_VBI turns every living mutant edible. Each turns green ($C6) with an edible counter of $AF - 16 x min(zone, 9): 159 in zone 1, falling to 31 from zone 9 (the subtraction runs with the carry clear, so it is one less than $B0 - 16n). It stays green for that many frames, then blinks: purple for 5 frames, green again for half the previous time, and so on until the counter reaches zero. The blinking speeds up as a warning. In zone 1 a mutant stays edible for about six seconds; from zone 9 on, about one and a half. Touching an edible mutant eats it for 80 or 90 points, chosen at random, and the mutant shows the number, flashing in random colours, while a rising tone plays for about two seconds. Touching a mutant that is not edible kills Bob.

# Items, scoring and time

## Items

Items are small pictures stored as whole bytes (four pixels per byte) and copied straight into the bitmap. Each has a width, a height and a points value. When Bob touches one, UPDATE_ITEMS does five things:

- draws the points number over the item ("100" ... "500"; the points byte doubles as the shape number of the number picture);

- erases the rest of the item;

- adds points x 100 to the score;

- sets ITEM_GOT, which turns the mutants edible;

- plays a falling whistle.

The "BF" (Big Five) logo on station 7 is worth 1,100.

On station 10 the items are bundles of TNT. Their points value, 1 to 3, is also the number of tons added to the cannon's charge.

On station 5 one item is a trap. UPDATE_ITEMS treats the goblet (shape 33) specially there: touching it sets KILLED. The goblet sits just above Bob's starting place, so a jump straight up at the start of the station kills him. This was confirmed in the emulator.

All 36 item shapes in ITEM_SHAPES, with their points. Shapes 1-5 and 11 are the points numbers themselves.

## Score

The score is six decimal digits, one per byte. ADD_SCORE adds a value to any digit and carries upwards. Points come from:

| Event | Points |
| --- | --- |
| claiming a section | 5 |
| item | 100-500 (BF logo 1,100) |
| eating a mutant | 80 or 90 |
| bonus left at the end of a station | counted in at 100 a tick |

When the score first reaches 10,000, Bob gets an extra life. The award is detected by the ten-thousands digit becoming non-zero while XLIFE still holds its initial value, 10. A short jingle then plays while the screen background flashes blue.

## The bonus timer

Each station starts with a bonus of BONUS_BASE + 500 per zone (at most 9,900), shown in a box on the status line. The box is the four missiles, set to double width. The bonus drops by 100 every three seconds. Below 700 the box flashes, Bob's face flickers and a tick plays faster and faster. If the bonus reaches zero before the station is clear, Bob dies.

## The end of a station

When the last section is claimed:

- Bob vanishes.

- TALLY counts the bonus into the score 100 at a time, with a click each time.

- WIPE removes the screen from the bottom up. Each frame it overwrites the next display-list line with a JVB (jump and wait for vertical blank), so ANTIC simply stops drawing there, and a rising tone plays.

- When the wipe reaches the top, the main loop builds the next station.

# Station machinery in detail

## Transporters (stations 3 and 7)

To use a transporter, Bob stands still inside a booth and presses a digit key 1-4. The transporter runs as a five-state sequence in TRANSPORT:

- Bob dissolves into sparkles: SPARKLE_BOB ANDs every line of his sprite with a random byte. A random buzz on voice 3 grows louder.

- A short pause follows.

- Bob appears at the destination booth, while sparkles linger at the old booth and his colours flash in both booths.

- He sparkles in place at the new booth.

- He re-forms as the buzz fades.

Afterwards the transporter is locked for five steps of 64 frames, about five seconds (128 frames each from zone 5 on, about ten seconds), and the booths are red until it recharges. They are also red, briefly, when the station starts.

## The lift (station 8)

Standing on the lift and pressing the space bar (the # key on a 5200) hands the joystick to the lift. The stick then drives it left and right along its track and up and down. It rises at half the speed it falls. Bob rides along, and pressing space or the button again gives control back.

The lift is player 3 in the band that contains it. As it moves up or down, LIFT_CONTROL rewrites the top of its column of player data: it clears the line above, writes the two-line platform and one more line of the scissor pattern. The arms seem to extend and fold because the pattern's phase steps through a seven-line cycle.

## Pulverizers (station 9)

Four heads hang from a beam. Each waits, drops 22 lines, rises again and waits for a delay that shrinks with the zone. Each head's top line is a thin rod. As the head descends, that line is left behind each frame, so the rod grows down from the beam with no extra drawing code. The heads are drawn in colour 1, and touching colour 1 on this station is fatal. Station 9's moving platforms behave as lifts: the code holds each one at the end of its run until Bob stands on it.

## The cannon (station 10)

Bob collects TNT bundles worth 1-3 tons each, then steps into the cannon's mouth. The cannon then follows him along its track, and the button fires it:

| Charge | Result |
| --- | --- |
| 1 ton | Bob reaches the first platform |
| 2 tons | the second platform |
| 3 tons | the third platform |
| 4 tons or more | he is shot off the top of the screen and falls back upside down |

With 4 tons or more loaded, the word "DANGER" appears on the screen as a warning. After such an over-charged shot, DRAW_BOB_FLIPPED draws Bob upside down for the rest of the turn, by copying his sprite top-to-bottom instead of bottom-to-top. SHOW_TNT keeps "TNT Charge= nn Tons" up to date on the screen.

# Title screens and the attract sequence

## The credits

After power-up the game plays station 11, a station of plain girders with no mutants, which hides the credits. The credits are drawn at the start in pixel colour 1, whose register is set to black, so they are invisible. Bob, driven by the recording DEMO_CREDITS, walks the girders. Every frame GAME_VBI looks at a one-pixel column 30 lines tall through him and repaints every colour-1 pixel in colour 2, so the words appear behind him as he passes. When he reaches the bottom right, colour 1 is made visible, the full text appears, and a display-list interrupt (DLI_RAINBOW) runs alone, with no vertical-blank interrupt, striping the text with a scrolling rainbow while Clementine plays.

The credits being revealed as Bob walks.

The finished credits. The last line is the cheat code.

## The high-score screen

The high-score screen surrounds the table with a frame of 36 "5" characters whose colours march round it like theatre lights. The effect combines most of the techniques in the program:

- The rows never move. The top and bottom rows are single lines of ANTIC mode 7 text kept in ROM. HSCROL slides each row by 0-15 colour clocks, which is two character cells. Each time the scroll wraps, the colours rotate one light, so the eye sees continuous motion.

- Ten colours on a line. Mode 7 gives four colours per line. A display-list interrupt jumps into TITLE_KERNEL, a 62-byte loop copied to RAM at $0400. On each of the row's 12 scan lines it writes all four playfield colour registers for the left four lights, then all four again for the right four, while the beam is half-way across. The two middle lights are players 1 and 2, which have their own colour registers: 4 + 2 + 4 = 10.

- Cycle counting. The kernel has one WSYNC before the first line and none inside the loop. Each pass is about 76-78 CPU cycles; with the cycles ANTIC steals for its own memory accesses on a mode 7 line, that is meant to fill a scan line. To follow the fine scroll, SET_FINE_SCROLL patches two opcodes, one before the loop and one in it, to CPX zp, CMP zp,X or ASL zp, which take 3, 4 or 5 cycles, so the colour changes move by 6, 8 or 10 colour clocks.

- Patched for both machines. Every frame, KERNEL_COLS_TOP and KERNEL_COLS_BOTTOM write the colours into the kernel's eight LDA # operands, and KERNEL_COLS_TOP also rewrites the high byte of each STA COLPFn with the GTIA page taken from the pointer table, $D0 or $C0, because the kernel has to use absolute addresses to stay fast enough. That is why it must run from RAM.

- Side columns. Players 0 and 3, in double-line resolution, form the side columns. A display-list interrupt on every text line recolours them. They move by redrawing into a second player/missile buffer and switching PMBASE between $0400 and $1C00, which is double buffering.

- Smaller effects. The ten score lines get a moving rainbow from the same interrupts, "HIGH SCORES:" pulses in brightness, "START" blinks in inverse video, and a soft tone repeats.

The display list itself is in ROM ($8000). Its final JVB instruction's address bytes are also the first two characters of the bottom row of lights, so the jump target is nonsense. That is harmless, because TITLE_VBI reloads the display list pointer every frame.

Who draws which light on the high-score screen.

One pass of TITLE_KERNEL.

Thirty seconds after the screen appears, unless a game has been started, the demo begins: station 1 played from the recording DEMO_STATION1. Each recording is a list of (input, frames) pairs, where the input byte has the joystick in the low four bits and the button in bit 4. READ_STICK and READ_BUTTON read from the recording instead of the hardware whenever DEMO_PTR is set.

## Other screens

"Prepare for Station": the background flashes and a siren plays while the station is drawn behind it.

"Game Over / Miner 1" is cut into a box that is filled column by column with a rising two-tone sound.

Name entry: up to eight characters, typed on the keyboard or picked with the 5200 stick. The missiles form the cursor and the background is a rainbow from a DLI.

## The phone-number cheat

During a game, typing Big Five's telephone number, 213-782-6861, then SHIFT plus a digit, warps to that station (0 means station 10). PHONE_CODE compares each digit typed with the bytes of the number as printed at the end of the credits text, "(213) 782-6861", skipping the bracket, the space and the dash. The printed text and the cheat code are the same bytes. A wrong digit starts the count again, and a held key repeating the same digit is ignored. On the 5200, the lower fire button plays the part of SHIFT.

# Sound and music

POKEY's four voices are used for different effects:

| Voice | Used for |
| --- | --- |
| 1 | game-over tune melody, siren on the prepare screen, death noise, time-running-out tick, title tone |
| 2 | game-over tune harmony, mutant being eaten, name-entry clicks |
| 3 | footsteps, jumps, falls, slides, transporter buzz, lift motor, cannon |
| 4 | item whistle, extra-life jingle, bonus tally clicks, screen-wipe tone |

GAME_OVER_TUNE plays "Oh My Darling, Clementine" three times: on voice 1, then on voice 2, then on both together. The tune table holds the melody as POKEY dividers ($28 C, $35 the G below, $1F E, $1A G, $1D F, $23 D, $2A B) with lengths in frames. Voice 2 does not read the table; its note is calculated from the melody note as (note - $1D) x 4 + $79, which places it roughly two octaves lower, and a single special case covers note $1A. Each note starts at full volume with a pure tone and decays one step per frame, which sounds like a plucked instrument. START, the button or a digit key cuts the tune short.

The footsteps are a 14-frame pattern on voice 3: a low thud, silence, a click, silence.

# Tricks lost to time

Many of the techniques in this program were common knowledge among Atari programmers in 1982 and are now mostly forgotten:

- A pointer table instead of fixed addresses lets one binary drive chips at different addresses on two machines.

- A diagnostic cartridge owns the machine from the first instruction.

- Self-modifying code for timing: opcodes chosen for their cycle counts, patched into a scan-line loop.

- Self-modifying code for portability: the high bytes of absolute stores patched with the chip's page at run time.

- Mid-line colour changes go beyond the hardware's colours-per-line limit.

- Sprite multiplexing with display-list interrupts, and collision registers read per band.

- Fifth-player mode, which turns the four missiles into one more sprite.

- Double-buffered player graphics by flipping PMBASE.

- The screen as the database: the girders' claimed or unclaimed state lives only in the bitmap.

- A display list rewritten on the fly to wipe the screen line by line.

- Hidden text revealed by recolouring pixels whose colour register is set to black.

- Harmony calculated from the melody instead of stored.

- Tables that overlap their neighbours to save bytes.

- Analog joystick input converted to digital directions, so one input routine serves both machines.

# Oddities and bugs

A harmless pointer bug. When Bob lands safely from a jump or fall, LAND_CHECK means to write 0 (silence) to AUDC3 with the same STA (pAUDF3+1,X) that START_DEATH, a few lines earlier, uses to start its death tone. START_DEATH has X = 1 at that point, which makes the pointer pAUDC3. LAND_CHECK has X = 0, so the 6502 forms an address from bytes $59 and $5A: the high byte of the AUDF3 pointer and the low byte of the AUDC3 pointer. The result is $05D2 ($05E8 on the 5200): row $D2 of player 1, which belongs to Bob's sprite only when he is at that height, and is redrawn whenever he is (on the 5200, row $E8 is below the visible screen). Voice 3 is left as it was and is set again by the footstep code a frame later, so nobody would ever notice.

Filler that does nothing. BUILD_STATION and SETUP_ITEMS contain short sequences with no effect, such as LDA #$4C / LDA $A546. $4C is the JMP opcode, so these may be places kept free for a patch. COLD_START has three NOPs, and UPDATE_ITEMS starts with a sequence that compares a ROM byte with itself minus 5 and ignores the result, padded with NOPs. That looks like a check that was patched out. The evidence for either reading is only the shape of the code.

Tables read past their ends. The bonus for station 10 is read from one byte past the end of BONUS_BASE, which is the first byte of the next table. Station 11 reads its starting x and its transporter column from one past the end of their tables in the same way. The values happen to be the ones wanted.

The poisoned goblet. Station 5's goblet kills Bob on contact. It is hard to know whether this was meant as a trap or as a protection against something; it is a deliberate test in the code, not a side effect.

The credits station's mutant pointer. MUTANT_LISTS has an entry for station 11 that points into RAM ($1E18), because that word is really the first two bytes of the mutant shapes. The 88 bytes it points to are read, but through a mask of zero, so the credits get no mutants.

# Graphics gallery

The ten stations below are screenshots from the Atari800 emulator, reached with the cheat code. Colours are chosen at random each time a station starts, so they will differ from play to play.

Station 1, The Mine Shaft.

Station 2, The Slides.

Station 3, The Transporters.

Station 4, The Lillipads.

Station 5, Advanced Lillipads.

Station 6, Radioactive Waste.

Station 7, Advanced Transporters (the "BF" logo is an item worth 1,100).

Station 8, The Lift.

Station 9, The Pulverizers.

Station 10, The Cannon.

# Sources

Facts about the Atari hardware, its history and the game's history come from these sources. Everything about how the program works comes from the disassembly itself.

- Atari 8-bit computers. Wikipedia. https://en.wikipedia.org/wiki/Atari_8-bit_computers

- Atari 400. Wikipedia. https://en.wikipedia.org/wiki/Atari_400

- ANTIC; CTIA and GTIA; POKEY. Wikipedia. https://en.wikipedia.org/wiki/ANTIC

- Atari 5200. Wikipedia. https://en.wikipedia.org/wiki/Atari_5200

- Claus Buchholz, "Converting to 5200", A.N.A.L.O.G. Computing #15, January 1984. https://atarimuseum.ctrl-alt-rees.com/videogames/consoles/5200/conv_to_5200.html

- oldcomputers.net, Atari 400. https://www.oldcomputers.net/atari400.html

- Low End Mac, "A history of Atari's 8-bit personal computers". https://lowendmac.com/2015/a-history-of-ataris-8-bit-personal-computers/

- Miner 2049er. Wikipedia. https://en.wikipedia.org/wiki/Miner_2049er

- Big Five Software. Wikipedia. https://en.wikipedia.org/wiki/Big_Five_Software

- Owen Linzmayer, "Atari strikes gold", Creative Computing, February 1983. https://www.atarimagazines.com/creative/v9n2/128_Atari_strikes_gold.php

- John J. Anderson, "Mastering Miner 2049er", Creative Computing Video & Arcade Games, Fall 1983. https://atarimagazines.com/cva/v1n2/miner.php

- "Miner 2049er" and "Interview with Bill Hogue", trs-80.org. https://www.trs-80.org/miner-2049er.html and https://www.trs-80.org/interview-bill-hogue.html

- Atarimania, Miner 2049er (the cheat code). https://www.atarimania.com/game-atari-400-800-xl-xe-miner-2049er_3410.html

- Oh My Darling, Clementine. Wikipedia. https://en.wikipedia.org/wiki/Oh_My_Darling,_Clementine

# Appendix A: the complete listing

This is miner2049er.asm in full. Labels are shown in bold; for jsr and jmp the target is highlighted, and its comment says what the called routine does. Routine headers are set between rules. Assembled with ca65 and linked with the cart.cfg shown in the file header, it reproduces the original 16,384-byte ROM exactly.

; ============================================================================

;  MINER 2049er  -  Big Five Software, 1982.  Designed and programmed by Bill Hogue.

;

;  Commented disassembly of the 16K cartridge ROM ($8000-$BFFF).

;  Assembles with ca65 (cc65 suite) to a byte-identical image:

;      ca65 miner2049er.asm -o miner.o

;      ld65 -C cart.cfg -o miner.rom miner.o

;  with cart.cfg:

;      MEMORY   { ROM: start=$8000, size=$4000, fill=yes, fillval=$FF, file=%O; }

;      SEGMENTS { CODE: load=ROM, type=ro; }

;

;  The same ROM image runs on the Atari 400/800 and on the Atari 5200: see COLD_START.

;  All names and comments are reconstructions from reading the code; nothing of the

;  original source survives. Addresses in comments are hexadecimal.

;

;  Memory map

;    $0000-$00FF  zero page: hardware pointer table ($10-$6F), game variables ($70-$FF)

;    $0200-$0223  interrupt vectors (both machines' layouts are written)

;    $0300-$07FF  player/missile graphics (single-line; PMBASE 3 = effectively 0)

;                 $0300 missiles (mutants), $0400-$06FF Bob (players 0-2), $0700 mutants

;    $0400-$043D  (on the high-score screen) the RAM copy of TITLE_KERNEL

;    $0800-$08C7  high-score table, $08C8 text for the title screens

;    $0917-$0991  text and display list for the prepare/name screens

;    $0992-$09BF  items of the current station, $09C0-$0A17 mutant records

;    $0A19-$0A24  pulverizers (station 9)

;    $1C00-$1FFF  second P/M buffer for the high-score screen; 5200 font copy

;    $2000-$20D7  game display list; $20D8 status line (40 characters)

;    $2100-$3DBF  the playfield: 184 lines x 160 pixels, 4 colours (ANTIC mode E)

;    $3DC0-$3F2F  bitmap line address tables (LINE_LO/LINE_HI = $3DB8/$3E70, indexed from

;                 y = 8); $3F30 moving platforms

;    $8000-$BFFF  this cartridge

;

;  Coordinates: horizontal positions use the P/M coordinate system (the playfield's left

;  edge is 48 = $30); bitmap routines subtract $30 to get the pixel column (0-159).

;  Vertical positions are bitmap lines 8-191 (line y is on scan line y + 32,

;  which is also P/M row y + 32); Bob's BOB_Y is the top of

;  his sprite and his feet are at BOB_Y + $15.

;

;  Data overlaps: to save ROM, many tables share bytes with their neighbours - the end of

;  one list is the start of the next, a display list's jump address is also the first

;  two characters of a text row, the cart's wheels are a bonus value, and so on. These

;  are pointed out where they occur.

; ============================================================================

.setcpu "6502"

LASTCHAR        = $02                   ; name entry: letter chosen with the 5200 stick / game: last cheat digit

;   typed

NAME_RPT        = $03                   ; 5200 name entry: stick auto-repeat counter

NAME_DLY        = $04                   ; 5200 name entry: keypad debounce timer

START_ZONE      = $05                   ; zone picked with the number keys on the title screens (1-10)

WARMST          = $08                   ; OS warm-start flag: non-zero after RESET, so high scores survive RESET

pHPOSP0         = $10                   ; -> GTIA HPOSP0 (write) / M0PF (read)

pHPOSP1         = $12                   ; -> HPOSP1 / M1PF

pHPOSP2         = $14                   ; -> HPOSP2 / M2PF

pHPOSP3         = $16                   ; -> HPOSP3 / M3PF

pHPOSM0         = $18                   ; -> HPOSM0 / P0PF

pHPOSM1         = $1A                   ; -> HPOSM1 / P1PF

pHPOSM2         = $1C                   ; -> HPOSM2 / P2PF

pHPOSM3         = $1E                   ; -> HPOSM3 / P3PF

pSIZEP0         = $20                   ; -> SIZEP0 / M0PL

pSIZEP1         = $22                   ; -> SIZEP1 / M1PL

pSIZEP2         = $24                   ; -> SIZEP2 / M2PL

pSIZEP3         = $26                   ; -> SIZEP3 / M3PL

pSIZEM          = $28                   ; -> SIZEM / P0PL

pGRAFP0         = $2A                   ; -> GRAFP0 / P1PL

pGRAFP1         = $2C                   ; -> GRAFP1 / P2PL

pGRAFP2         = $2E                   ; -> GRAFP2 / P3PL

pTRIG0          = $30                   ; -> GRAFP3 / TRIG0 (joystick button, 0 = pressed)

pGRAFM          = $32                   ; -> GRAFM / TRIG1

pCOLPM0         = $34                   ; -> COLPM0 (player 0 colour)

pCOLPM1         = $36                   ; -> COLPM1

pCOLPM2         = $38                   ; -> COLPM2

pCOLPM3         = $3A                   ; -> COLPM3

pCOLPF0         = $3C                   ; -> COLPF0 (playfield colour 0)

pCOLPF1         = $3E                   ; -> COLPF1

pCOLPF2         = $40                   ; -> COLPF2

pCOLPF3         = $42                   ; -> COLPF3 (also the colour of the missiles in fifth-player mode)

pCOLBK          = $44                   ; -> COLBK (background)

pPRIOR          = $46                   ; -> PRIOR

pVDELAY         = $48                   ; -> VDELAY

pGRACTL         = $4A                   ; -> GRACTL

pHITCLR         = $4C                   ; -> HITCLR

pCONSOL         = $4E                   ; -> CONSOL

pAUDF1          = $50                   ; -> POKEY AUDF1 (write) / POT0 (read)

pAUDC1          = $52                   ; -> AUDC1 / POT1

pAUDF2          = $54                   ; -> AUDF2 / POT2

pAUDC2          = $56                   ; -> AUDC2 / POT3

pAUDF3          = $58                   ; -> AUDF3 / POT4

pAUDC3          = $5A                   ; -> AUDC3 / POT5

pAUDF4          = $5C                   ; -> AUDF4 / POT6

pAUDC4          = $5E                   ; -> AUDC4 / POT7

pAUDCTL         = $60                   ; -> AUDCTL / ALLPOT

pSTIMER         = $62                   ; -> STIMER / KBCODE

pSKRES          = $64                   ; -> SKRES / RANDOM

pPOTGO          = $66                   ; -> POTGO

pPOKEY_C        = $68                   ; -> (unused POKEY register $0C)

pSEROUT         = $6A                   ; -> SEROUT / SERIN

pIRQEN          = $6C                   ; -> IRQEN / IRQST

pSKCTL          = $6E                   ; -> SKCTL / SKSTAT

TXT_DST         = $70                   ; (2) bitmap line address for the pixel being drawn

TXT_FONT        = $72                   ; (2) address of the font page holding the character

TXT_COLOR       = $74                   ; colour of the "ink": 0 = colour 1, 4 = colour 2, 8 = colour 3, 12 =

;   background

TXT_X           = $75                   ; x position (P/M horizontal coordinates: 48 = left edge of the playfield)

TXT_Y           = $76                   ; y position (scan line, 8-191)

TXT_BITS        = $77                   ; bits left in the current font byte

TXT_ROWS        = $78                   ; font rows left in the current character

TXT_VSCALE      = $79                   ; vertical magnification (1 or 2 scan lines per font row)

TXT_ROW         = $7A                   ; offset of the current font byte within the font page

TXT_LEN         = $7B                   ; characters left to draw

TXT_IDX         = $7C                   ; index of the next character in TEXT_POOL

PAUSE_DLY       = $7D                   ; ESC-key debounce for pause

SAVE_Y          = $7F                   ; scratch save of Y

PTR             = $80                   ; (2) general pointer

CNT0            = $82                   ; scratch counter

CNT1            = $83                   ; scratch counter / index

PLOT_COLOR      = $84                   ; colour for PLOT: 0 = colour 1 (COLPF0), 4 = colour 2 (COLPF1), 8 = colour

;   3 (COLPF2), 12 = background

PLOT_X          = $85                   ; x for PLOT (P/M coordinates or raw pixel, depending on routine)

PLOT_Y          = $86                   ; y for PLOT (scan line)

CNT2            = $87                   ; scratch counter

CNT3            = $88                   ; scratch counter

CNT4            = $89                   ; scratch counter / slope

COLBK_V         = $8A                   ; background colour (0 = black; flashes blue during the extra-life jingle)

COLPF0_V        = $8B                   ; station colour for pixel colour 1 (COLPF0): items, text

COLPF1_V        = $8C                   ; station colour for pixel colour 2 (COLPF1): ladders, slides

COLPF2_V        = $8D                   ; station colour for pixel colour 3 (COLPF2): girders, platforms

BLINK_TMR       = $8E                   ; frames until the status line blinks (CHACTL blank toggle)

BLINK_ON        = $8F                   ; current CHACTL value (0/1)

BOB_X           = $90                   ; Bounty Bob horizontal position (P/M coordinates)

BOB_Y           = $91                   ; Bounty Bob vertical position (top of his 21-line sprite)

BOB_FRAME       = $92                   ; animation frame 0-12 (1-3 walk right, 4-6 walk left, 7-8 climb, 9-10

;   ladder top, 11-12 jump)

BOB_FALL        = $93                   ; non-zero while Bob is falling freely

SPR_PTR         = $94                   ; (2) pointer to the current sprite or list

PIX_PTR         = $96                   ; (2) bitmap line pointer used by READ_PIXEL

JUMP_LOCK       = $98                   ; frames before the button may start another jump

PROBE_Y         = $99                   ; scan line being tested under Bob's feet

PROBE_X         = $9A                   ; P/M x being tested under Bob's feet

TMP9B           = $9B                   ; scratch

WALK_STEP       = $9C                   ; walk animation step 1-3

FRAMES          = $9D                   ; frame counter, incremented by every vertical-blank interrupt

LADDERS         = $9E                   ; (2) pointer to this station's ladder list

MOVE_A          = $A0                   ; movement state A (jump frame, ladder rung, transporter timer ...)

MOVE_B          = $A1                   ; movement state B (jump direction, ladder limit ...)

FALL_LEN        = $A2                   ; frames of falling counted by LAND_CHECK (40 or more is fatal)

SPECIAL         = $A3                   ; special state: 0 normal, 1 dying, 2 dead, 3-7 transporter phases

SND3_F          = $A4                   ; voice 3 frequency (footsteps, jump and fall effects)

SND3_VOL        = $A5                   ; voice 3 decaying volume

ALIVE           = $A6                   ; 2 while the station is being played; counts down after death; 0 ends the

;   turn

CNT5            = $A7                   ; scratch counter

SLIDES          = $A8                   ; (2) pointer to this station's slide (chute) list

SLIDE_LEN       = $AA                   ; frames left on a slide

CNT6            = $AB                   ; scratch

TRANS_LOCK      = $AC                   ; transporter lock-out after a trip / flashing colour on some stations

TRANS_DEST      = $AD                   ; transporter tube Bob asked for (1-4)

TRANS_Y         = $AE                   ; Bob's vertical position saved while he is transported

DLI_BAND        = $AF                   ; which horizontal band of mutants the next DLI handles (4..0)

TRANS_SRC       = $B0                   ; tube Bob is standing in

T_SCROLL        = $B0                   ; title: fine-scroll phase now showing (0-15)

LIFTS           = $B1                   ; (2) pointer to this station's moving-platform list

T_TMP           = $B1                   ; title: scratch (also the operand of the kernel's delay instruction)

T_NEWSCROLL     = $B2                   ; title: scroll phase the main loop wants shown next

SPECIAL_X       = $B3                   ; position of this station's special object (cannon, transporter column)

T_PTR           = $B3                   ; title: (2) pointer into the RAM kernel / PM memory

DYING           = $B4                   ; death animation counter (non-zero while Bob is dying)

PLAYER          = $B5                   ; current player (0 or 1)

T_CNT           = $B5                   ; title: scratch counter

T_CNT2          = $B6                   ; title: scratch counter

STATION         = $B7                   ; station being played (1-10; 11 = the title/credits screen)

T_LINE          = $B7                   ; title: which display-list interrupt is next

ZONE            = $B8                   ; zone (each zone is one pass through the ten stations, getting harder)

T_ROT           = $B8                   ; title: colour rotation of the chasing lights (0-9)

LIVES           = $B9                   ; lives left

T_HUE           = $B9                   ; title: start of the rainbow on the high-score lines

XLIFE           = $BA                   ; extra life: 10 = not yet earned, 9..1 = jingle playing, 0 = done

T_TEXTCOL       = $BA                   ; title: colour of "HIGH SCORES:" (pulsing)

SCORE           = $BB                   ; score: six decimal digits, one per byte, most significant first ($BB-$C0)

T_TEXTSTEP      = $BB                   ; title: +1/-1 brightness step for T_TEXTCOL

T_TIMER30       = $BC                   ; title: 30-frame timer (changes every frame)

T_TIMER14       = $BD                   ; title: 14-frame timer for the rainbow

T_INVERSE       = $BE                   ; title: CHACTL value (blinks "START" in inverse video)

T_PMPAGE        = $BF                   ; title: PM buffer being drawn ($1C or $04)

T_PMPTR         = $C0                   ; title: (2) pointer into the back PM buffer

P2_DATA         = $C1                   ; the other player's copy of STATION..SCORE ($C1-$CA)

T_NEXT          = $C2                   ; title: next scroll phase

P2_LIVES        = $C3                   ; the other player's lives (non-zero means he is still playing)

T_TIMER4        = $C3                   ; title: 4-frame timer for the text pulsing

T_COLIX         = $C4                   ; title: colour index for the rainbow DLI

T_SELDLY        = $C5                   ; title: SELECT debounce

T_STARTDLY      = $C6                   ; title: START debounce (START must be released first)

T_TIMER60       = $C7                   ; title: 60-frame timer

T_DEMOSEC       = $C8                   ; title: seconds left before the demo starts

T_TIMER11       = $C9                   ; title: 11-frame timer for the background tone

T_VOLUME        = $CA                   ; title: AUDC1 value of the fading background tone

IS5200          = $CB                   ; 0 on the Atari 400/800, 1 on the Atari 5200

FONTPAGE        = $CC                   ; high byte of the ROM font: $E0 on the 400/800, $F8 on the 5200

LINECNT         = $CD                   ; scan-line counter inside display list interrupts

CNT7            = $CE                   ; scratch counter

POT_X           = $CF                   ; 5200: analog stick X (POT0)

POT_Y           = $D0                   ; 5200: analog stick Y (POT1)

ITEM_GOT        = $D1                   ; set when an item is picked up: the mutants turn edible on the next frame

CANNON_DONE     = $D2                   ; station 10: Bob has been fired to the top and is no longer drawn as a

;   cannonball

CANNON_ST       = $D3                   ; station 10 cannon state: 0 idle, 1 Bob inside, 2 fired

CANNON_X        = $D4                   ; station 10 cannon position

TNT             = $D5                   ; station 10 TNT charge loaded (tons)

LIFT_DLY        = $D6                   ; station 8 lift: key debounce

LIFT_ON         = $D7                   ; station 8 lift: Bob is operating the lift

LIFT_ANIM       = $D8                   ; station 8 lift: scissor animation frame 0-6

ON_LIFT         = $D9                   ; zero when Bob is standing on the lift platform

LIFT_X          = $DA                   ; station 8 lift platform x

LIFT_Y          = $DB                   ; station 8 lift platform y

CHEAT_IDX       = $DD                   ; how many digits of the phone-number code have been typed (13 = all)

SAVE_Y2         = $DE                   ; scratch save of Y

DEMO_CNT        = $DF                   ; frames left for the current demo input

KEYCODE         = $E0                   ; last key read from POKEY ($FF = none)

DEMO_PTR        = $E1                   ; (2) pointer into the recorded demo; high byte 0 = a real player is playing

START_DLY       = $E3                   ; START/fire debounce

PAUSED          = $E4                   ; non-zero while the game is paused (ESC key)

MUT_SCORE       = $E5                   ; hundreds to add for an eaten mutant (8 or 9)

HIT_M           = $E6                   ; missile-to-player collisions, one byte per mutant band ($E6-$E9)

HIT_P3          = $EA                   ; player 3 (mutant) to player collisions, one byte per band ($EA-$ED)

TMPEF           = $EF                   ; scratch

MUT_PM          = $F0                   ; (2) pointer into player-3 memory when a mutant's shape is drawn

MUT_LOOP        = $F2                   ; mutants left to process in this DLI

MUTANTS         = $F3                   ; (2) pointer to this station's mutant definitions

MUT_IDX         = $F5                   ; offset of the current mutant record (multiples of 11)

SECTIONS        = $F6                   ; floor sections still to be claimed (0 = station complete)

TICK_SND        = $F7                   ; bonus-timer tick sound counter

WIPE_LINE       = $F8                   ; screen wipe: display-list line being removed (0 = finished)

TWO_PLAYER      = $F9                   ; 0 = one-player game, 1 = two-player game

LIFE_USED       = $FA                   ; set once a life has been taken for this turn

BCD_100         = $FB                   ; scratch digit / DLI timer

SEC_TMR         = $FC                   ; 60-frame timer

BONUS_TMR       = $FD                   ; frames until the bonus timer drops by 100

MOVE_OK         = $FE                   ; 0 while the station is starting, then 7 (Bob may move)

KILLED          = $FF                   ; non-zero: Bob has touched a mutant or run out of time

VDSLST          = $0200                 ; 400/800 OS: display-list-interrupt vector

VVBLKI5         = $0202                 ; 5200 BIOS: immediate vertical-blank vector

VVBLKD5         = $0204                 ; 5200 BIOS: deferred vertical-blank vector

VDSLST5         = $0206                 ; 5200 BIOS: display-list-interrupt vector

VVBLKI          = $0222                 ; 400/800 OS: immediate vertical-blank vector

PM_MISSILES     = $0300                 ; player/missile memory (single-line resolution, PMBASE effectively 0):

;   missiles

PM_P0           = $0400                 ; player 0: Bob's outline colour

PM_P1           = $0500                 ; player 1: Bob's second colour

PM_P2           = $0600                 ; player 2: Bob's third colour

PM_P3           = $0700                 ; player 3: the mutants (re-used five times down the screen)

HISCORES        = $0800                 ; high-score table: 10 lines of 20 screen-code characters

TEXT_RAM        = $08C8                 ; text lines for the title, high-score and "prepare" screens

ITEMS           = $0992                 ; this station's items, 3 bytes each: shape number (bit 7 = still there), y,

;   x

MUT_TAB         = $09C0                 ; mutant records, 11 bytes each (8 mutants)

PULV_TAB        = $0A19                 ; station 9 pulverizers: 3 bytes each (timer, direction, height)

FONT5200        = $1C00                 ; 5200: RAM copy of the BIOS font with a few changed characters

DLIST           = $2000                 ; game display list

STATUS          = $20D8                 ; status line (40 characters of ANTIC mode 2)

BITMAP          = $2100                 ; the playfield: 184 lines x 40 bytes, 4 pixels per byte (ANTIC mode E)

LINE_LO         = $3DB8                 ; low byte of the address of each bitmap line, indexed by y (8-191)

LINE_HI         = $3E70                 ; high byte of the address of each bitmap line

LIFT_TAB        = $3F30                 ; moving-platform records, 6 bytes each

CONSOL          = $D01F                 ; GTIA: console keys START/SELECT/OPTION (bits 0-2, 0 = pressed)

PORTA           = $D300                 ; PIA: joystick 0/1 directions (bits 0-3 stick 0: up,down,left,right; 0 =

;   pushed)

PACTL           = $D302                 ; PIA: port A control (bit 2 selects data register / direction register)

DMACTL          = $D400                 ; ANTIC: DMA control (playfield width, P/M DMA, P/M resolution)

CHACTL          = $D401                 ; ANTIC: character control (bit 0 = blank bit-7 characters, bit 1 = inverse)

DLISTL          = $D402                 ; ANTIC: display list address, low byte

DLISTH          = $D403                 ; ANTIC: display list address, high byte

HSCROL          = $D404                 ; ANTIC: horizontal fine scroll (0-15 colour clocks)

PMBASE          = $D407                 ; ANTIC: player/missile base page

CHBASE          = $D409                 ; ANTIC: character set base page

WSYNC           = $D40A                 ; ANTIC: any write halts the CPU until the end of the scan line

VCOUNT          = $D40B                 ; ANTIC: current scan line / 2

NMIEN           = $D40E                 ; ANTIC: NMI enable (bit 7 = DLI, bit 6 = VBI)

NMIRES          = $D40F                 ; ANTIC: write = reset NMI status (read = NMIST)

FONT_800        = $E000                 ; OS ROM character set on the 400/800 (the game draws text from it)

FONT_5200       = $F800                 ; 5200 BIOS character set

.org $8000

;-----------------------------------------------------------------------------

;  TITLE_DLIST - display list of the high-score screen. The JVB's address bytes ($801F)

;  are also the first two characters of the bottom row of lights (LMS $801F), so its

;  target ($1500) is nonsense - harmless, because TITLE_VBI reloads DLIST every frame.

;-----------------------------------------------------------------------------

TITLE_DLIST:    .byte $70               ; 8 blank lines

.byte $70               ; 8 blank lines

.byte $F0               ; 8 blank lines +DLI

.byte $D7,$59,$80       ; top row of lights: the "5"s at $8059, fine-scrolled  mode 7 LMS $8059

;   +HSCROL +DLI

.byte $70               ; 8 blank lines

.byte $86               ; "HIGH SCORES:" (continues from the top row)  mode 6 +DLI

.byte $70               ; 8 blank lines

.byte $F0               ; 8 blank lines +DLI

.byte $C7,$00,$08       ; the best score, in big letters  mode 7 LMS $0800 +DLI

.byte $F0               ; 8 blank lines +DLI

.byte $86               ; high scores 2-10  mode 6 +DLI

.byte $86               ; mode 6 +DLI

.byte $86               ; mode 6 +DLI

.byte $86               ; mode 6 +DLI

.byte $86               ; mode 6 +DLI

.byte $86               ; mode 6 +DLI

.byte $86               ; mode 6 +DLI

.byte $86               ; mode 6 +DLI

.byte $86               ; mode 6 +DLI

.byte $70               ; 8 blank lines

.byte $82               ; "Press START For a 1 Miner Game"  mode 2 +DLI

.byte $70               ; 8 blank lines

.byte $F0               ; 8 blank lines +DLI

.byte $57,$1F,$80       ; bottom row of lights (its data begins at $801F)  mode 7 LMS $801F +HSCROL

.byte $41,$00,$15       ; JVB $1500

.byte $00,$55,$00,$95,$00,$D5,$00,$00,$00,$00,$00,$15,$00,$55,$00,$95; rest of the bottom row of

;   lights: '5' ($15) with colour

;   bits $40/$80/$C0

.byte $00,$D5

;-----------------------------------------------------------------------------

;  TITLE_TEXT - "Press START For a 1 Miner Game" (40 characters, copied to TEXT_RAM).

;  Its first bytes are also the end of the bottom row of lights.

;-----------------------------------------------------------------------------

TITLE_TEXT:     .byte $00,$00,$00,$00,$00,$30,$72,$65,$73,$73; "     Press"

.byte $00,$B3,$B4,$A1,$B2,$B4,$00,$26,$6F,$72; " START For"

.byte $00,$61,$00,$11,$00,$2D,$69,$6E,$65,$72; " a 1 Miner"

.byte $00,$27,$61,$6D,$65,$00,$00,$00,$00,$00; " Game     "

;-----------------------------------------------------------------------------

;  TOP_ROW - the top row of lights. ANTIC fetches 24 bytes from $8059 (the last two

;  characters of TITLE_TEXT are its first two), then "HIGH SCORES:" at $8071.

;-----------------------------------------------------------------------------

TOP_ROW:        .byte $15,$00,$55,$00,$95,$00,$D5,$00,$00,$00,$00

.byte $00,$15,$00,$55,$00,$95,$00,$D5,$00,$00,$00

.byte $00,$00,$00,$00,$28,$29,$27,$28,$00,$33; "    HIGH S"

.byte $23,$2F,$32,$25,$33,$1A,$00,$00,$00,$00; "CORES:    "

;-----------------------------------------------------------------------------

;  TEXT_POOL - strings drawn into the bitmap by DRAW_STRING (screen codes; TXT_IDX offsets):

;    $00 DANGER  $06 Radioactive  $11 Waste  $16 MINER 2049er  $22 By  $24 Bill Hogue

;    $2E Starring  $36 "Bounty Bob"  $42 Copyright (C)  $4F 1982 By  $56 Big Five Software

;    $67 P.O. Box 9078-185  $78 Van Nuys Ca 91409  $89 (213) 782-6861  $97 Game Over

;    $A0 Miner  $A5 TNT Charge=   0 Tons

;  The phone number at $810E is also the cheat code (CHEAT_CODE, see PHONE_CODE).

;-----------------------------------------------------------------------------

TEXT_POOL:      .byte $24,$21,$2E,$27,$25,$32,$32,$61,$64,$69,$6F,$61,$63,$74,$69,$76; "DANGERRadioactiv"

.byte $65,$37,$61,$73,$74,$65,$2D,$29,$2E,$25,$32,$00,$12,$10,$14,$19; "eWasteMINER 2049"

.byte $65,$72,$22,$79,$22,$69,$6C,$6C,$00,$28,$6F,$67,$75,$65,$33,$74; "erByBill HogueSt"

.byte $61,$72,$72,$69,$6E,$67,$02,$22,$6F,$75,$6E,$74,$79,$00,$22,$6F; "arring"Bounty Bo"

.byte $62,$02,$23,$6F,$70,$79,$72,$69,$67,$68,$74,$00,$08,$23,$09,$11; "b"Copyright (C)1"

.byte $19,$18,$12,$00,$22,$79,$22,$69,$67,$00,$26,$69,$76,$65,$00,$33; "982 ByBig Five S"

.byte $6F,$66,$74,$77,$61,$72,$65,$30,$0E,$2F,$0E,$00,$22,$6F,$78,$00; "oftwareP.O. Box "

.byte $19,$10,$17,$18,$0D,$11,$18,$15,$36,$61,$6E,$00,$2E,$75,$79,$73; "9078-185Van Nuys"

.byte $00,$23,$61,$00,$19,$11,$14,$10,$19; " Ca 91409"

PHONE_TEXT:     .byte $08               ; "("

CHEAT_CODE:     .byte $12,$11,$13,$09,$00,$17,$18,$12,$0D,$16,$18,$16,$11; "213) 782-6861"

.byte $27,$61,$6D,$65,$00,$2F,$76,$65,$72,$2D,$69,$6E,$65,$72,$34,$2E; "Game OverMinerTN"

.byte $34,$00,$23,$68,$61,$72,$67,$65,$1D,$00,$00,$00,$10,$00,$34,$6F; "T Charge=   0 To"

.byte $6E,$73           ; "ns"

;-----------------------------------------------------------------------------

;  NAME_TEXT - the name-entry screen (copied to $0918): a 24-byte mode 7 line (fine

;  scrolled), then three 20-byte lines. Upper case with colour bits set = coloured text.

;-----------------------------------------------------------------------------

NAME_TEXT:      .byte $00,$00,$00,$00,$00,$00,$00,$00,$AD,$A9,$AE,$A5; "        MINE"

.byte $B2,$00,$91,$00,$00,$00,$00,$00,$00,$00,$00,$00; "R 1         "

.byte $00,$00,$00,$00,$67,$72,$65,$61,$74,$00; "    great "

.byte $73,$63,$6F,$72,$65,$41,$00,$00,$00,$00; "score.    "

.byte $00,$00,$E5,$EE,$F4,$E5,$F2,$00,$F9,$EF; "  enter yo"

.byte $F5,$F2,$00,$EE,$E1,$ED,$E5,$C1,$00,$00; "ur name.  "

.byte $00,$00,$00,$9E,$9E,$00,$00,$00,$00,$00; "   >>     "

.byte $00,$00,$00,$00,$00,$9C,$9C,$00,$00,$00; "     <<   "

;-----------------------------------------------------------------------------

;  PREP_TEXT - "MINER 1" / "prepare for" lines of the prepare screen (48 bytes copied to

;  $0918, starting one byte earlier at $8191), then the "STATION n" lines (from $81C2, or

;  $81C3 for station 10 so that the two digits stay centred) and "Zone" at $81F1.

;-----------------------------------------------------------------------------

PREP_TEXT:      .byte $00,$00,$00,$00,$00,$00,$00,$2D,$29,$2E,$25,$32,$00,$11,$00,$00; "       MINER 1  "

.byte $00,$00,$00,$00,$00,$00,$00,$00,$00,$00,$00,$00,$00,$70,$72,$65; "             pre"

.byte $70,$61,$72,$65,$00,$66,$6F,$72,$00,$00,$00,$00,$00,$00,$00,$00; "pare for        "

.byte $00,$00,$00,$00,$00,$00,$B3,$B4,$A1,$B4,$A9,$AF,$AE,$00,$91,$00; "      STATION 1 "

.byte $00,$00,$00,$00,$00,$00,$00,$00,$00,$00,$00,$00,$00,$00,$00,$00; "                "

.byte $00,$00,$00,$00,$00,$00,$00,$00,$00,$00,$00,$00,$00,$00,$00; "               "

ZONE_TEXT:      .byte $FA,$EF,$EE,$E5   ; "zone"

;-----------------------------------------------------------------------------

;  KEY_TO_SCREEN - 400/800 keyboard code (KBCODE and $3F) -> screen code; 0 = no character

;  (RETURN, DELETE, TAB ... are handled by code).

;-----------------------------------------------------------------------------

KEY_TO_SCREEN:  .byte $2C,$2A,$1B,$00,$00,$2B,$0B,$0A,$2F,$00,$30,$35,$00,$29,$0D,$1D

.byte $36,$00,$23,$00,$00,$22,$38,$3A,$14,$00,$13,$16,$00,$15,$12,$11

.byte $0C,$00,$0E,$2E,$00,$2D,$0F,$00,$32,$00,$25,$39,$00,$34,$37,$31

.byte $19,$00,$10,$17,$00,$18,$1C

;-----------------------------------------------------------------------------

;  SHIFT_MAP - for screen codes 1-27 (digits and punctuation, as SHIFT_MAP-1 indexed by

;  the code): the character SHIFT gives on that key ('1' -> '!', '2' -> '"' ...).

;  It starts inside KEY_TO_SCREEN.

;-----------------------------------------------------------------------------

SHIFT_MAP:      .byte $1E,$26,$28,$24,$00,$00,$27,$33,$21,$3E,$3C,$3B,$3F,$3D,$1F,$09

.byte $01,$02,$03,$04,$05,$06,$07,$20,$08,$1A,$1A

;-----------------------------------------------------------------------------

;  BOB_FRAME_LO / BOB_FRAME_HI - addresses of Bob's 13 animation frames.

;   0 standing (same as 1)  1-3 walking right  4-6 walking left  7-8 climbing

;   9-10 at the top of a ladder  11 jumping/sliding right  12 left

;-----------------------------------------------------------------------------

BOB_FRAME_LO:   .byte $60,$60,$93,$C6,$F9,$2C,$5F,$92,$C5,$F8,$2B,$5F,$92

BOB_FRAME_HI:   .byte $82,$82,$82,$82,$82,$83,$83,$83,$83,$83,$84,$84,$84

;-----------------------------------------------------------------------------

;  Bob frame 0 (standing): player 0 (20 lines), player 1 (17), player 2 (17), drawn from the

;  last byte upwards; each layer's first line is the previous layer's last.

;-----------------------------------------------------------------------------

BOB_0:          .byte $00               ; ........

.byte $18               ; ...##...

.byte $3C               ; ..####..

.byte $FF               ; ########

.byte $00               ; ........

.byte $00               ; ........

.byte $00               ; ........

.byte $00               ; ........

.byte $00               ; ........

.byte $00               ; ........

.byte $18               ; ...##...

.byte $18               ; ...##...

.byte $3C               ; ..####..

.byte $6E               ; .##.###.

.byte $00               ; ........

.byte $00               ; ........

.byte $6E               ; .##.###.

.byte $7E               ; .######.

.byte $3C               ; ..####..

.byte $00               ; ........

.byte $18               ; ...##...

.byte $3C               ; ..####..

.byte $FF               ; ########

.byte $3C               ; ..####..

.byte $34               ; ..##.#..

.byte $3E               ; ..#####.

.byte $34               ; ..##.#..

.byte $38               ; ..###...

.byte $1C               ; ...###..

.byte $00               ; ........

.byte $00               ; ........

.byte $00               ; ........

.byte $00               ; ........

.byte $6E               ; .##.###.

.byte $6E               ; .##.###.

.byte $00               ; ........

.byte $00               ; ........

.byte $00               ; ........

.byte $00               ; ........

.byte $00               ; ........

.byte $00               ; ........

.byte $00               ; ........

.byte $10               ; ...#....

.byte $10               ; ...#....

.byte $10               ; ...#....

.byte $10               ; ...#....

.byte $00               ; ........

.byte $00               ; ........

.byte $18               ; ...##...

.byte $18               ; ...##...

.byte $1C

;-----------------------------------------------------------------------------

;  Bob frame 2 (walk right 2): player 0 (20 lines), player 1 (17), player 2 (17), drawn from the

;  last byte upwards; each layer's first line is the previous layer's last.

;-----------------------------------------------------------------------------

BOB_2:          .byte $00               ; ........

.byte $18               ; ...##...

.byte $3C               ; ..####..

.byte $FF               ; ########

.byte $00               ; ........

.byte $00               ; ........

.byte $00               ; ........

.byte $00               ; ........

.byte $00               ; ........

.byte $00               ; ........

.byte $18               ; ...##...

.byte $18               ; ...##...

.byte $3C               ; ..####..

.byte $6E               ; .##.###.

.byte $00               ; ........

.byte $00               ; ........

.byte $7A               ; .####.#.

.byte $7E               ; .######.

.byte $3C               ; ..####..

.byte $00               ; ........

.byte $18               ; ...##...

.byte $3C               ; ..####..

.byte $FF               ; ########

.byte $3C               ; ..####..

.byte $34               ; ..##.#..

.byte $3E               ; ..#####.

.byte $34               ; ..##.#..

.byte $38               ; ..###...

.byte $1C               ; ...###..

.byte $00               ; ........

.byte $00               ; ........

.byte $00               ; ........

.byte $00               ; ........

.byte $6E               ; .##.###.

.byte $76               ; .###.##.

.byte $00               ; ........

.byte $00               ; ........

.byte $00               ; ........

.byte $00               ; ........

.byte $00               ; ........

.byte $00               ; ........

.byte $00               ; ........

.byte $10               ; ...#....

.byte $10               ; ...#....

.byte $08               ; ....#...

.byte $04               ; .....#..

.byte $00               ; ........

.byte $00               ; ........

.byte $14               ; ...#.#..

.byte $14               ; ...#.#..

.byte $1E

;-----------------------------------------------------------------------------

;  Bob frame 3 (walk right 3): player 0 (20 lines), player 1 (17), player 2 (17), drawn from the

;  last byte upwards; each layer's first line is the previous layer's last.

;-----------------------------------------------------------------------------

BOB_3:          .byte $00               ; ........

.byte $18               ; ...##...

.byte $3C               ; ..####..

.byte $FF               ; ########

.byte $00               ; ........

.byte $00               ; ........

.byte $00               ; ........

.byte $00               ; ........

.byte $00               ; ........

.byte $00               ; ........

.byte $18               ; ...##...

.byte $18               ; ...##...

.byte $3C               ; ..####..

.byte $6E               ; .##.###.

.byte $00               ; ........

.byte $00               ; ........

.byte $3E               ; ..#####.

.byte $7E               ; .######.

.byte $3C               ; ..####..

.byte $00               ; ........

.byte $18               ; ...##...

.byte $3C               ; ..####..

.byte $FF               ; ########

.byte $3C               ; ..####..

.byte $34               ; ..##.#..

.byte $3E               ; ..#####.

.byte $34               ; ..##.#..

.byte $38               ; ..###...

.byte $1C               ; ...###..

.byte $00               ; ........

.byte $00               ; ........

.byte $00               ; ........

.byte $00               ; ........

.byte $6E               ; .##.###.

.byte $5E               ; .#.####.

.byte $00               ; ........

.byte $00               ; ........

.byte $00               ; ........

.byte $00               ; ........

.byte $00               ; ........

.byte $00               ; ........

.byte $00               ; ........

.byte $10               ; ...#....

.byte $10               ; ...#....

.byte $20               ; ..#.....

.byte $40               ; .#......

.byte $00               ; ........

.byte $00               ; ........

.byte $24               ; ..#..#..

.byte $24               ; ..#..#..

.byte $36

;-----------------------------------------------------------------------------

;  Bob frame 4 (walk left 1): player 0 (20 lines), player 1 (17), player 2 (17), drawn from the

;  last byte upwards; each layer's first line is the previous layer's last.

;-----------------------------------------------------------------------------

BOB_4:          .byte $00               ; ........

.byte $18               ; ...##...

.byte $3C               ; ..####..

.byte $FF               ; ########

.byte $00               ; ........

.byte $00               ; ........

.byte $00               ; ........

.byte $00               ; ........

.byte $00               ; ........

.byte $00               ; ........

.byte $18               ; ...##...

.byte $18               ; ...##...

.byte $3C               ; ..####..

.byte $76               ; .###.##.

.byte $00               ; ........

.byte $00               ; ........

.byte $76               ; .###.##.

.byte $7E               ; .######.

.byte $3C               ; ..####..

.byte $00               ; ........

.byte $18               ; ...##...

.byte $3C               ; ..####..

.byte $FF               ; ########

.byte $3C               ; ..####..

.byte $2C               ; ..#.##..

.byte $7C               ; .#####..

.byte $2C               ; ..#.##..

.byte $1C               ; ...###..

.byte $38               ; ..###...

.byte $00               ; ........

.byte $00               ; ........

.byte $00               ; ........

.byte $00               ; ........

.byte $76               ; .###.##.

.byte $76               ; .###.##.

.byte $00               ; ........

.byte $00               ; ........

.byte $00               ; ........

.byte $00               ; ........

.byte $00               ; ........

.byte $00               ; ........

.byte $00               ; ........

.byte $08               ; ....#...

.byte $08               ; ....#...

.byte $08               ; ....#...

.byte $08               ; ....#...

.byte $00               ; ........

.byte $00               ; ........

.byte $18               ; ...##...

.byte $18               ; ...##...

.byte $38

;-----------------------------------------------------------------------------

;  Bob frame 5 (walk left 2): player 0 (20 lines), player 1 (17), player 2 (17), drawn from the

;  last byte upwards; each layer's first line is the previous layer's last.

;-----------------------------------------------------------------------------

BOB_5:          .byte $00               ; ........

.byte $18               ; ...##...

.byte $3C               ; ..####..

.byte $FF               ; ########

.byte $00               ; ........

.byte $00               ; ........

.byte $00               ; ........

.byte $00               ; ........

.byte $00               ; ........

.byte $00               ; ........

.byte $18               ; ...##...

.byte $18               ; ...##...

.byte $3C               ; ..####..

.byte $76               ; .###.##.

.byte $00               ; ........

.byte $00               ; ........

.byte $5E               ; .#.####.

.byte $7E               ; .######.

.byte $3C               ; ..####..

.byte $00               ; ........

.byte $18               ; ...##...

.byte $3C               ; ..####..

.byte $FF               ; ########

.byte $3C               ; ..####..

.byte $2C               ; ..#.##..

.byte $7C               ; .#####..

.byte $2C               ; ..#.##..

.byte $1C               ; ...###..

.byte $38               ; ..###...

.byte $00               ; ........

.byte $00               ; ........

.byte $00               ; ........

.byte $00               ; ........

.byte $76               ; .###.##.

.byte $6E               ; .##.###.

.byte $00               ; ........

.byte $00               ; ........

.byte $00               ; ........

.byte $00               ; ........

.byte $00               ; ........

.byte $00               ; ........

.byte $00               ; ........

.byte $08               ; ....#...

.byte $08               ; ....#...

.byte $10               ; ...#....

.byte $20               ; ..#.....

.byte $00               ; ........

.byte $00               ; ........

.byte $28               ; ..#.#...

.byte $28               ; ..#.#...

.byte $78

;-----------------------------------------------------------------------------

;  Bob frame 6 (walk left 3): player 0 (20 lines), player 1 (17), player 2 (17), drawn from the

;  last byte upwards; each layer's first line is the previous layer's last.

;-----------------------------------------------------------------------------

BOB_6:          .byte $00               ; ........

.byte $18               ; ...##...

.byte $3C               ; ..####..

.byte $FF               ; ########

.byte $00               ; ........

.byte $00               ; ........

.byte $00               ; ........

.byte $00               ; ........

.byte $00               ; ........

.byte $00               ; ........

.byte $18               ; ...##...

.byte $18               ; ...##...

.byte $3C               ; ..####..

.byte $76               ; .###.##.

.byte $00               ; ........

.byte $00               ; ........

.byte $7C               ; .#####..

.byte $7E               ; .######.

.byte $3C               ; ..####..

.byte $00               ; ........

.byte $18               ; ...##...

.byte $3C               ; ..####..

.byte $FF               ; ########

.byte $3C               ; ..####..

.byte $2C               ; ..#.##..

.byte $7C               ; .#####..

.byte $2C               ; ..#.##..

.byte $1C               ; ...###..

.byte $38               ; ..###...

.byte $00               ; ........

.byte $00               ; ........

.byte $00               ; ........

.byte $00               ; ........

.byte $76               ; .###.##.

.byte $7A               ; .####.#.

.byte $00               ; ........

.byte $00               ; ........

.byte $00               ; ........

.byte $00               ; ........

.byte $00               ; ........

.byte $00               ; ........

.byte $00               ; ........

.byte $08               ; ....#...

.byte $08               ; ....#...

.byte $04               ; .....#..

.byte $02               ; ......#.

.byte $00               ; ........

.byte $00               ; ........

.byte $24               ; ..#..#..

.byte $24               ; ..#..#..

.byte $6C

;-----------------------------------------------------------------------------

;  Bob frame 7 (climb 1): player 0 (20 lines), player 1 (17), player 2 (17), drawn from the

;  last byte upwards; each layer's first line is the previous layer's last.

;-----------------------------------------------------------------------------

BOB_7:          .byte $00               ; ........

.byte $18               ; ...##...

.byte $3C               ; ..####..

.byte $FF               ; ########

.byte $00               ; ........

.byte $00               ; ........

.byte $00               ; ........

.byte $00               ; ........

.byte $00               ; ........

.byte $00               ; ........

.byte $18               ; ...##...

.byte $18               ; ...##...

.byte $3C               ; ..####..

.byte $7E               ; .######.

.byte $00               ; ........

.byte $00               ; ........

.byte $7E               ; .######.

.byte $7E               ; .######.

.byte $3C               ; ..####..

.byte $00               ; ........

.byte $18               ; ...##...

.byte $3C               ; ..####..

.byte $FF               ; ########

.byte $3C               ; ..####..

.byte $7E               ; .######.

.byte $7E               ; .######.

.byte $7E               ; .######.

.byte $3C               ; ..####..

.byte $3C               ; ..####..

.byte $00               ; ........

.byte $00               ; ........

.byte $00               ; ........

.byte $00               ; ........

.byte $7E               ; .######.

.byte $7E               ; .######.

.byte $00               ; ........

.byte $00               ; ........

.byte $00               ; ........

.byte $00               ; ........

.byte $C0               ; ##......

.byte $80               ; #.......

.byte $83               ; #.....##

.byte $81               ; #......#

.byte $00               ; ........

.byte $00               ; ........

.byte $00               ; ........

.byte $00               ; ........

.byte $00               ; ........

.byte $24               ; ..#..#..

.byte $26               ; ..#..##.

.byte $60

;-----------------------------------------------------------------------------

;  Bob frame 8 (climb 2): player 0 (20 lines), player 1 (17), player 2 (17), drawn from the

;  last byte upwards; each layer's first line is the previous layer's last.

;-----------------------------------------------------------------------------

BOB_8:          .byte $00               ; ........

.byte $18               ; ...##...

.byte $3C               ; ..####..

.byte $FF               ; ########

.byte $00               ; ........

.byte $00               ; ........

.byte $00               ; ........

.byte $00               ; ........

.byte $00               ; ........

.byte $00               ; ........

.byte $18               ; ...##...

.byte $18               ; ...##...

.byte $3C               ; ..####..

.byte $7E               ; .######.

.byte $00               ; ........

.byte $00               ; ........

.byte $7E               ; .######.

.byte $7E               ; .######.

.byte $3C               ; ..####..

.byte $00               ; ........

.byte $18               ; ...##...

.byte $3C               ; ..####..

.byte $FF               ; ########

.byte $3C               ; ..####..

.byte $7E               ; .######.

.byte $7E               ; .######.

.byte $7E               ; .######.

.byte $3C               ; ..####..

.byte $3C               ; ..####..

.byte $00               ; ........

.byte $00               ; ........

.byte $00               ; ........

.byte $00               ; ........

.byte $7E               ; .######.

.byte $7E               ; .######.

.byte $00               ; ........

.byte $00               ; ........

.byte $00               ; ........

.byte $00               ; ........

.byte $03               ; ......##

.byte $01               ; .......#

.byte $C1               ; ##.....#

.byte $81               ; #......#

.byte $00               ; ........

.byte $00               ; ........

.byte $00               ; ........

.byte $00               ; ........

.byte $00               ; ........

.byte $24               ; ..#..#..

.byte $64               ; .##..#..

.byte $06

;-----------------------------------------------------------------------------

;  Bob frame 9 (ladder top 1): player 0 (20 lines), player 1 (17), player 2 (17), drawn from the

;  last byte upwards; each layer's first line is the previous layer's last.

;-----------------------------------------------------------------------------

BOB_9:          .byte $00               ; ........

.byte $00               ; ........

.byte $00               ; ........

.byte $18               ; ...##...

.byte $3C               ; ..####..

.byte $FF               ; ########

.byte $00               ; ........

.byte $00               ; ........

.byte $3E               ; ..#####.

.byte $3E               ; ..#####.

.byte $00               ; ........

.byte $00               ; ........

.byte $7E               ; .######.

.byte $7F               ; .#######

.byte $7F               ; .#######

.byte $7E               ; .######.

.byte $66               ; .##..##.

.byte $66               ; .##..##.

.byte $60               ; .##.....

.byte $00               ; ........

.byte $00               ; ........

.byte $00               ; ........

.byte $18               ; ...##...

.byte $3C               ; ..####..

.byte $FF               ; ########

.byte $7E               ; .######.

.byte $3C               ; ..####..

.byte $00               ; ........

.byte $00               ; ........

.byte $7E               ; .######.

.byte $7E               ; .######.

.byte $00               ; ........

.byte $00               ; ........

.byte $00               ; ........

.byte $00               ; ........

.byte $00               ; ........

.byte $01               ; .......#

.byte $41               ; .#.....#

.byte $C0               ; ##......

.byte $80               ; #.......

.byte $00               ; ........

.byte $00               ; ........

.byte $00               ; ........

.byte $00               ; ........

.byte $00               ; ........

.byte $00               ; ........

.byte $00               ; ........

.byte $04               ; .....#..

.byte $26               ; ..#..##.

.byte $20               ; ..#.....

.byte $60

;-----------------------------------------------------------------------------

;  Bob frame 10 (ladder top 2): player 0 (20 lines), player 1 (17), player 2 (17), drawn from the

;  last byte upwards; each layer's first line is the previous layer's last.

;-----------------------------------------------------------------------------

BOB_10:         .byte $00               ; ........

.byte $00               ; ........

.byte $00               ; ........

.byte $18               ; ...##...

.byte $3C               ; ..####..

.byte $FF               ; ########

.byte $00               ; ........

.byte $00               ; ........

.byte $7C               ; .#####..

.byte $7C               ; .#####..

.byte $00               ; ........

.byte $00               ; ........

.byte $7E               ; .######.

.byte $FE               ; #######.

.byte $FE               ; #######.

.byte $7E               ; .######.

.byte $66               ; .##..##.

.byte $66               ; .##..##.

.byte $06               ; .....##.

.byte $00               ; ........

.byte $00               ; ........

.byte $00               ; ........

.byte $18               ; ...##...

.byte $3C               ; ..####..

.byte $FF               ; ########

.byte $7E               ; .######.

.byte $3C               ; ..####..

.byte $00               ; ........

.byte $00               ; ........

.byte $7E               ; .######.

.byte $7E               ; .######.

.byte $00               ; ........

.byte $00               ; ........

.byte $00               ; ........

.byte $00               ; ........

.byte $00               ; ........

.byte $80               ; #.......

.byte $82               ; #.....#.

.byte $03               ; ......##

.byte $01               ; .......#

.byte $00               ; ........

.byte $00               ; ........

.byte $00               ; ........

.byte $00               ; ........

.byte $00               ; ........

.byte $00               ; ........

.byte $00               ; ........

.byte $20               ; ..#.....

.byte $64               ; .##..#..

.byte $04               ; .....#..

.byte $06,$00

;-----------------------------------------------------------------------------

;  Bob frame 11 (jump right): player 0 (20 lines), player 1 (17), player 2 (17), drawn from the

;  last byte upwards; each layer's first line is the previous layer's last.

;-----------------------------------------------------------------------------

BOB_11:         .byte $00               ; ........

.byte $18               ; ...##...

.byte $3C               ; ..####..

.byte $FF               ; ########

.byte $00               ; ........

.byte $00               ; ........

.byte $00               ; ........

.byte $00               ; ........

.byte $00               ; ........

.byte $00               ; ........

.byte $18               ; ...##...

.byte $18               ; ...##...

.byte $3C               ; ..####..

.byte $6E               ; .##.###.

.byte $00               ; ........

.byte $00               ; ........

.byte $7E               ; .######.

.byte $7E               ; .######.

.byte $38               ; ..###...

.byte $00               ; ........

.byte $18               ; ...##...

.byte $3C               ; ..####..

.byte $FF               ; ########

.byte $3C               ; ..####..

.byte $34               ; ..##.#..

.byte $3E               ; ..#####.

.byte $3C               ; ..####..

.byte $30               ; ..##....

.byte $1C               ; ...###..

.byte $00               ; ........

.byte $00               ; ........

.byte $00               ; ........

.byte $00               ; ........

.byte $5E               ; .#.####.

.byte $3E               ; ..#####.

.byte $00               ; ........

.byte $00               ; ........

.byte $00               ; ........

.byte $00               ; ........

.byte $01               ; .......#

.byte $01               ; .......#

.byte $02               ; ......#.

.byte $10               ; ...#....

.byte $20               ; ..#.....

.byte $40               ; .#......

.byte $80               ; #.......

.byte $80               ; #.......

.byte $05               ; .....#.#

.byte $57               ; .#.#.###

.byte $72               ; .###..#.

.byte $20

;-----------------------------------------------------------------------------

;  Bob frame 12 (jump left): player 0 (20 lines), player 1 (17), player 2 (17), drawn from the

;  last byte upwards; each layer's first line is the previous layer's last.

;-----------------------------------------------------------------------------

BOB_12:         .byte $00               ; ........

.byte $18               ; ...##...

.byte $3C               ; ..####..

.byte $FF               ; ########

.byte $00               ; ........

.byte $00               ; ........

.byte $00               ; ........

.byte $00               ; ........

.byte $00               ; ........

.byte $00               ; ........

.byte $18               ; ...##...

.byte $18               ; ...##...

.byte $3C               ; ..####..

.byte $76               ; .###.##.

.byte $00               ; ........

.byte $00               ; ........

.byte $7E               ; .######.

.byte $7E               ; .######.

.byte $1C               ; ...###..

.byte $00               ; ........

.byte $18               ; ...##...

.byte $3C               ; ..####..

.byte $FF               ; ########

.byte $3C               ; ..####..

.byte $2C               ; ..#.##..

.byte $7C               ; .#####..

.byte $3C               ; ..####..

.byte $0C               ; ....##..

.byte $38               ; ..###...

.byte $00               ; ........

.byte $00               ; ........

.byte $00               ; ........

.byte $00               ; ........

.byte $7A               ; .####.#.

.byte $7C               ; .#####..

.byte $00               ; ........

.byte $00               ; ........

.byte $00               ; ........

.byte $00               ; ........

.byte $80               ; #.......

.byte $80               ; #.......

.byte $40               ; .#......

.byte $08               ; ....#...

.byte $04               ; .....#..

.byte $02               ; ......#.

.byte $01               ; .......#

.byte $01               ; .......#

.byte $A0               ; #.#.....

.byte $EA               ; ###.#.#.

.byte $4E               ; .#..###.

;-----------------------------------------------------------------------------

;  ITEM_SHAPES - addresses of the 37 item pictures (index = item shape number). Entries

;  33-36 (the goblet and the three TNT bundles) are the first 8 bytes of STATION_TABLE,

;  whose entry 0 is never used as a station: the two tables overlap.

;    1-5   the points numbers "100" .. "500"       11  "1100"      26  the "BF" logo

;-----------------------------------------------------------------------------

ITEM_SHAPES:    .word $0004,ITEM_1,ITEM_2,ITEM_3

.word ITEM_4,ITEM_5,ITEM_6,ITEM_7

.word ITEM_8,ITEM_9,ITEM_10,ITEM_11

.word ITEM_12,ITEM_13,ITEM_14,ITEM_15

.word ITEM_16,ITEM_17,ITEM_18,ITEM_19

.word ITEM_20,ITEM_21,ITEM_22,ITEM_23

.word ITEM_24,ITEM_25,ITEM_26,ITEM_27

.word ITEM_28,ITEM_29,ITEM_30,ITEM_31

.word ITEM_32

;-----------------------------------------------------------------------------

;  STATION_TABLE - per station (1-11), the addresses of its ladder, slide, moving-platform

;  and girder lists. Entry 0 is ITEM_SHAPES 33-36. The last entry's girder address is

;  also the first word of ITEM_LISTS.

;-----------------------------------------------------------------------------

STATION_TABLE:  .word ITEM_33,ITEM_34,ITEM_35,ITEM_36

.word LADDERS_1,EMPTY_LIST,EMPTY_LIST,GIRDERS_1

.word LADDERS_2,SLIDES_2,EMPTY_LIST,GIRDERS_2

.word LADDERS_3,EMPTY_LIST,EMPTY_LIST,GIRDERS_3

.word LADDERS_4,EMPTY_LIST,EMPTY_LIST,GIRDERS_4

.word EMPTY_LIST,SLIDES_5,LIFTS_5,GIRDERS_5

.word LADDERS_6,EMPTY_LIST,LIFTS_6,GIRDERS_6

.word EMPTY_LIST,SLIDES_7,LIFTS_7,GIRDERS_7

.word LADDERS_8,SLIDES_8,LIFTS_8,GIRDERS_8

.word LADDERS_9,SLIDES_9,LIFTS_9,GIRDERS_9

.word LADDERS_10,SLIDES_10,EMPTY_LIST,GIRDERS_10

.word LADDERS_11,EMPTY_LIST,EMPTY_LIST

;-----------------------------------------------------------------------------

;  ITEM_LISTS - per station, the address of its item list.

;-----------------------------------------------------------------------------

ITEM_LISTS:     .word GIRDERS_11,ITEMLIST_1,ITEMLIST_2,ITEMLIST_3,ITEMLIST_4,ITEMLIST_5

.word ITEMLIST_6,ITEMLIST_7,ITEMLIST_8,ITEMLIST_9,ITEMLIST_10

;-----------------------------------------------------------------------------

;  MUTANT_LISTS - per station, the address of its 8 mutant records. Entry 11 (the credits)

;  is the first two bytes of MUTANT_SHAPES ($1E18): the 88 bytes there are read, but through

;  a mask of 0, so the credits get no mutants.

;-----------------------------------------------------------------------------

MUTANT_LISTS:   .word EMPTY_LIST,MUTANTS_1,MUTANTS_2,MUTANTS_3,MUTANTS_4,MUTANTS_5

.word MUTANTS_6,MUTANTS_7,MUTANTS_8,MUTANTS_9,MUTANTS_10,$1E18

;-----------------------------------------------------------------------------

;  MUTANT_SHAPES - 10 shapes of 12 bytes (only 10 lines are copied), indexed by the

;  mutant's state: 0-1 facing right, 2-3 facing left, 4-7 the same when edible,

;  8-9 eaten (the numbers "80" and "90", the points scored). The table really starts at $8591 (state 0 = $8591), so

;  its first two

;  bytes are also the last word of MUTANT_LISTS, and its last two overlap the item list

;  of station 1.

;-----------------------------------------------------------------------------

MUTANT_SHAPES:  .byte $3E               ; ..#####.

.byte $39               ; ..###..#

.byte $4F               ; .#..####

.byte $74               ; .###.#..

.byte $3C               ; ..####..

.byte $18               ; ...##...

.byte $36               ; ..##.##.

.byte $33               ; ..##..##

.byte $00               ; ........

.byte $00               ; ........

.byte $18               ; ...##...

.byte $1E               ; ...####.

.byte $3E               ; ..#####.

.byte $39               ; ..###..#

.byte $4F               ; .#..####

.byte $74               ; .###.#..

.byte $3C               ; ..####..

.byte $18               ; ...##...

.byte $6C               ; .##.##..

.byte $CC               ; ##..##..

.byte $00               ; ........

.byte $00               ; ........

.byte $18               ; ...##...

.byte $78               ; .####...

.byte $7C               ; .#####..

.byte $9C               ; #..###..

.byte $F2               ; ####..#.

.byte $2E               ; ..#.###.

.byte $3C               ; ..####..

.byte $18               ; ...##...

.byte $6C               ; .##.##..

.byte $CC               ; ##..##..

.byte $00               ; ........

.byte $00               ; ........

.byte $18               ; ...##...

.byte $78               ; .####...

.byte $7C               ; .#####..

.byte $9C               ; #..###..

.byte $F2               ; ####..#.

.byte $2E               ; ..#.###.

.byte $3C               ; ..####..

.byte $18               ; ...##...

.byte $36               ; ..##.##.

.byte $33               ; ..##..##

.byte $00               ; ........

.byte $00               ; ........

.byte $18               ; ...##...

.byte $3C               ; ..####..

.byte $42               ; .#....#.

.byte $7E               ; .######.

.byte $5A               ; .#.##.#.

.byte $66               ; .##..##.

.byte $3C               ; ..####..

.byte $18               ; ...##...

.byte $66               ; .##..##.

.byte $C3               ; ##....##

.byte $00               ; ........

.byte $00               ; ........

.byte $18               ; ...##...

.byte $3C               ; ..####..

.byte $42               ; .#....#.

.byte $7E               ; .######.

.byte $5A               ; .#.##.#.

.byte $66               ; .##..##.

.byte $3C               ; ..####..

.byte $18               ; ...##...

.byte $18               ; ...##...

.byte $66               ; .##..##.

.byte $00               ; ........

.byte $00               ; ........

.byte $18               ; ...##...

.byte $3C               ; ..####..

.byte $42               ; .#....#.

.byte $7E               ; .######.

.byte $5A               ; .#.##.#.

.byte $66               ; .##..##.

.byte $3C               ; ..####..

.byte $18               ; ...##...

.byte $66               ; .##..##.

.byte $C3               ; ##....##

.byte $00               ; ........

.byte $00               ; ........

.byte $18               ; ...##...

.byte $3C               ; ..####..

.byte $42               ; .#....#.

.byte $7E               ; .######.

.byte $5A               ; .#.##.#.

.byte $66               ; .##..##.

.byte $3C               ; ..####..

.byte $18               ; ...##...

.byte $18               ; ...##...

.byte $66               ; .##..##.

.byte $00               ; ........

.byte $00               ; ........

.byte $EE               ; ###.###.

.byte $AA               ; #.#.#.#.

.byte $EA               ; ###.#.#.

.byte $AA               ; #.#.#.#.

.byte $EE               ; ###.###.

.byte $00               ; ........

.byte $00               ; ........

.byte $00               ; ........

.byte $00               ; ........

.byte $00               ; ........

.byte $00               ; ........

.byte $00               ; ........

.byte $EE               ; ###.###.

.byte $AA               ; #.#.#.#.

.byte $EA               ; ###.#.#.

.byte $2A               ; ..#.#.#.

.byte $2E               ; ..#.###.

.byte $00               ; ........

.byte $00               ; ........

.byte $00               ; ........

.byte $00               ; ........

.byte $00               ; ........

;-----------------------------------------------------------------------------

;  station 1 (The Mine Shaft) items: shape (+$80), y, x

;-----------------------------------------------------------------------------

ITEMLIST_1:     .byte $8D,$74,$08

.byte $86,$48,$40

.byte $88,$40,$8C

.byte $95,$19,$18

.byte $8F,$59,$90

.byte $96,$92,$85

;-----------------------------------------------------------------------------

;  EMPTY_LIST - the 0 that ends station 1's item list doubles as "no ladders / slides /

;  platforms" for the stations that have none.

;-----------------------------------------------------------------------------

EMPTY_LIST:     .byte $00               ; end of list

;-----------------------------------------------------------------------------

;  station 2 (The Slides) items: shape (+$80), y, x

;-----------------------------------------------------------------------------

ITEMLIST_2:     .byte $8A,$11,$44

.byte $91,$0C,$94

.byte $9B,$26,$78

.byte $90,$25,$3C

.byte $8C,$59,$1C

.byte $8D,$5A,$6E

.byte $8E,$AC,$36

.byte $00               ; end of list

;-----------------------------------------------------------------------------

;  station 3 (The Transporters) items: shape (+$80), y, x

;-----------------------------------------------------------------------------

ITEMLIST_3:     .byte $91,$3A,$04

.byte $94,$6C,$80

.byte $9F,$22,$20

.byte $93,$AA,$54

.byte $9E,$3D,$8C

.byte $9C,$42,$60

.byte $00               ; end of list

;-----------------------------------------------------------------------------

;  station 4 (The Lillipads) items: shape (+$80), y, x

;-----------------------------------------------------------------------------

ITEMLIST_4:     .byte $9B,$0A,$34

.byte $88,$57,$10

.byte $8C,$23,$4C

.byte $A1,$3C,$54

.byte $97,$46,$90

.byte $89,$8D,$90

.byte $A0,$A9,$20

.byte $98,$63,$50

.byte $00               ; end of list

;-----------------------------------------------------------------------------

;  station 5 (Advanced Lillipads) items: shape (+$80), y, x

;-----------------------------------------------------------------------------

ITEMLIST_5:     .byte $99,$88,$30

.byte $A1,$9E,$04

.byte $90,$0B,$30

.byte $92,$0A,$68

.byte $9D,$3A,$78

.byte $89,$3D,$18

.byte $A0,$0D,$4C

.byte $00               ; end of list

;-----------------------------------------------------------------------------

;  station 6 (Radioactive Waste) items: shape (+$80), y, x

;-----------------------------------------------------------------------------

ITEMLIST_6:     .byte $8C,$87,$88

.byte $8A,$08,$78

.byte $99,$25,$3C

.byte $A1,$0B,$10

.byte $8D,$30,$1E

.byte $86,$7D,$74

.byte $9B,$75,$10

.byte $00               ; end of list

;-----------------------------------------------------------------------------

;  station 7 (Advanced Transporters) items: shape (+$80), y, x

;-----------------------------------------------------------------------------

ITEMLIST_7:     .byte $9E,$79,$28

.byte $93,$95,$60

.byte $9C,$3E,$38

.byte $9A,$0A,$5C

.byte $91,$46,$00

.byte $94,$6E,$68

.byte $9F,$A0,$84

.byte $00               ; end of list

;-----------------------------------------------------------------------------

;  station 8 (The Lift) items: shape (+$80), y, x

;-----------------------------------------------------------------------------

ITEMLIST_8:     .byte $9B,$09,$29

.byte $A1,$08,$72

.byte $98,$42,$32

.byte $89,$61,$02

.byte $8A,$5E,$34

.byte $91,$34,$8B

.byte $00               ; end of list

;-----------------------------------------------------------------------------

;  station 9 (The Pulverizers) items: shape (+$80), y, x

;-----------------------------------------------------------------------------

ITEMLIST_9:     .byte $93,$A9,$88

.byte $91,$A4,$11

.byte $88,$86,$2C

.byte $86,$12,$2C

.byte $8E,$27,$2C

.byte $90,$3D,$2C

.byte $99,$55,$2C

.byte $8C,$70,$2C

.byte $00               ; end of list

;-----------------------------------------------------------------------------

;  station 10 (The Cannon) items: shape (+$80), y, x

;-----------------------------------------------------------------------------

ITEMLIST_10:    .byte $A2,$6C,$04

.byte $A2,$6C,$10

.byte $A2,$6C,$1C

.byte $A2,$82,$04

.byte $A2,$82,$10

.byte $A2,$82,$1C

.byte $A3,$9C,$04

.byte $A3,$9C,$10

.byte $A3,$9C,$1C

.byte $A4,$B3,$04

.byte $A4,$B3,$10

.byte $A4,$B3,$1C

.byte $00               ; end of list

;-----------------------------------------------------------------------------

;  item shape 1 - "100": 11 x 5 pixels, worth 100 points

;-----------------------------------------------------------------------------

ITEM_1:         .byte $0B,$05,$01       ; width, height, points (also the shape number of the points picture)

.byte $08,$A8,$A8       ; ..2. 222. 222.

.byte $08,$88,$88       ; ..2. 2.2. 2.2.

.byte $08,$88,$88       ; ..2. 2.2. 2.2.

.byte $08,$88,$88       ; ..2. 2.2. 2.2.

.byte $08,$A8,$A8       ; ..2. 222. 222.

;-----------------------------------------------------------------------------

;  item shape 2 - "200": 11 x 5 pixels, worth 100 points

;-----------------------------------------------------------------------------

ITEM_2:         .byte $0B,$05,$01       ; width, height, points (also the shape number of the points picture)

.byte $A8,$A8,$A8       ; 222. 222. 222.

.byte $08,$88,$88       ; ..2. 2.2. 2.2.

.byte $A8,$88,$88       ; 222. 2.2. 2.2.

.byte $80,$88,$88       ; 2... 2.2. 2.2.

.byte $A8,$A8,$A8       ; 222. 222. 222.

;-----------------------------------------------------------------------------

;  item shape 3 - "300": 11 x 5 pixels, worth 100 points

;-----------------------------------------------------------------------------

ITEM_3:         .byte $0B,$05,$01       ; width, height, points (also the shape number of the points picture)

.byte $A8,$A8,$A8       ; 222. 222. 222.

.byte $08,$88,$88       ; ..2. 2.2. 2.2.

.byte $A8,$88,$88       ; 222. 2.2. 2.2.

.byte $08,$88,$88       ; ..2. 2.2. 2.2.

.byte $A8,$A8,$A8       ; 222. 222. 222.

;-----------------------------------------------------------------------------

;  item shape 4 - "400": 11 x 5 pixels, worth 100 points

;-----------------------------------------------------------------------------

ITEM_4:         .byte $0B,$05,$01       ; width, height, points (also the shape number of the points picture)

.byte $88,$A8,$A8       ; 2.2. 222. 222.

.byte $88,$88,$88       ; 2.2. 2.2. 2.2.

.byte $A8,$88,$88       ; 222. 2.2. 2.2.

.byte $08,$88,$88       ; ..2. 2.2. 2.2.

.byte $08,$A8,$A8       ; ..2. 222. 222.

;-----------------------------------------------------------------------------

;  item shape 5 - "500": 11 x 5 pixels, worth 100 points

;-----------------------------------------------------------------------------

ITEM_5:         .byte $0B,$05,$01       ; width, height, points (also the shape number of the points picture)

.byte $A8,$A8,$A8       ; 222. 222. 222.

.byte $80,$88,$88       ; 2... 2.2. 2.2.

.byte $A8,$88,$88       ; 222. 2.2. 2.2.

.byte $08,$88,$88       ; ..2. 2.2. 2.2.

.byte $A8,$A8,$A8       ; 222. 222. 222.

;-----------------------------------------------------------------------------

;  item shape 6: 7 x 6 pixels, worth 100 points

;-----------------------------------------------------------------------------

ITEM_6:         .byte $07,$06,$01       ; width, height, points (also the shape number of the points picture)

.byte $55,$20           ; 1111 .2..

.byte $AA,$88           ; 2222 2.2.

.byte $55,$08           ; 1111 ..2.

.byte $55,$20           ; 1111 .2..

.byte $AA,$80           ; 2222 2...

.byte $14,$00           ; .11. ....

;-----------------------------------------------------------------------------

;  item shape 7: 4 x 11 pixels, worth 200 points

;-----------------------------------------------------------------------------

ITEM_7:         .byte $04,$0B,$02       ; width, height, points (also the shape number of the points picture)

.byte $28               ; .22.

.byte $28               ; .22.

.byte $AA               ; 2222

.byte $AA               ; 2222

.byte $AA               ; 2222

.byte $AA               ; 2222

.byte $AA               ; 2222

.byte $AA               ; 2222

.byte $AA               ; 2222

.byte $AA               ; 2222

.byte $AA               ; 2222

;-----------------------------------------------------------------------------

;  item shape 8: 7 x 5 pixels, worth 100 points

;-----------------------------------------------------------------------------

ITEM_8:         .byte $07,$05,$01       ; width, height, points (also the shape number of the points picture)

.byte $2A,$A0           ; .222 22..

.byte $AA,$A8           ; 2222 222.

.byte $66,$64           ; 1212 121.

.byte $55,$54           ; 1111 111.

.byte $15,$50           ; .111 11..

;-----------------------------------------------------------------------------

;  item shape 9: 8 x 9 pixels, worth 500 points

;-----------------------------------------------------------------------------

ITEM_9:         .byte $08,$09,$05       ; width, height, points (also the shape number of the points picture)

.byte $08,$00           ; ..2. ....

.byte $08,$00           ; ..2. ....

.byte $08,$01           ; ..2. ...1

.byte $08,$01           ; ..2. ...1

.byte $08,$04           ; ..2. ..1.

.byte $48,$01           ; 1.2. ...1

.byte $AA,$81           ; 2222 2..1

.byte $AA,$94           ; 2222 211.

.byte $AA,$80           ; 2222 2...

;-----------------------------------------------------------------------------

;  item shape 10: 8 x 6 pixels, worth 100 points

;-----------------------------------------------------------------------------

ITEM_10:        .byte $08,$06,$01       ; width, height, points (also the shape number of the points picture)

.byte $00,$50           ; .... 11..

.byte $01,$40           ; ...1 1...

.byte $05,$00           ; ..11 ....

.byte $AA,$AA           ; 2222 2222

.byte $AA,$AA           ; 2222 2222

.byte $2A,$A8           ; .222 222.

;-----------------------------------------------------------------------------

;  item shape 11 - "1100": 12 x 5 pixels, worth 000 points

;-----------------------------------------------------------------------------

ITEM_11:        .byte $0C,$05,$00       ; width, height, points (also the shape number of the points picture)

.byte $88,$A8,$A8       ; 2.2. 222. 222.

.byte $88,$88,$88       ; 2.2. 2.2. 2.2.

.byte $88,$88,$88       ; 2.2. 2.2. 2.2.

.byte $88,$88,$88       ; 2.2. 2.2. 2.2.

.byte $88,$A8,$A8       ; 2.2. 222. 222.

;-----------------------------------------------------------------------------

;  item shape 12: 8 x 6 pixels, worth 300 points

;-----------------------------------------------------------------------------

ITEM_12:        .byte $08,$06,$03       ; width, height, points (also the shape number of the points picture)

.byte $2A,$A8           ; .222 222.

.byte $AA,$AA           ; 2222 2222

.byte $15,$54           ; .111 111.

.byte $15,$54           ; .111 111.

.byte $AA,$AA           ; 2222 2222

.byte $2A,$A8           ; .222 222.

;-----------------------------------------------------------------------------

;  item shape 13: 4 x 9 pixels, worth 100 points

;-----------------------------------------------------------------------------

ITEM_13:        .byte $04,$09,$01       ; width, height, points (also the shape number of the points picture)

.byte $28               ; .22.

.byte $28               ; .22.

.byte $AA               ; 2222

.byte $AA               ; 2222

.byte $55               ; 1111

.byte $55               ; 1111

.byte $55               ; 1111

.byte $55               ; 1111

.byte $55               ; 1111

;-----------------------------------------------------------------------------

;  item shape 14: 8 x 12 pixels, worth 400 points

;-----------------------------------------------------------------------------

ITEM_14:        .byte $08,$0C,$04       ; width, height, points (also the shape number of the points picture)

.byte $5A,$A5           ; 1122 2211

.byte $16,$94           ; .112 211.

.byte $05,$50           ; ..11 11..

.byte $01,$40           ; ...1 1...

.byte $01,$40           ; ...1 1...

.byte $02,$A8           ; ...2 222.

.byte $02,$80           ; ...2 2...

.byte $01,$40           ; ...1 1...

.byte $01,$40           ; ...1 1...

.byte $01,$40           ; ...1 1...

.byte $01,$40           ; ...1 1...

.byte $01,$40           ; ...1 1...

;-----------------------------------------------------------------------------

;  item shape 15: 5 x 21 pixels, worth 200 points

;-----------------------------------------------------------------------------

ITEM_15:        .byte $05,$15,$02       ; width, height, points (also the shape number of the points picture)

.byte $2A,$00           ; .222 ....

.byte $80,$80           ; 2... 2...

.byte $22,$00           ; .2.2 ....

.byte $08,$00           ; ..2. ....

.byte $04,$00           ; ..1. ....

.byte $04,$00           ; ..1. ....

.byte $04,$00           ; ..1. ....

.byte $04,$00           ; ..1. ....

.byte $04,$00           ; ..1. ....

.byte $04,$00           ; ..1. ....

.byte $04,$00           ; ..1. ....

.byte $04,$00           ; ..1. ....

.byte $04,$00           ; ..1. ....

.byte $04,$00           ; ..1. ....

.byte $04,$00           ; ..1. ....

.byte $15,$00           ; .111 ....

.byte $55,$40           ; 1111 1...

.byte $55,$40           ; 1111 1...

.byte $55,$40           ; 1111 1...

.byte $15,$00           ; .111 ....

.byte $04,$00           ; ..1. ....

;-----------------------------------------------------------------------------

;  item shape 16: 10 x 14 pixels, worth 200 points

;-----------------------------------------------------------------------------

ITEM_16:        .byte $0A,$0E,$02       ; width, height, points (also the shape number of the points picture)

.byte $00,$A0,$00       ; .... 22.. ....

.byte $2A,$AA,$80       ; .222 2222 2...

.byte $28,$A2,$80       ; .22. 22.2 2...

.byte $80,$50,$20       ; 2... 11.. .2..

.byte $00,$50,$00       ; .... 11.. ....

.byte $00,$50,$00       ; .... 11.. ....

.byte $00,$50,$00       ; .... 11.. ....

.byte $00,$50,$00       ; .... 11.. ....

.byte $00,$50,$00       ; .... 11.. ....

.byte $00,$50,$00       ; .... 11.. ....

.byte $00,$50,$00       ; .... 11.. ....

.byte $00,$50,$00       ; .... 11.. ....

.byte $00,$50,$00       ; .... 11.. ....

.byte $00,$50,$00       ; .... 11.. ....

;-----------------------------------------------------------------------------

;  item shape 17: 6 x 13 pixels, worth 200 points

;-----------------------------------------------------------------------------

ITEM_17:        .byte $06,$0D,$02       ; width, height, points (also the shape number of the points picture)

.byte $20,$80           ; .2.. 2...

.byte $A0,$A0           ; 22.. 22..

.byte $80,$20           ; 2... .2..

.byte $80,$20           ; 2... .2..

.byte $2A,$80           ; .222 2...

.byte $0A,$00           ; ..22 ....

.byte $20,$80           ; .2.. 2...

.byte $80,$20           ; 2... .2..

.byte $80,$20           ; 2... .2..

.byte $80,$20           ; 2... .2..

.byte $80,$20           ; 2... .2..

.byte $80,$20           ; 2... .2..

.byte $80,$20           ; 2... .2..

;-----------------------------------------------------------------------------

;  item shape 18: 8 x 12 pixels, worth 300 points

;-----------------------------------------------------------------------------

ITEM_18:        .byte $08,$0C,$03       ; width, height, points (also the shape number of the points picture)

.byte $02,$AA           ; ...2 2222

.byte $0A,$AA           ; ..22 2222

.byte $28,$5A           ; .22. 1122

.byte $A0,$50           ; 22.. 11..

.byte $00,$50           ; .... 11..

.byte $00,$50           ; .... 11..

.byte $00,$50           ; .... 11..

.byte $00,$50           ; .... 11..

.byte $00,$50           ; .... 11..

.byte $00,$50           ; .... 11..

.byte $00,$50           ; .... 11..

.byte $00,$50           ; .... 11..

;-----------------------------------------------------------------------------

;  item shape 19: 8 x 6 pixels, worth 100 points

;-----------------------------------------------------------------------------

ITEM_19:        .byte $08,$06,$01       ; width, height, points (also the shape number of the points picture)

.byte $2A,$80           ; .222 2...

.byte $80,$20           ; 2... .2..

.byte $80,$20           ; 2... .2..

.byte $20,$80           ; .2.. 2...

.byte $AA,$A8           ; 2222 222.

.byte $AA,$AA           ; 2222 2222

;-----------------------------------------------------------------------------

;  item shape 20: 8 x 4 pixels, worth 100 points

;-----------------------------------------------------------------------------

ITEM_20:        .byte $08,$04,$01       ; width, height, points (also the shape number of the points picture)

.byte $2A,$A8           ; .222 222.

.byte $82,$82           ; 2..2 2..2

.byte $82,$82           ; 2..2 2..2

.byte $28,$28           ; .22. .22.

;-----------------------------------------------------------------------------

;  item shape 21: 8 x 17 pixels, worth 400 points

;-----------------------------------------------------------------------------

ITEM_21:        .byte $08,$11,$04       ; width, height, points (also the shape number of the points picture)

.byte $00,$20           ; .... .2..

.byte $00,$A8           ; .... 222.

.byte $16,$AA           ; .112 2222

.byte $AA,$AA           ; 2222 2222

.byte $AA,$AA           ; 2222 2222

.byte $AA,$AA           ; 2222 2222

.byte $AA,$A8           ; 2222 222.

.byte $14,$20           ; .11. .2..

.byte $14,$00           ; .11. ....

.byte $14,$00           ; .11. ....

.byte $14,$00           ; .11. ....

.byte $14,$00           ; .11. ....

.byte $14,$00           ; .11. ....

.byte $14,$00           ; .11. ....

.byte $14,$00           ; .11. ....

.byte $14,$00           ; .11. ....

.byte $14,$00           ; .11. ....

;-----------------------------------------------------------------------------

;  item shape 22: 8 x 4 pixels, worth 200 points

;-----------------------------------------------------------------------------

ITEM_22:        .byte $08,$04,$02       ; width, height, points (also the shape number of the points picture)

.byte $0A,$A0           ; ..22 22..

.byte $5A,$A5           ; 1122 2211

.byte $5A,$A5           ; 1122 2211

.byte $0A,$A0           ; ..22 22..

;-----------------------------------------------------------------------------

;  item shape 23: 8 x 12 pixels, worth 300 points

;-----------------------------------------------------------------------------

ITEM_23:        .byte $08,$0C,$03       ; width, height, points (also the shape number of the points picture)

.byte $02,$80           ; ...2 2...

.byte $22,$88           ; .2.2 2.2.

.byte $22,$88           ; .2.2 2.2.

.byte $22,$88           ; .2.2 2.2.

.byte $55,$55           ; 1111 1111

.byte $55,$55           ; 1111 1111

.byte $AA,$AA           ; 2222 2222

.byte $55,$55           ; 1111 1111

.byte $55,$55           ; 1111 1111

.byte $AA,$AA           ; 2222 2222

.byte $55,$55           ; 1111 1111

.byte $55,$55           ; 1111 1111

;-----------------------------------------------------------------------------

;  item shape 24: 10 x 12 pixels, worth 300 points

;-----------------------------------------------------------------------------

ITEM_24:        .byte $0A,$0C,$03       ; width, height, points (also the shape number of the points picture)

.byte $00,$02,$00       ; .... ...2 ....

.byte $08,$0A,$80       ; ..2. ..22 2...

.byte $2A,$29,$A0       ; .222 .221 22..

.byte $A6,$0A,$80       ; 2212 ..22 2...

.byte $29,$12,$00       ; .221 .1.2 ....

.byte $09,$40,$00       ; ..21 1... ....

.byte $01,$40,$00       ; ...1 1... ....

.byte $2A,$A8,$00       ; .222 222. ....

.byte $2A,$A8,$00       ; .222 222. ....

.byte $0A,$A0,$00       ; ..22 22.. ....

.byte $0A,$A0,$00       ; ..22 22.. ....

.byte $0A,$A0,$00       ; ..22 22.. ....

;-----------------------------------------------------------------------------

;  item shape 25: 11 x 8 pixels, worth 200 points

;-----------------------------------------------------------------------------

ITEM_25:        .byte $0B,$08,$02       ; width, height, points (also the shape number of the points picture)

.byte $05,$50,$00       ; ..11 11.. ....

.byte $04,$10,$00       ; ..1. .1.. ....

.byte $10,$04,$28       ; .1.. ..1. .22.

.byte $2A,$A8,$A0       ; .222 222. 22..

.byte $AA,$AA,$80       ; 2222 2222 2...

.byte $AA,$AA,$00       ; 2222 2222 ....

.byte $AA,$AA,$00       ; 2222 2222 ....

.byte $2A,$A8,$00       ; .222 222. ....

;-----------------------------------------------------------------------------

;  item shape 26 - the "BF" (Big Five) logo: 9 x 7 pixels, worth 1100 points

;-----------------------------------------------------------------------------

ITEM_26:        .byte $09,$07,$0B       ; width, height, points (also the shape number of the points picture)

.byte $A8,$3F,$C0       ; 222. .333 3...

.byte $82,$30,$00       ; 2..2 .3.. ....

.byte $A8,$30,$00       ; 222. .3.. ....

.byte $82,$3F,$00       ; 2..2 .333 ....

.byte $82,$30,$00       ; 2..2 .3.. ....

.byte $82,$30,$00       ; 2..2 .3.. ....

.byte $A8,$30,$00       ; 222. .3.. ....

;-----------------------------------------------------------------------------

;  item shape 27: 9 x 9 pixels, worth 200 points

;-----------------------------------------------------------------------------

ITEM_27:        .byte $09,$09,$02       ; width, height, points (also the shape number of the points picture)

.byte $00,$01,$00       ; .... ...1 ....

.byte $00,$08,$00       ; .... ..2. ....

.byte $00,$08,$00       ; .... ..2. ....

.byte $00,$08,$00       ; .... ..2. ....

.byte $00,$08,$00       ; .... ..2. ....

.byte $10,$08,$00       ; .1.. ..2. ....

.byte $44,$08,$00       ; 1.1. ..2. ....

.byte $41,$04,$00       ; 1..1 ..1. ....

.byte $15,$55,$40       ; .111 1111 1...

;-----------------------------------------------------------------------------

;  item shape 28: 11 x 10 pixels, worth 300 points

;-----------------------------------------------------------------------------

ITEM_28:        .byte $0B,$0A,$03       ; width, height, points (also the shape number of the points picture)

.byte $AA,$AA,$A8       ; 2222 2222 222.

.byte $AA,$AA,$A0       ; 2222 2222 22..

.byte $AA,$AA,$80       ; 2222 2222 2...

.byte $AA,$A0,$00       ; 2222 22.. ....

.byte $A2,$A0,$00       ; 22.2 22.. ....

.byte $02,$A0,$00       ; ...2 22.. ....

.byte $02,$A0,$00       ; ...2 22.. ....

.byte $02,$A0,$00       ; ...2 22.. ....

.byte $2A,$AA,$A0       ; .222 2222 22..

.byte $AA,$AA,$A8       ; 2222 2222 222.

;-----------------------------------------------------------------------------

;  item shape 29: 9 x 20 pixels, worth 500 points

;-----------------------------------------------------------------------------

ITEM_29:        .byte $09,$14,$05       ; width, height, points (also the shape number of the points picture)

.byte $10,$01,$00       ; .1.. ...1 ....

.byte $10,$01,$00       ; .1.. ...1 ....

.byte $10,$41,$00       ; .1.. 1..1 ....

.byte $10,$41,$00       ; .1.. 1..1 ....

.byte $10,$41,$00       ; .1.. 1..1 ....

.byte $10,$41,$00       ; .1.. 1..1 ....

.byte $A8,$4A,$80       ; 222. 1.22 2...

.byte $20,$42,$00       ; .2.. 1..2 ....

.byte $22,$A2,$00       ; .2.2 22.2 ....

.byte $20,$82,$00       ; .2.. 2..2 ....

.byte $20,$82,$00       ; .2.. 2..2 ....

.byte $20,$82,$00       ; .2.. 2..2 ....

.byte $0A,$A8,$00       ; ..22 222. ....

.byte $02,$A0,$00       ; ...2 22.. ....

.byte $00,$80,$00       ; .... 2... ....

.byte $00,$80,$00       ; .... 2... ....

.byte $00,$80,$00       ; .... 2... ....

.byte $00,$80,$00       ; .... 2... ....

.byte $02,$A0,$00       ; ...2 22.. ....

.byte $0A,$A8,$00       ; ..22 222. ....

;-----------------------------------------------------------------------------

;  item shape 30: 9 x 11 pixels, worth 200 points

;-----------------------------------------------------------------------------

ITEM_30:        .byte $09,$0B,$02       ; width, height, points (also the shape number of the points picture)

.byte $2A,$AA,$00       ; .222 2222 ....

.byte $88,$88,$80       ; 2.2. 2.2. 2...

.byte $20,$82,$00       ; .2.. 2..2 ....

.byte $02,$A0,$00       ; ...2 22.. ....

.byte $02,$A0,$00       ; ...2 22.. ....

.byte $02,$A0,$00       ; ...2 22.. ....

.byte $02,$A0,$00       ; ...2 22.. ....

.byte $00,$80,$00       ; .... 2... ....

.byte $00,$80,$00       ; .... 2... ....

.byte $00,$80,$00       ; .... 2... ....

.byte $00,$80,$00       ; .... 2... ....

;-----------------------------------------------------------------------------

;  item shape 31: 7 x 11 pixels, worth 300 points

;-----------------------------------------------------------------------------

ITEM_31:        .byte $07,$0B,$03       ; width, height, points (also the shape number of the points picture)

.byte $2A,$A0           ; .222 22..

.byte $02,$00           ; ...2 ....

.byte $02,$00           ; ...2 ....

.byte $22,$20           ; .2.2 .2..

.byte $22,$20           ; .2.2 .2..

.byte $AA,$A8           ; 2222 222.

.byte $AA,$A8           ; 2222 222.

.byte $AA,$A8           ; 2222 222.

.byte $AA,$A8           ; 2222 222.

.byte $AA,$A8           ; 2222 222.

.byte $AA,$A8           ; 2222 222.

;-----------------------------------------------------------------------------

;  item shape 32: 11 x 11 pixels, worth 500 points

;-----------------------------------------------------------------------------

ITEM_32:        .byte $0B,$0B,$05       ; width, height, points (also the shape number of the points picture)

.byte $AA,$AA,$A8       ; 2222 2222 222.

.byte $AA,$9A,$A8       ; 2222 2122 222.

.byte $AA,$56,$A8       ; 2222 1112 222.

.byte $A9,$99,$A8       ; 2221 2121 222.

.byte $A9,$9A,$A8       ; 2221 2122 222.

.byte $AA,$56,$A8       ; 2222 1112 222.

.byte $AA,$99,$A8       ; 2222 2121 222.

.byte $A9,$99,$A8       ; 2221 2121 222.

.byte $AA,$56,$A8       ; 2222 1112 222.

.byte $AA,$9A,$A8       ; 2222 2122 222.

.byte $AA,$AA,$A8       ; 2222 2222 222.

;-----------------------------------------------------------------------------

;  item shape 33 - goblet (poisoned on station 5): 7 x 9 pixels, worth 200 points

;-----------------------------------------------------------------------------

ITEM_33:        .byte $07,$09,$02       ; width, height, points (also the shape number of the points picture)

.byte $95,$58           ; 2111 112.

.byte $25,$60           ; .211 12..

.byte $0A,$80           ; ..22 2...

.byte $02,$00           ; ...2 ....

.byte $02,$00           ; ...2 ....

.byte $02,$00           ; ...2 ....

.byte $02,$00           ; ...2 ....

.byte $0A,$80           ; ..22 2...

.byte $2A,$A0           ; .222 22..

;-----------------------------------------------------------------------------

;  item shape 34 - TNT, 3 tons: 7 x 15 pixels, worth 300 points

;-----------------------------------------------------------------------------

ITEM_34:        .byte $07,$0F,$03       ; width, height, points (also the shape number of the points picture)

.byte $02,$00           ; ...2 ....

.byte $02,$00           ; ...2 ....

.byte $0A,$80           ; ..22 2...

.byte $0A,$80           ; ..22 2...

.byte $0A,$80           ; ..22 2...

.byte $0A,$80           ; ..22 2...

.byte $0A,$80           ; ..22 2...

.byte $00,$00           ; .... ....

.byte $20,$20           ; .2.. .2..

.byte $20,$20           ; .2.. .2..

.byte $A8,$A8           ; 222. 222.

.byte $A8,$A8           ; 222. 222.

.byte $A8,$A8           ; 222. 222.

.byte $A8,$A8           ; 222. 222.

.byte $A8,$A8           ; 222. 222.

;-----------------------------------------------------------------------------

;  item shape 35 - TNT, 2 tons: 7 x 7 pixels, worth 200 points

;-----------------------------------------------------------------------------

ITEM_35:        .byte $07,$07,$02       ; width, height, points (also the shape number of the points picture)

.byte $20,$20           ; .2.. .2..

.byte $20,$20           ; .2.. .2..

.byte $A8,$A8           ; 222. 222.

.byte $A8,$A8           ; 222. 222.

.byte $A8,$A8           ; 222. 222.

.byte $A8,$A8           ; 222. 222.

.byte $A8,$A8           ; 222. 222.

;-----------------------------------------------------------------------------

;  item shape 36 - TNT, 1 ton: 5 x 7 pixels, worth 100 points

;-----------------------------------------------------------------------------

ITEM_36:        .byte $05,$07,$01       ; width, height, points (also the shape number of the points picture)

.byte $02,$00           ; ...2 ....

.byte $02,$00           ; ...2 ....

.byte $0A,$80           ; ..22 2...

.byte $0A,$80           ; ..22 2...

.byte $0A,$80           ; ..22 2...

.byte $0A,$80           ; ..22 2...

.byte $0A,$80           ; ..22 2...

;-----------------------------------------------------------------------------

;  station 1 (The Mine Shaft) mutants: 8 records of 11 bytes in the MUT_TAB layout; the y, step

;  timer, patrol length and colour timer are re-read from here while playing.

;-----------------------------------------------------------------------------

MUTANTS_1:      .byte $3D,$26,$01,$00,$04,$02,$32,$01,$07,$32,$00

.byte $99,$26,$03,$00,$03,$02,$22,$FF,$05,$3C,$00

.byte $00,$00,$00,$00,$00,$00,$00,$00,$00,$00,$00

.byte $00,$00,$00,$00,$00,$00,$00,$00,$00,$00,$00

.byte $41,$8A,$00,$00,$02,$FE,$34,$01,$08,$28,$00

.byte $00,$00,$00,$00,$00,$00,$00,$00,$00,$00,$00

.byte $00,$00,$00,$00,$00,$00,$00,$00,$00,$00,$00

.byte $AF,$B5,$03,$00,$03,$02,$7F,$FF,$05,$46,$00

;-----------------------------------------------------------------------------

;  station 2 (The Slides) mutants: 8 records of 11 bytes in the MUT_TAB layout; the y, step

;  timer, patrol length and colour timer are re-read from here while playing.

;-----------------------------------------------------------------------------

MUTANTS_2:      .byte $41,$11,$02,$00,$02,$02,$2B,$01,$05,$1E,$00

.byte $A7,$11,$01,$00,$04,$FE,$24,$FF,$06,$14,$00

.byte $00,$00,$00,$00,$00,$00,$00,$00,$00,$00,$00

.byte $AA,$48,$00,$00,$02,$02,$04,$01,$06,$38,$00

.byte $00,$00,$00,$00,$00,$00,$00,$00,$00,$00,$00

.byte $00,$00,$00,$00,$00,$00,$00,$00,$00,$00,$00

.byte $4B,$B5,$03,$00,$04,$02,$13,$01,$07,$2C,$00

.byte $C3,$B5,$00,$00,$03,$FE,$45,$FF,$05,$54,$00

;-----------------------------------------------------------------------------

;  station 3 (The Transporters) mutants: 8 records of 11 bytes in the MUT_TAB layout; the y, step

;  timer, patrol length and colour timer are re-read from here while playing.

;-----------------------------------------------------------------------------

MUTANTS_3:      .byte $4B,$28,$01,$00,$04,$FE,$20,$01,$05,$1E,$00

.byte $96,$28,$03,$00,$02,$02,$12,$FF,$08,$1E,$00

.byte $36,$43,$00,$00,$03,$FE,$1F,$01,$06,$46,$00

.byte $A4,$45,$02,$00,$02,$FE,$20,$01,$05,$5A,$00

.byte $51,$77,$00,$00,$04,$02,$11,$01,$08,$1E,$00

.byte $91,$73,$03,$00,$02,$FE,$2D,$01,$06,$50,$00

.byte $45,$B5,$01,$00,$03,$02,$84,$01,$07,$0A,$00

.byte $00,$00,$00,$00,$00,$00,$00,$00,$00,$00,$00

;-----------------------------------------------------------------------------

;  station 4 (The Lillipads) mutants: 8 records of 11 bytes in the MUT_TAB layout; the y, step

;  timer, patrol length and colour timer are re-read from here while playing.

;-----------------------------------------------------------------------------

MUTANTS_4:      .byte $3C,$11,$00,$00,$04,$FE,$3C,$01,$05,$28,$00

.byte $BB,$11,$03,$00,$03,$02,$3B,$FF,$08,$05,$00

.byte $6F,$50,$01,$00,$02,$FE,$0B,$01,$07,$46,$00

.byte $A7,$48,$02,$00,$03,$02,$08,$FF,$06,$64,$00

.byte $00,$00,$00,$00,$00,$00,$00,$00,$00,$00,$00

.byte $B5,$86,$01,$00,$03,$FE,$07,$FF,$05,$82,$00

.byte $62,$A3,$01,$00,$03,$02,$10,$01,$07,$1E,$00

.byte $C7,$B5,$02,$00,$04,$FE,$97,$FF,$05,$3C,$00

;-----------------------------------------------------------------------------

;  station 5 (Advanced Lillipads) mutants: 8 records of 11 bytes in the MUT_TAB layout; the y, step

;  timer, patrol length and colour timer are re-read from here while playing.

;-----------------------------------------------------------------------------

MUTANTS_5:      .byte $38,$13,$01,$00,$02,$FE,$40,$01,$05,$28,$00

.byte $C4,$13,$03,$00,$03,$02,$44,$FF,$07,$50,$00

.byte $36,$4E,$00,$00,$03,$02,$08,$01,$06,$3C,$00

.byte $00,$00,$00,$00,$00,$00,$00,$00,$00,$00,$00

.byte $00,$00,$00,$00,$00,$00,$00,$00,$00,$00,$00

.byte $00,$00,$00,$00,$00,$00,$00,$00,$00,$00,$00

.byte $40,$B5,$01,$00,$04,$FE,$40,$01,$05,$14,$00

.byte $81,$B5,$03,$00,$03,$02,$47,$01,$07,$1E,$00

;-----------------------------------------------------------------------------

;  station 6 (Radioactive Waste) mutants: 8 records of 11 bytes in the MUT_TAB layout; the y, step

;  timer, patrol length and colour timer are re-read from here while playing.

;-----------------------------------------------------------------------------

MUTANTS_6:      .byte $34,$13,$01,$00,$03,$FE,$10,$01,$05,$3C,$00

.byte $B8,$13,$03,$00,$02,$02,$10,$01,$07,$1E,$00

.byte $00,$00,$00,$00,$00,$00,$00,$00,$00,$00,$00

.byte $00,$00,$00,$00,$00,$00,$00,$00,$00,$00,$00

.byte $65,$71,$00,$00,$04,$FE,$04,$01,$06,$0A,$00

.byte $00,$00,$00,$00,$00,$00,$00,$00,$00,$00,$00

.byte $00,$00,$00,$00,$00,$00,$00,$00,$00,$00,$00

.byte $C6,$9B,$01,$00,$05,$02,$05,$01,$06,$FE,$00

;-----------------------------------------------------------------------------

;  station 7 (Advanced Transporters) mutants: 8 records of 11 bytes in the MUT_TAB layout; the y, step

;  timer, patrol length and colour timer are re-read from here while playing.

;-----------------------------------------------------------------------------

MUTANTS_7:      .byte $80,$28,$00,$00,$03,$FE,$0D,$01,$06,$05,$00

.byte $00,$00,$00,$00,$00,$00,$00,$00,$00,$00,$00

.byte $00,$00,$00,$00,$00,$00,$00,$00,$00,$00,$00

.byte $00,$00,$00,$00,$00,$00,$00,$00,$00,$00,$00

.byte $4C,$86,$02,$00,$02,$02,$14,$FF,$06,$1E,$00

.byte $00,$00,$00,$00,$00,$00,$00,$00,$00,$00,$00

.byte $4F,$B5,$01,$00,$04,$FE,$31,$01,$05,$5A,$00

.byte $38,$B5,$02,$00,$03,$02,$47,$01,$08,$C8,$00

;-----------------------------------------------------------------------------

;  station 8 (The Lift) mutants: 8 records of 11 bytes in the MUT_TAB layout; the y, step

;  timer, patrol length and colour timer are re-read from here while playing.

;-----------------------------------------------------------------------------

MUTANTS_8:      .byte $33,$14,$00,$00,$03,$02,$57,$01,$03,$1E,$00

.byte $C6,$14,$02,$00,$04,$FE,$46,$FF,$02,$3C,$00

.byte $00,$00,$00,$00,$00,$00,$00,$00,$00,$00,$00

.byte $32,$50,$00,$00,$02,$02,$2A,$01,$04,$5A,$00

.byte $00,$00,$00,$00,$00,$00,$00,$00,$00,$00,$00

.byte $38,$6E,$00,$00,$04,$02,$20,$01,$05,$78,$00

.byte $00,$00,$00,$00,$00,$00,$00,$00,$00,$00,$00

.byte $50,$9C,$02,$00,$05,$FE,$19,$FF,$01,$96,$00

;-----------------------------------------------------------------------------

;  station 9 (The Pulverizers) mutants: 8 records of 11 bytes in the MUT_TAB layout; the y, step

;  timer, patrol length and colour timer are re-read from here while playing.

;-----------------------------------------------------------------------------

MUTANTS_9:      .byte $32,$10,$00,$00,$02,$02,$12,$01,$03,$01,$00

.byte $44,$28,$02,$00,$03,$FE,$12,$FF,$03,$02,$00

.byte $32,$40,$00,$00,$04,$02,$12,$01,$03,$03,$00

.byte $44,$58,$02,$00,$02,$FE,$12,$FF,$03,$04,$00

.byte $32,$70,$00,$00,$03,$02,$12,$01,$03,$05,$00

.byte $44,$88,$02,$00,$04,$FE,$12,$FF,$03,$06,$00

.byte $34,$B5,$00,$00,$03,$02,$11,$01,$06,$04,$00

.byte $BC,$B5,$02,$00,$03,$FE,$60,$FF,$08,$04,$00

;-----------------------------------------------------------------------------

;  station 10 (The Cannon) mutants: 8 records of 11 bytes in the MUT_TAB layout; the y, step

;  timer, patrol length and colour timer are re-read from here while playing.

;-----------------------------------------------------------------------------

MUTANTS_10:     .byte $31,$18,$00,$00,$02,$02,$4B,$01,$07,$03,$00

.byte $C4,$18,$02,$00,$04,$FE,$28,$FF,$06,$02,$00

.byte $7D,$42,$02,$00,$02,$02,$4B,$FF,$08,$03,$00

.byte $9D,$42,$00,$00,$04,$FE,$2C,$01,$07,$03,$00

.byte $7E,$6C,$02,$00,$02,$02,$16,$FF,$0A,$05,$00

.byte $9C,$6C,$00,$00,$04,$FE,$2A,$01,$09,$06,$00

.byte $00,$00,$00,$00,$00,$00,$00,$00,$00,$00,$00

.byte $00

;-----------------------------------------------------------------------------

;  station 1 (The Mine Shaft) girders: sections, x, y - three lists: sloping down, sloping up, flat

;-----------------------------------------------------------------------------

GIRDERS_1:      .byte $12,$0C,$48

.byte $11,$50,$70

.byte $12,$48,$95

.byte $00               ; end of the down-sloping girders

.byte $10,$54,$58

.byte $0D,$1C,$7D

.byte $08,$4C,$B4

.byte $04,$0C,$B8

.byte $00               ; end of the up-sloping girders

.byte $24,$08,$30

.byte $12,$00,$94

.byte $0C,$1C,$B5

.byte $0D,$6C,$AD

.byte $02,$08,$82

.byte $00               ; end of the flat girders

;-----------------------------------------------------------------------------

;  station 1 (The Mine Shaft) ladders: height, x, y, style

;-----------------------------------------------------------------------------

LADDERS_1:      .byte $1C,$5C,$35,$04

.byte $18,$B0,$35,$04

.byte $14,$7E,$5D,$04

.byte $19,$64,$7B,$03

.byte $23,$A8,$7F,$02

.byte $1C,$52,$99,$04

.byte $0D,$9E,$B2,$03

;-----------------------------------------------------------------------------

;  station 2 (The Slides) girders: sections, x, y - three lists: sloping down, sloping up, flat

;-----------------------------------------------------------------------------

GIRDERS_2:      .byte $00               ; end of list; end of the down-sloping girders

.byte $00               ; end of the up-sloping girders

.byte $28,$00,$1B

.byte $07,$00,$35

.byte $07,$28,$35

.byte $14,$50,$35

.byte $08,$00,$67

.byte $0A,$2C,$6A

.byte $10,$60,$69

.byte $09,$0E,$9E

.byte $19,$3B,$9C

.byte $02,$7D,$52

.byte $00               ; end of the flat girders

;-----------------------------------------------------------------------------

;  station 2 (The Slides) ladders: height, x, y, style

;-----------------------------------------------------------------------------

LADDERS_2:      .byte $15,$3A,$20,$01

.byte $15,$62,$20,$01

.byte $15,$89,$20,$01

.byte $15,$BA,$20,$01

.byte $2D,$32,$3A,$01

.byte $32,$3E,$6C,$02

.byte $2E,$AA,$6E,$02

.byte $1E,$75,$A1,$02

.byte $00               ; end of list

;-----------------------------------------------------------------------------

;  station 2 (The Slides) slides: rows, x, y, step

;-----------------------------------------------------------------------------

SLIDES_2:       .byte $35,$A8,$20,$FF

.byte $32,$5F,$55,$01

.byte $1A,$C8,$06,$00

.byte $1D,$C8,$20,$FF

.byte $17,$AD,$3D,$01

.byte $33,$C2,$54,$00

.byte $23,$C2,$87,$FF

;-----------------------------------------------------------------------------

;  station 3 (The Transporters) girders: sections, x, y - three lists: sloping down, sloping up, flat

;-----------------------------------------------------------------------------

GIRDERS_3:      .byte $00               ; end of list; end of the down-sloping girders

.byte $00               ; end of the up-sloping girders

.byte $24,$08,$32

.byte $0A,$04,$4D

.byte $0C,$37,$61

.byte $0A,$74,$4F

.byte $0B,$0D,$81

.byte $06,$3F,$90

.byte $0D,$62,$7D

.byte $07,$04,$9D

.byte $06,$24,$A2

.byte $07,$57,$A2

.byte $06,$7D,$A1

.byte $00               ; end of the flat girders

;-----------------------------------------------------------------------------

;  station 3 (The Transporters) ladders: height, x, y, style

;-----------------------------------------------------------------------------

LADDERS_3:      .byte $16,$52,$37,$03

.byte $2F,$42,$52,$03

.byte $1C,$5F,$86,$03

.byte $1D,$36,$A2,$01

.byte $29,$A6,$54,$01

.byte $1F,$B4,$82,$03

.byte $18,$99,$A7,$01

.byte $00               ; end of list

;-----------------------------------------------------------------------------

;  station 4 (The Lillipads) girders: sections, x, y - three lists: sloping down, sloping up, flat

;-----------------------------------------------------------------------------

GIRDERS_4:      .byte $00               ; end of the down-sloping girders

.byte $00               ; end of the up-sloping girders

.byte $22,$0C,$1B

.byte $0A,$00,$3C

.byte $07,$00,$4E

.byte $02,$0C,$85

.byte $03,$10,$6A

.byte $05,$15,$9F

.byte $04,$2F,$3D

.byte $04,$35,$67

.byte $04,$28,$74

.byte $05,$34,$81

.byte $05,$30,$93

.byte $06,$32,$AD

.byte $02,$4E,$32

.byte $05,$3E,$5A

.byte $04,$4F,$86

.byte $03,$54,$B4

.byte $04,$64,$40

.byte $05,$55,$4D

.byte $07,$79,$3C

.byte $04,$6F,$52

.byte $04,$85,$5B

.byte $04,$79,$68

.byte $04,$63,$73

.byte $03,$79,$7C

.byte $03,$6B,$8B

.byte $04,$7E,$90

.byte $05,$83,$9D

.byte $04,$68,$A9

.byte $00               ; end of the flat girders

;-----------------------------------------------------------------------------

;  station 4 (The Lillipads) ladders: height, x, y, style

;-----------------------------------------------------------------------------

LADDERS_4:      .byte $1C,$46,$20,$04

.byte $1C,$B6,$20,$04

.byte $6C,$32,$53,$03

;-----------------------------------------------------------------------------

;  station 5 (Advanced Lillipads) girders: sections, x, y - three lists: sloping down, sloping up, flat

;-----------------------------------------------------------------------------

GIRDERS_5:      .byte $00               ; end of list; end of the down-sloping girders

.byte $00               ; end of the up-sloping girders

.byte $24,$08,$1D

.byte $02,$08,$29

.byte $02,$08,$35

.byte $02,$08,$41

.byte $02,$08,$4E

.byte $04,$06,$58

.byte $03,$16,$6D

.byte $03,$07,$78

.byte $02,$07,$85

.byte $03,$11,$91

.byte $02,$10,$A7

.byte $02,$21,$54

.byte $03,$25,$61

.byte $03,$31,$7A

.byte $03,$22,$87

.byte $04,$26,$9E

.byte $02,$26,$AA

.byte $04,$49,$59

.byte $02,$39,$69

.byte $03,$4D,$79

.byte $02,$4F,$87

.byte $02,$4F,$94

.byte $02,$3E,$A0

.byte $03,$4B,$AA

.byte $03,$39,$B5

.byte $02,$65,$99

.byte $02,$80,$71

.byte $02,$7B,$9F

.byte $02,$98,$29

.byte $02,$8F,$3F

.byte $02,$8E,$4B

.byte $03,$8E,$58

.byte $02,$90,$67

.byte $02,$8C,$7B

.byte $02,$94,$88

.byte $03,$88,$96

.byte $02,$91,$A4

.byte $02,$86,$B0

.byte $02,$4D,$32

.byte $02,$4D,$44

.byte $00               ; end of the flat girders

;-----------------------------------------------------------------------------

;  station 5 (Advanced Lillipads) slides: rows, x, y, step

;-----------------------------------------------------------------------------

SLIDES_5:       .byte $3C,$43,$08,$01

.byte $27,$58,$08,$01

.byte $15,$6A,$08,$01

.byte $15,$7D,$08,$00

.byte $15,$90,$08,$FF

.byte $27,$A2,$08,$FF

.byte $3C,$B7,$08,$FF

.byte $12,$7D,$1D,$00

.byte $15,$7D,$2F,$00

.byte $00               ; end of list

;-----------------------------------------------------------------------------

;  station 5 (Advanced Lillipads) moving platforms: timer, width, x, y, travel, speed

;-----------------------------------------------------------------------------

LIFTS_5:        .byte $02,$0E,$59,$65,$25,$01

.byte $03,$08,$98,$34,$13,$FF

.byte $01,$0C,$53,$B4,$22,$01

;-----------------------------------------------------------------------------

;  station 6 (Radioactive Waste) girders: sections, x, y - three lists: sloping down, sloping up, flat

;-----------------------------------------------------------------------------

GIRDERS_6:      .byte $00               ; end of list; end of the down-sloping girders

.byte $00               ; end of the up-sloping girders

.byte $06,$04,$1D

.byte $04,$0C,$49

.byte $02,$11,$98

.byte $02,$00,$A5

.byte $02,$00,$B3

.byte $02,$21,$89

.byte $03,$35,$7B

.byte $04,$49,$6C

.byte $02,$53,$5D

.byte $02,$55,$4F

.byte $02,$57,$40

.byte $03,$3C,$3B

.byte $02,$54,$26

.byte $05,$3F,$1D

.byte $04,$69,$1D

.byte $03,$68,$34

.byte $03,$6F,$4E

.byte $02,$79,$5E

.byte $03,$68,$70

.byte $02,$55,$8C

.byte $02,$6D,$8C

.byte $03,$87,$98

.byte $02,$98,$A5

.byte $02,$98,$B5

.byte $05,$84,$77

.byte $05,$8C,$2E

.byte $06,$88,$1D

.byte $00               ; end of the flat girders

;-----------------------------------------------------------------------------

;  station 6 (Radioactive Waste) ladders: height, x, y, style

;-----------------------------------------------------------------------------

LADDERS_6:      .byte $27,$3E,$22,$02

.byte $44,$BE,$33,$04

.byte $00               ; end of list

;-----------------------------------------------------------------------------

;  station 6 (Radioactive Waste) moving platforms: timer, width, x, y, travel, speed

;-----------------------------------------------------------------------------

LIFTS_6:        .byte $04,$06,$1A,$57,$6C,$01

.byte $00               ; end of list

;-----------------------------------------------------------------------------

;  station 7 (Advanced Transporters) girders: sections, x, y - three lists: sloping down, sloping up, flat

;-----------------------------------------------------------------------------

GIRDERS_7:      .byte $03,$4F,$93

.byte $00               ; end of the down-sloping girders

.byte $03,$4F,$A7

.byte $00               ; end of the up-sloping girders

.byte $07,$08,$32

.byte $07,$04,$61

.byte $07,$08,$90

.byte $05,$50,$32

.byte $02,$47,$41

.byte $04,$57,$51

.byte $05,$39,$5F

.byte $03,$39,$8E

.byte $02,$3E,$AC

.byte $03,$75,$A9

.byte $04,$90,$A6

.byte $04,$68,$8D

.byte $04,$6C,$7F

.byte $04,$8C,$84

.byte $02,$96,$75

.byte $02,$8D,$69

.byte $02,$81,$5D

.byte $02,$94,$59

.byte $02,$81,$4E

.byte $02,$74,$45

.byte $02,$94,$33

.byte $09,$7C,$1E

.byte $00               ; end of the flat girders

;-----------------------------------------------------------------------------

;  station 7 (Advanced Transporters) moving platforms: timer, width, x, y, travel, speed

;-----------------------------------------------------------------------------

LIFTS_7:        .byte $03,$04,$20,$28,$50,$01

.byte $04,$05,$1F,$68,$15,$01

.byte $03,$05,$21,$97,$13,$01

.byte $04,$05,$27,$B8,$4B,$01

.byte $00               ; end of list

;-----------------------------------------------------------------------------

;  station 7 (Advanced Transporters) slides: rows, x, y, step

;-----------------------------------------------------------------------------

SLIDES_7:       .byte $2D,$40,$1D,$01

.byte $2E,$6B,$4A,$01

.byte $33,$BF,$09,$FF

.byte $33,$8E,$3C,$01

.byte $15,$B1,$09,$01

.byte $26,$C4,$1E,$00

.byte $26,$C4,$44,$FF

.byte $27,$A0,$6A,$01

.byte $19,$C5,$91

;-----------------------------------------------------------------------------

;  station 8 (The Lift) girders: sections, x, y - three lists: sloping down, sloping up, flat

;-----------------------------------------------------------------------------

GIRDERS_8:      .byte $00               ; end of the down-sloping girders

.byte $00               ; end of list; end of the up-sloping girders

.byte $28,$00,$1E

.byte $05,$00,$3C

.byte $0F,$38,$3C

.byte $0D,$00,$5A

.byte $05,$8C,$5A

.byte $0D,$00,$78

.byte $03,$80,$6C

.byte $0A,$00,$A6

.byte $02,$7E,$5A

.byte $08,$80,$A0

.byte $00               ; end of the flat girders

;-----------------------------------------------------------------------------

;  station 8 (The Lift) ladders: height, x, y, style

;-----------------------------------------------------------------------------

LADDERS_8:      .byte $19,$3A,$23,$03

.byte $19,$6A,$23,$03

.byte $19,$7A,$41,$03

.byte $0D,$C6,$23,$03

.byte $22,$C6,$38,$81

.byte $41,$BE,$5F,$03

.byte $00               ; end of list

;-----------------------------------------------------------------------------

;  station 8 (The Lift) moving platforms: timer, width, x, y, travel, speed

;-----------------------------------------------------------------------------

LIFTS_8:        .byte $01,$33,$02,$AE,$69,$01

.byte $00               ; end of list

;-----------------------------------------------------------------------------

;  station 8 (The Lift) slides: rows, x, y, step

;-----------------------------------------------------------------------------

SLIDES_8:       .byte $1E,$8C,$09,$00

.byte $1E,$34,$27,$00

.byte $1E,$98,$27,$01

.byte $12,$B4,$45,$00

.byte $2E,$34,$63

;-----------------------------------------------------------------------------

;  station 9 (The Pulverizers) girders: sections, x, y - three lists: sloping down, sloping up, flat

;-----------------------------------------------------------------------------

GIRDERS_9:      .byte $00               ; end of the down-sloping girders

.byte $00               ; end of list; end of the up-sloping girders

.byte $07,$00,$1A

.byte $07,$00,$32

.byte $07,$00,$4A

.byte $07,$00,$62

.byte $07,$00,$7A

.byte $07,$00,$92

.byte $07,$84,$1A

.byte $06,$82,$32

.byte $05,$82,$4A

.byte $05,$81,$62

.byte $06,$84,$7A

.byte $07,$84,$92

.byte $00               ; end of the flat girders

;-----------------------------------------------------------------------------

;  station 9 (The Pulverizers) ladders: height, x, y, style

;-----------------------------------------------------------------------------

LADDERS_9:      .byte $13,$C2,$1F,$03

.byte $13,$BC,$37,$01

.byte $13,$B8,$4F,$03

.byte $13,$BB,$67,$01

.byte $13,$C2,$7F,$03

.byte $28,$C6,$97,$02

.byte $00               ; end of list

;-----------------------------------------------------------------------------

;  station 9 (The Pulverizers) moving platforms: timer, width, x, y, travel, speed

;-----------------------------------------------------------------------------

LIFTS_9:        .byte $01,$04,$81,$21,$66,$FF

.byte $01,$04,$7F,$39,$64,$FF

.byte $01,$04,$7F,$51,$64,$FF

.byte $01,$04,$7E,$69,$63,$FF

.byte $01,$04,$81,$81,$66,$FF

.byte $01,$04,$81,$99,$66,$FF

.byte $00               ; end of list

;-----------------------------------------------------------------------------

;  station 9 (The Pulverizers) slides: rows, x, y, step

;-----------------------------------------------------------------------------

SLIDES_9:       .byte $9E,$4E,$0C,$00

.byte $86,$4E,$24,$00

.byte $6E,$4E,$3C,$00

.byte $56,$4E,$54,$00

.byte $3E,$4E,$6C,$00

.byte $26,$4E,$84

;-----------------------------------------------------------------------------

;  station 10 (The Cannon) girders: sections, x, y - three lists: sloping down, sloping up, flat

;-----------------------------------------------------------------------------

GIRDERS_10:     .byte $00               ; end of the down-sloping girders

.byte $00               ; end of list; end of the up-sloping girders

.byte $15,$00,$22

.byte $15,$00,$4C

.byte $0B,$02,$7C

.byte $0B,$02,$92

.byte $0B,$02,$A8

.byte $02,$2C,$87

.byte $02,$2C,$9D

.byte $02,$2B,$B4

.byte $07,$38,$76

.byte $0D,$6C,$22

.byte $0D,$6C,$4C

.byte $0D,$6C,$76

.byte $07,$7A,$8E

.byte $00               ; end of the flat girders

;-----------------------------------------------------------------------------

;  station 10 (The Cannon) ladders: height, x, y, style

;-----------------------------------------------------------------------------

LADDERS_10:     .byte $2C,$BC,$93,$02

.byte $00               ; end of list

;-----------------------------------------------------------------------------

;  station 10 (The Cannon) slides: rows, x, y, step

;-----------------------------------------------------------------------------

SLIDES_10:      .byte $2A,$3E,$0D,$01

.byte $73,$66,$37,$00

.byte $49,$66,$61,$00

.byte $6C,$BC,$0D,$00

.byte $42,$BC,$37,$00

.byte $18,$BC,$61

;-----------------------------------------------------------------------------

;  station 11 (the credits) girders: sections, x, y - three lists: sloping down, sloping up, flat

;-----------------------------------------------------------------------------

GIRDERS_11:     .byte $00               ; end of the down-sloping girders

.byte $00               ; end of list; end of the up-sloping girders

.byte $28,$00,$24

.byte $05,$00,$45

.byte $1E,$24,$45

.byte $04,$00,$76

.byte $1F,$19,$70

.byte $08,$75,$94

.byte $1A,$00,$94

.byte $02,$00,$94

.byte $00               ; end of the flat girders

;-----------------------------------------------------------------------------

;  station 11 (the credits) ladders: height, x, y, style

;-----------------------------------------------------------------------------

LADDERS_11:     .byte $1C,$C2,$29,$02

.byte $2C,$32,$4A,$04

.byte $1F,$BB,$75,$03

.byte $26,$32,$99,$02

.byte $00               ; end of list

;-----------------------------------------------------------------------------

;  SET_VBI - install a vertical-blank interrupt handler.            In: Y/X = handler lo/hi

;

;  The same cartridge runs on the Atari 400/800 and on the Atari 5200, and the two

;  machines dispatch the VBI differently:

;    * 400/800 OS: its NMI routine pushes A, X and Y, then jumps through VVBLKI ($0222).

;    * 5200 BIOS:  jumps through its own VVBLKI ($0202) with nothing saved; Miner points that

;                  at VBI_5200 below, which saves the registers and jumps through VVBLKD

;                  ($0204), the 5200's deferred vector.

;  Writing the handler into both $0222 and $0204 means every handler can end with the same

;  PLA/TAY/PLA/TAX/PLA/RTI sequence (NMI_EXIT) on either machine.

;-----------------------------------------------------------------------------

SET_VBI:        lda #$00                ; disable all NMIs while the vectors are half-written

sta NMIEN               ; NMIEN = 0

sty VVBLKD5             ; 5200: deferred VBI vector = handler

stx VVBLKD5+1

sty VVBLKI              ; 400/800: VBI vector = handler

stx VVBLKI+1

lda #$10                ; 5200: immediate VBI vector -> VBI_5200 ($9210)

sta VVBLKI5

lda #$92                ; VVBLKI5+1 = $92

sta VVBLKI5+1

sta NMIRES              ; clear any pending NMI status (the value written is irrelevant)

rts

;-----------------------------------------------------------------------------

;  VBI_5200 - 5200 only: make the BIOS's bare VBI look like the 400/800 OS's.

;  Push A, X, Y exactly as the 400/800 OS does, then continue at the game's handler.

;-----------------------------------------------------------------------------

VBI_5200:       pha                     ; save A

txa                     ; save X

pha

tya                     ; save Y

pha

jmp (VVBLKD5)           ; continue at the handler stored by SET_VBI

;-----------------------------------------------------------------------------

;  POLL_KEYS - called at the start of every VBI handler. Reads a key if one was pressed.

;

;  The game never enables POKEY interrupts. Instead it polls the keyboard bit (6) of IRQST

;  each frame: 0 there means a key has been latched in KBCODE. The key is stored in KEYCODE

;  and the IRQ status bit is re-armed by writing IRQEN 0 then $40.

;    400/800: KEYCODE = raw KBCODE (keyboard matrix code, bit 6 = SHIFT, bit 7 = CTRL).

;    5200:    the keypad code (bits 1-4) is translated through KEYPAD_MAP into the 400/800

;             code of the same key, so the rest of the game sees a single keyboard. The

;             keypad's RESET key maps to 0 and restarts the cartridge.

;-----------------------------------------------------------------------------

POLL_KEYS:      ldy #$00                ; Y = 0 for the (pointer),Y register accesses

lda (pIRQEN),y          ; read IRQST

and #$40                ; bit 6 = keyboard interrupt status (0 = key pressed)

bne POLL_DONE           ; no key: return

lda (pSTIMER),y         ; read the key code

ldx IS5200              ; which machine?

beq STORE_KEY           ; 400/800: keep the code as it is

lsr a                   ; 5200 keypad code is in bits 1-4

and #$0F

tax

lda KEYPAD_MAP,x        ; translate to a 400/800 key code

beq RESET_5200          ; 0 = RESET key: restart the cartridge

STORE_KEY:      sta KEYCODE             ; remember the key

tya                     ; IRQEN = 0 then $40: clears the keyboard status bit, ready for the next key

sta (pIRQEN),y          ; IRQEN = A

lda #$40                ; IRQEN = $40

sta (pIRQEN),y          ; IRQEN = A

POLL_DONE:      rts

RESET_5200:     jmp (CART_TRAILER)      ; restart through the cartridge's start vector (= RESTART: the high scores

;   are kept)

;-----------------------------------------------------------------------------

;  NMI_EXIT - common tail of every interrupt handler: restore Y, X, A and return.

;-----------------------------------------------------------------------------

NMI_EXIT:       pla

tay                     ; restore Y

pla

tax                     ; restore X

pla                     ; restore A

rti

;-----------------------------------------------------------------------------

;  KEYPAD_MAP - 5200 keypad code (0-15) -> the 400/800 KBCODE the game expects.

;    code:  0    1    2    3    4     5    6    7    8     9    A    B    C     D    E    F

;    key:   -    #    0    *    RESET 9    8    7    PAUSE 6    5    4    START 3    2    1

;    ->     $FF  $21  $32  $02  $00   $30  $35  $33  $1C   $1B  $1D  $18  $03   $1A  $1E  $1F

;  Digits map to the computer's digit keys, # to the space bar (runs the lift in station 8),

;  PAUSE to ESC (pause), * to $02 and START to $03 (the 5200 stand-ins for SELECT/OPTION and

;  START, see TITLE_LOOP). RESET maps to 0, which POLL_KEYS treats as "restart".

;-----------------------------------------------------------------------------

KEYPAD_MAP:     .byte $FF,$21,$32,$02,$00,$30,$35,$33,$1C,$1B,$1D,$18,$03,$1A,$1E,$1F

;-----------------------------------------------------------------------------

;  SET_DLI - install a display-list-interrupt handler.             In: Y/X = handler lo/hi

;  Written to both the 400/800 vector (VDSLST) and the 5200 one ($0206).

;-----------------------------------------------------------------------------

SET_DLI:        sty VDSLST5             ; 5200 DLI vector

stx VDSLST5+1

sty VDSLST              ; 400/800 DLI vector

stx VDSLST+1

rts

;-----------------------------------------------------------------------------

;  READ_DIGIT - was a digit key (1-9, 0) pressed?

;  Out: Z clear and A = Y = 1-10 if so ('0' counts as 10), and the key is consumed;

;       Z set and Y = 0 if no key, or a key that is not a digit.

;  KEY_TO_SCREEN converts the key code to a screen code; digits are $10-$19.

;-----------------------------------------------------------------------------

READ_DIGIT:     lda KEYCODE             ; any key waiting?

cmp #$FF                ; if A == $FF goto NO_DIGIT

beq NO_DIGIT            ; no

and #$3F                ; drop SHIFT and CTRL

tay

lda KEY_TO_SCREEN,y     ; key code -> screen code

sec                     ; screen code - $10 = digit value

sbc #$10

bne READ_DIGIT_CHK      ; '0'

lda #$0A                ; counts as 10

READ_DIGIT_CHK: cmp #$0B                ; not a digit at all?

bcs NO_DIGIT

ldy #$FF                ; consume the key

sty KEYCODE

tay                     ; Y = digit, Z clear

rts

NO_DIGIT:       ldy #$00                ; Y = 0, Z set

RD_RTS:         rts

;-----------------------------------------------------------------------------

;  CHECK_ZONE_KEY - on the title screens a digit key starts a one-player game at that zone.

;  If a digit was pressed it is saved in START_ZONE and the game starts (via START_GAME_A

;  with A = 0, i.e. not a demo). Otherwise it simply returns.

;-----------------------------------------------------------------------------

CHECK_ZONE_KEY: jsr READ_DIGIT          ; was a digit key (1-9, 0) pressed?

beq RD_RTS              ; no digit: return

sty START_ZONE          ; zone to start in

lda #$00                ; A = 0: DEMO_PTR+1 = 0 = a real game

jmp START_GAME_A        ; start the game

;-----------------------------------------------------------------------------

;  HW_RESET - silence and blank everything.

;  Writes 0 to every GTIA, POKEY and ANTIC register (256 times over, through the mirrors),

;  turns the POKEY keyboard status back on, forgets any key, selects the ROM font and sets

;  up the serial/keyboard control.

;-----------------------------------------------------------------------------

HW_RESET:       lda #$00                ; no NMIs

sta NMIEN

tay                     ; Y = 0

HW_RESET_LOOP:  sta (pHPOSP0),y         ; GTIA register Y (mirrored every 32 bytes)

sta (pAUDF1),y          ; POKEY register Y (mirrored every 16 bytes)

sta DMACTL,y            ; ANTIC register Y: DMA off, display list, scroll, NMIEN ...

iny

bne HW_RESET_LOOP       ; 256 bytes

lda #$40                ; IRQEN = $40: allow the keyboard status bit

sta (pIRQEN),y          ; IRQEN = A

lda #$FF                ; KEYCODE = none

sta KEYCODE

lda FONTPAGE            ; character set = the machine's ROM font

sta CHBASE

lda IS5200              ; if IS5200 == 0 goto HW_RESET_800

beq HW_RESET_800        ; 400/800:

lda #$04                ; 5200: CONSOL bit 2 enables the controller ports' pot scanning

sta (pCONSOL),y         ; CONSOL = A

lda #$02                ; 5200: SKCTL = 2 (keyboard debounce off)

bne HW_RESET_SKCTL

HW_RESET_800:   lda #$03                ; 400/800: SKCTL = 3 (keyboard scan + debounce enabled)

HW_RESET_SKCTL: sta (pSKCTL),y          ; SKCTL = A

rts

;-----------------------------------------------------------------------------

;  TITLE_SCREEN - the high-score screen with its ring of "chasing lights".

;

;  Reached after a game ends, after the game-over tune, or by pressing OPTION. It shows the

;  ten best scores inside a frame of 36 '5' characters (taken from the ROM font) whose

;  colours march round the frame. The tricks that make it work:

;

;   * Top and bottom rows: one ANTIC mode 7 line each (20 big characters), held in ROM at

;     $8059 and $801F. The text never moves. HSCROL slides the row by 0-15 colour clocks

;     (two character cells), and each time the scroll wraps the colours rotate one step, so

;     the eye sees a continuous march.

;   * Ten colours on one line: mode 7 gives only four colours (COLPF0-3). A display list

;     interrupt jumps into TITLE_KERNEL, a cycle-counted loop copied to RAM at $0400 that

;     rewrites all four colour registers twice on every scan line: once for the left four

;     '5's and once, half-way across, for the right four. The middle two '5's are players

;     1 and 2, which have their own colour registers. 4 + 2 + 4 = 10 colours.

;   * The kernel must follow the fine scroll, so the two delay instructions (KERNEL_DELAY1

;     before the loop, KERNEL_DELAY2 in it) are patched to a 3-, 4- or 5-cycle opcode (CPX zp / CMP zp,X / ASL zp)

;     to move the colour changes

;     by 6-10 colour clocks. Its STA instructions also get their high bytes (by

;     KERNEL_COLS_TOP only, since both halves share the kernel) patched to the

;     GTIA's page, because GTIA is at $D0xx on the 400/800 but at $C0xx on the 5200.

;   * Left and right columns: players 0 and 3 in double-line resolution. A DLI on every text

;     line recolours them. They move by redrawing into a second P/M buffer and flipping

;     PMBASE between $0400 and $1C00 (double buffering), so no half-drawn frame is seen.

;   * The ten high-score lines get a moving rainbow (one DLI per line), "HIGH SCORES:"

;     pulses in brightness, and "START" blinks in inverse video.

;

;  While it runs it watches for START or the joystick button (start a game), SELECT (one or

;  two players), a digit key (start in that zone), and after 30 seconds starts the demo.

;-----------------------------------------------------------------------------

TITLE_SCREEN:   jsr HW_RESET            ; everything off and silent

lda TWO_PLAYER          ; number of players shown in "Press START For a 1 Miner Game":

clc                     ; '1' + TWO_PLAYER

adc #$11

sta TEXT_RAM+23         ; the digit sits at TEXT_RAM+23

ldx #$1B                ; copy the 27 title variables T_SCROLL..T_VOLUME from TITLE_INIT

TS_INIT_LOOP:   lda TITLE_INIT-1,x      ; T_SCROLL-1[X] = TITLE_INIT-1[X]

sta T_SCROLL-1,x

dex                     ; loop until X = 0

bne TS_INIT_LOOP

jsr CLEAR_PM_BUFS       ; clear both P/M buffers

jsr DRAW_MID_FIVES      ; put the middle top/bottom '5's into players 1 and 2

jsr DRAW_COLUMNS        ; draw the side columns into the first buffer

ldx #$1C                ; next buffer to draw into is $1C00

stx T_PMPAGE

ldx #$3E                ; copy the 62-byte colour kernel to RAM at $0400 (it is self-modified)

TS_COPY_KERNEL: lda DTR_GO+7,x          ; $03FF[X] = DTR_GO+7[X]

sta $03FF,x

dex                     ; loop until X = 0

bne TS_COPY_KERNEL

ldx #>DLI_TOP_ROW       ; first DLI = DLI_TOP_ROW

ldy #<DLI_TOP_ROW

jsr SET_DLI             ; install a display-list-interrupt handler

ldx #$04                ; show P/M buffer $0400 (double-line players: base+$200..$3FF)

stx PMBASE

ldy #$00

lda #$30                ; left column (player 0) at x = $30

sta (pHPOSP0),y         ; HPOSP0 = A

lda #$C8                ; right column (player 3) at x = $C8

sta (pHPOSP3),y         ; HPOSP3 = A

ldy #<TITLE_VBI         ; VBI = TITLE_VBI

ldx #>TITLE_VBI

jsr SET_VBI             ; install a vertical-blank interrupt handler

lda #$C0                ; enable DLI and VBI

sta NMIEN

TITLE_LOOP:     lda T_SCROLL            ; wait until the VBI has shown the scroll phase asked for last time

cmp T_NEWSCROLL         ; if A <> T_NEWSCROLL goto TITLE_LOOP

bne TITLE_LOOP

ldx #$04                ; 4 frames per scroll step

TS_FRAME:       lda T_TIMER30           ; T_TIMER30 changes every frame: remember it to spot the next frame

TS_POLL:        pha

jsr CHECK_ZONE_KEY      ; a digit key starts a game in that zone

lda T_VOLUME+1          ; START: 5200 keypad START arrives as key code 3

beq TS_START_800

lda KEYCODE

cmp #$03

jmp TS_START_CHK

TS_START_800:   lda CONSOL              ; 400/800: START key (CONSOL bit 0, 0 = pressed)

and #$01                ; test bits $01

beq TS_START_PRESSED    ; pressed

ldy #$00                ; ... or the joystick button

lda (pTRIG0),y          ; A = TRIG0

and #$01

TS_START_CHK:   bne TS_NO_START         ; not pressed

lda #$FF                ; forget any key

sta KEYCODE

TS_START_PRESSED:

lda T_STARTDLY          ; START must have been released for a few frames first

beq START_GAME_A        ; it was: start a game (A = 0 = not a demo)

lda #$05                ; still held from before: keep waiting

sta T_STARTDLY

bne TS_CHECK_DEMO

TS_DEMO:        lda #$01                ; A = 1: run the demo instead

;-----------------------------------------------------------------------------

;  START_GAME_A - start a game. A = 0 for a real game, A <> 0 for the demo (it becomes

;  DEMO_PTR+1 and is replaced by the real pointer in START_GAME).

;-----------------------------------------------------------------------------

START_GAME_A:   sta DEMO_PTR+1

jmp START_GAME          ; start a game (DEMO_PTR+1 = 0) or the demo (DEMO_PTR+1 <> 0)

TS_NO_START:    lda T_STARTDLY          ; count the START debounce down while START is up

beq TS_CHECK_DEMO

dec T_STARTDLY

TS_CHECK_DEMO:  lda T_DEMOSEC           ; 30 seconds gone? then show the demo

beq TS_DEMO

lda T_VOLUME+1          ; SELECT: on the 5200 the * key (code 2)

beq TS_SELECT_800

lda KEYCODE

cmp #$02

jmp TS_SELECT_CHK

TS_SELECT_800:  lda CONSOL              ; 400/800: SELECT key

and #$02

TS_SELECT_CHK:  bne TS_SELECT_UP        ; not pressed

lda #$FF                ; forget any key

sta KEYCODE

lda T_SELDLY            ; only act on a new press

bne TS_SELECT_HELD

lda #$01                ; TWO_PLAYER = 1 - TWO_PLAYER

sec

sbc TWO_PLAYER

sta TWO_PLAYER

clc                     ; and update the digit on screen

adc #$11

sta TEXT_RAM+23

TS_SELECT_HELD: lda #$FF                ; hold-off while SELECT stays down

sta T_SELDLY

TS_SELECT_UP:   lda T_SELDLY            ; if T_SELDLY == 0 goto TS_NEXT_FRAME

beq TS_NEXT_FRAME

dec T_SELDLY

TS_NEXT_FRAME:  pla                     ; has a frame gone by?

cmp T_TIMER30           ; if A == T_TIMER30 goto TS_POLL_J

beq TS_POLL_J           ; no: keep polling

dex                     ; 4 frames?

bne TS_FRAME_J

lda T_SCROLL            ; next scroll phase = (T_SCROLL + 2) and 15

clc

adc #$02

and #$0F

sta T_NEXT

jsr DRAW_COLUMNS        ; draw the side columns for it in the hidden buffer

lda #$20                ; flip buffers: $1C <-> $04

sec

sbc T_PMPAGE

sta T_PMPAGE

lda T_NEXT              ; ask the VBI to show it

sta T_NEWSCROLL

jmp TITLE_LOOP

TS_POLL_J:      jmp TS_POLL

TS_FRAME_J:     jmp TS_FRAME

;-----------------------------------------------------------------------------

;  SET_FINE_SCROLL - set HSCROL and patch the kernel's delay to match.     In: A = 0-15

;  A/2 = 0     -> $E4 CPX zp      (3 cycles)

;  A/2 = 1-4   -> $D5 CMP zp,X    (4 cycles)

;  A/2 = 5-7   -> $06 ASL zp      (5 cycles; shifts T_TMP, which is harmless)

;  The opcode goes into both delay slots of TITLE_KERNEL.

;-----------------------------------------------------------------------------

SET_FINE_SCROLL:

sta HSCROL              ; fine scroll

ldx #$E4

lsr a

beq SFS_PATCH

ldx #$D5

cmp #$05                ; if A < 5 goto SFS_PATCH

bcc SFS_PATCH

ldx #$06

SFS_PATCH:      stx KERNEL_DELAY2       ; patch the delay opcode inside the loop

stx KERNEL_DELAY1       ; ... and the one before the first line

rts

;-----------------------------------------------------------------------------

;  CLEAR_PM_BUFS - zero 1K at $0400 and 1K at $1C00 (the two P/M buffers).

;-----------------------------------------------------------------------------

CLEAR_PM_BUFS:  ldx #$04                ; $0400

stx T_PTR+1

ldx #$00

stx T_PTR

ldx #$FC                ; count $FC00 up to $10000 = 1024 bytes

ldy #$00

jsr CLEAR_MEM           ; zero memory from (T_PTR) for -(T_CNT) bytes

ldx #$1C                ; $1C00

stx T_PTR+1

ldx #$00

stx T_PTR

ldx #$FC

ldy #$00

;-----------------------------------------------------------------------------

;  CLEAR_MEM - zero memory from (T_PTR) for -(T_CNT) bytes.     In: X/Y = negative count hi/lo

;-----------------------------------------------------------------------------

CLEAR_MEM:      stx T_CNT2

sty T_CNT

lda #$00

tay

CLEAR_MEM_LOOP: sta (T_PTR),y

inc T_PTR               ; advance the pointer

bne CLEAR_MEM_CNT

inc T_PTR+1

CLEAR_MEM_CNT:  inc T_CNT               ; count up to zero

bne CLEAR_MEM_LOOP

inc T_CNT2              ; T_CNT2 += 1, until 0

bne CLEAR_MEM_LOOP

rts

;-----------------------------------------------------------------------------

;  DRAW_MID_FIVES - copy the '5' glyph from the ROM font into players 1 and 2 of both P/M

;  buffers, at the heights of the top and bottom rows. These are the two middle lights of

;  each row (they need colours of their own; see TITLE_SCREEN).

;-----------------------------------------------------------------------------

DRAW_MID_FIVES: lda FONTPAGE            ; T_PTR = font + 8 * $15 = the '5' glyph

sta T_PTR+1

lda #$A8                ; T_PTR = $A8

sta T_PTR

ldx #$16                ; player 1, top row

jsr COPY_GLYPH          ; copy 6 rows of the '5' glyph into both P/M buffers at row X

ldx #$6E                ; player 1, bottom row

jsr COPY_GLYPH          ; copy 6 rows of the '5' glyph into both P/M buffers at row X

ldx #$96                ; player 2, top row

jsr COPY_GLYPH          ; copy 6 rows of the '5' glyph into both P/M buffers at row X

ldx #$EE                ; player 2, bottom row

COPY_GLYPH:     ldy #$06                ; 6 rows (glyph rows 1-6)

COPY_GLYPH_LOOP:

lda (T_PTR),y           ; $0680[X] = (T_PTR)[Y]

sta $0680,x             ; buffer at $0400 (players at $0600-$07FF)

sta $1E80,x             ; buffer at $1C00

dex

dey                     ; loop until Y = 0

bne COPY_GLYPH_LOOP

rts

;-----------------------------------------------------------------------------

;  DRAW_COLUMNS - draw the left and right columns of '5's into the hidden P/M buffer for

;  scroll phase T_NEXT. The right column (player 3) is drawn from the bottom up shifted one

;  way, the left column (player 0) shifted the other way, so the lights run round the frame.

;  Each column is: a blank gap, then glyphs separated by 8-line gaps.

;-----------------------------------------------------------------------------

DRAW_COLUMNS:   ldx #$80                ; T_PMPTR = hidden buffer + $380 = player 3 (right column)

stx T_PMPTR

ldx T_PMPAGE

inx

inx

inx

stx T_PMPTR+1

ldy #$7F                ; start at the bottom line of the player

lda #$0F                ; blank gap of 15 - phase lines

sec

sbc T_NEXT

tax

jsr DC_GAP              ; X blank lines in the column being drawn

lda T_NEXT              ; phase 0: glyph first

beq DC_GLYPH1

jsr DC_GAP8             ; 8 blank lines in the column being drawn

beq DC_REST1

DC_GLYPH1:      jsr DC_GLYPH            ; the 8 lines of the '5' glyph in the column being drawn

DC_REST1:       jsr DC_GAP8             ; 8 blank lines in the column being drawn

jsr DC_FIVE_PAIRS       ; five more glyph+gap pairs

lda T_NEXT              ; phase >= 8: one more glyph else a gap

cmp #$08                ; if A < 8 goto DC_GAP1

bcc DC_GAP1

jsr DC_GLYPH            ; the 8 lines of the '5' glyph in the column being drawn

beq DC_LEFT

DC_GAP1:        jsr DC_GAP8             ; 8 blank lines in the column being drawn

DC_LEFT:        jsr DC_GAP8             ; 8 blank lines in the column being drawn

dec T_PMPTR+1           ; T_PMPTR = hidden buffer + $200 = player 0 (left column)

ldx #$00

stx T_PMPTR

ldy #$79

ldx T_NEXT              ; blank gap of phase+1 lines

inx

jsr DC_GAP              ; X blank lines in the column being drawn

lda T_NEXT              ; phase >= 10: glyph else gap

cmp #$0A                ; if A < $0A goto DC_GAP2

bcc DC_GAP2

jsr DC_GLYPH            ; the 8 lines of the '5' glyph in the column being drawn

beq DC_REST2

DC_GAP2:        jsr DC_GAP8             ; 8 blank lines in the column being drawn

DC_REST2:       jsr DC_GAP8             ; 8 blank lines in the column being drawn

jsr DC_FIVE_PAIRS       ; five glyph+gap pairs

ldx #$10                ; and a final 16-line gap

bne DC_GAP

DC_FIVE_PAIRS:  ldx #$05                ; 5 times: glyph then gap

stx T_TMP

DC_PAIR:        jsr DC_GLYPH            ; the 8 lines of the '5' glyph in the column being drawn

jsr DC_GAP8             ; 8 blank lines in the column being drawn

dec T_TMP               ; T_TMP -= 1, until 0

bne DC_PAIR

rts

DC_GAP8:        ldx #$08                ; 8 blank lines

DC_GAP:         lda #$00                ; X blank lines, going up

DC_GAP_LOOP:    sta (T_PMPTR),y

dey

dex                     ; loop until X = 0

bne DC_GAP_LOOP

rts

DC_GLYPH:       ldx #$08                ; 8 lines of the '5' glyph from the machine's ROM font

lda T_VOLUME+1          ; if T_VOLUME+1 <> 0 goto DC_GLYPH_5200

bne DC_GLYPH_5200

DC_GLYPH_800:   lda $E0A8,x             ; 400/800 font at $E000

sta (T_PMPTR),y

dey

dex                     ; loop until X = 0

bne DC_GLYPH_800

rts

DC_GLYPH_5200:  lda $F8A8,x             ; 5200 font at $F800

sta (T_PMPTR),y

dey

dex                     ; loop until X = 0

bne DC_GLYPH_5200

rts

;-----------------------------------------------------------------------------

;  TITLE_VBI - vertical blank on the high-score screen.

;  Shows the scroll phase / P/M buffer the main loop prepared, rotates the colours when the

;  scroll wraps, pulses "HIGH SCORES:", moves the rainbow, blinks "START", counts down to

;  the demo and plays a soft repeating tone.

;-----------------------------------------------------------------------------

TITLE_VBI:      jsr POLL_KEYS           ; read the keyboard

lda #$80                ; display list at $8000 (in ROM)

sta DLISTH

lda #$00                ; DLISTL = 0

sta DLISTL

lda #$2A                ; DMACTL: normal playfield, players on, double-line, display list on

sta DMACTL

lda #$02                ; GRACTL: player DMA on

sta (pGRACTL),y         ; GRACTL = A

lda T_NEWSCROLL         ; new scroll phase requested?

cmp T_SCROLL            ; if A == T_SCROLL goto TV_SCROLL

beq TV_SCROLL

sta T_SCROLL            ; yes: take it

lda #$20                ; show the buffer just drawn (the other one of $1C/$04)

sec

sbc T_PMPAGE

sta PMBASE

ldx T_SCROLL            ; if T_SCROLL <> 0 goto TV_SCROLL

bne TV_SCROLL

dec T_ROT               ; rotate the colours one light along

bpl TV_RECOLOUR

ldx #$09

stx T_ROT

TV_RECOLOUR:    jsr KERNEL_COLS_TOP     ; load the new colours into the kernel for the top row

TV_SCROLL:      lda T_SCROLL            ; fine scroll (and kernel delay) for the top row

jsr SET_FINE_SCROLL     ; set HSCROL and patch the kernel's delay to match

dec T_TIMER4            ; every 4 frames step the brightness of "HIGH SCORES:"

bne TV_RAINBOW

lda #$04                ; T_TIMER4 = 4

sta T_TIMER4

lda T_TEXTCOL           ; brightness 2 or 12 ends a sweep

and #$0F

cmp #$02                ; if A == 2 goto TV_NEXT_HUE

beq TV_NEXT_HUE

cmp #$0C                ; if A <> $0C goto TV_STEP

bne TV_STEP

beq TV_TURN

TV_NEXT_HUE:    lda T_TEXTCOL           ; at the dark end move to the next hue

clc

adc #$10

sta T_TEXTCOL

TV_TURN:        lda #$00                ; reverse the brightness step

sec

sbc T_TEXTSTEP

sta T_TEXTSTEP

TV_STEP:        lda T_TEXTCOL           ; T_TEXTCOL = T_TEXTCOL + T_TEXTSTEP

clc

adc T_TEXTSTEP

sta T_TEXTCOL

TV_RAINBOW:     lda T_HUE               ; every 14 frames move the rainbow start (15 colours)

dec T_TIMER14           ; T_TIMER14 -= 1, until 0

bne TV_RAIN_SET

ldx #$0E

stx T_TIMER14

sec

sbc #$01

bcs TV_RAIN_STORE

lda #$0E

TV_RAIN_STORE:  sta T_HUE

TV_RAIN_SET:    sta T_COLIX             ; the DLIs count T_COLIX down from here, line by line

dec T_TIMER30           ; every 30 frames toggle inverse video ("START" blinks)

bne TV_SECONDS

ldx #$1E

stx T_TIMER30

lda #$02                ; T_INVERSE = 2 - T_INVERSE

sec

sbc T_INVERSE

sta T_INVERSE

sta CHACTL

TV_SECONDS:     dec T_TIMER60           ; every 60 frames count a second off the demo timer

bne TV_TONE

lda #$3C                ; T_TIMER60 = $3C

sta T_TIMER60

dec T_DEMOSEC

TV_TONE:        dec T_TIMER11           ; every 11 frames lower the tone's volume one step

bne TV_EXIT

lda #$0B                ; T_TIMER11 = $0B

sta T_TIMER11

ldx T_VOLUME

dex

cpx #$5F                ; volume wraps from 0 back to 15 (distortion 6)

bne TV_TONE_SET

ldx #$6F

TV_TONE_SET:    txa

ldy #$00

sta T_VOLUME

sta (pAUDC1),y          ; AUDC1 = A

lda #$C8                ; AUDF1 = $C8

sta (pAUDF1),y          ; AUDF1 = A

TV_EXIT:        jmp NMI_EXIT            ; common tail of every interrupt handler

;-----------------------------------------------------------------------------

;  SET_MIDDLE_PLAYERS - put players 1 and 2 (the two middle lights) at A and A+16, and

;  point T_PTR at the kernel's first colour operand. Returns T_CNT = 8 colours to load.

;-----------------------------------------------------------------------------

SET_MIDDLE_PLAYERS:

ldy #$00

sta (pHPOSP1),y         ; HPOSP1 = A

clc

adc #$10

sta (pHPOSP2),y         ; HPOSP2 = A

ldx #$04                ; T_PTR = $0406 = first "LDA #" operand in the RAM kernel

stx T_PTR+1

ldx #$06

stx T_PTR

ldx #$08                ; 8 colour writes per line

stx T_CNT

clc

rts

;-----------------------------------------------------------------------------

;  KERNEL_COLS_TOP - load the top row's colours into the RAM kernel.

;  The eight "LDA #colour" operands get eight consecutive colours from the ten-colour ring

;  CHASE_COLOURS (starting at T_ROT); the two middle players get the other two. The high

;  byte of each "STA COLPFn" is rewritten with the GTIA page taken from the pointer table

;  (pHPOSP0+1 = $D0 or $C0), so the same kernel works on the 400/800 and the 5200.

;-----------------------------------------------------------------------------

KERNEL_COLS_TOP:

lda #$70                ; middle players follow the scroll

clc

adc T_SCROLL

jsr SET_MIDDLE_PLAYERS  ; put players 1 and 2 (the two middle lights) at A and A+16, and

lda #$09                ; colour index = 9 + rotation

adc T_ROT

tax

ldy #$23                ; operands at +$23, +$1E ... +$00 (5 bytes per LDA/STA pair)

lda T_SCROLL            ; phase 8-15 shifts a light into the gap: blank it

cmp #$08                ; if A < 8 goto KCT_LOOP

bcc KCT_LOOP

lda #$00

beq KCT_STORE

KCT_LOOP:       lda CHASE_COLOURS,x     ; next colour of the ring

KCT_STORE:      sta (T_PTR),y           ; patch "LDA #colour"

lda pHPOSP0+1           ; GTIA page ($D0 or $C0)

iny

iny

iny

sta (T_PTR),y           ; patch the high byte of "STA COLPFn"

dey

dey

dey

tya                     ; previous LDA/STA pair

sec

sbc #$05

tay

dex

cpy #$0F                ; at the half-way point skip the two players' colours

bne KCT_NEXT

dex

dex

KCT_NEXT:       dec T_CNT               ; T_CNT -= 1, until 0

bne KCT_LOOP

ldx T_ROT               ; the two middle players

lda CHASE_COLOURS+4,x

ldy #$00

sta (pCOLPM1),y         ; COLPM1 = A

lda CHASE_COLOURS+5,x   ; COLPM2 = CHASE_COLOURS+5[X]

sta (pCOLPM2),y         ; COLPM2 = A

lda CHASE_COLOURS+9,x   ; and the colour shared by the two side columns

sta (pCOLPM0),y         ; COLPM0 = A

sta (pCOLPM3),y         ; COLPM3 = A

rts

;-----------------------------------------------------------------------------

;  KERNEL_COLS_BOTTOM - as KERNEL_COLS_TOP but for the bottom row, which scrolls the other

;  way: players at $68 + (16 - phase), colours taken in the opposite order.

;-----------------------------------------------------------------------------

KERNEL_COLS_BOTTOM:

lda #$10

sec

sbc T_SCROLL

and #$0F

clc

adc #$68

jsr SET_MIDDLE_PLAYERS  ; put players 1 and 2 (the two middle lights) at A and A+16, and

lda #$0E

adc T_ROT

tax

lda T_SCROLL            ; if T_SCROLL <> 0 goto KCB_START

bne KCB_START

inx

KCB_START:      ldy #$00

cmp #$0A                ; if A < $0A goto KCB_LOOP

bcc KCB_LOOP

lda #$00

beq KCB_STORE

KCB_LOOP:       lda CHASE_COLOURS,x

KCB_STORE:      sta (T_PTR),y           ; patch "LDA #colour"

tya                     ; next pair, going forwards this time

clc

adc #$05

tay

dex

cpy #$14                ; if Y <> $14 goto KCB_NEXT

bne KCB_NEXT

dex

dex

KCB_NEXT:       dec T_CNT               ; T_CNT -= 1, until 0

bne KCB_LOOP

txa                     ; the two middle players

clc

adc #$06

tax

lda CHASE_COLOURS,x

ldy #$00

sta (pCOLPM1),y         ; COLPM1 = A

lda CHASE_COLOURS-1,x   ; COLPM2 = CHASE_COLOURS-1[X]

sta (pCOLPM2),y         ; COLPM2 = A

rts

;-----------------------------------------------------------------------------

;  DLI_TOP_ROW - display list interrupt just above the top (and the bottom) row of lights.

;  Runs the colour kernel for the 12 scan lines of the mode 7 row.

;-----------------------------------------------------------------------------

DLI_TOP_ROW:    pha                     ; save A, X, Y

txa

pha

tya

pha

ldy #$00

sta WSYNC

lda T_LINE              ; bottom row at phase 0: give player 3 its colour

beq DTR_GO

lda T_SCROLL            ; if T_SCROLL <> 0 goto DTR_GO

bne DTR_GO

ldx T_ROT

lda CHASE_COLOURS+5,x   ; COLPM3 = CHASE_COLOURS+5[X]

sta (pCOLPM3),y         ; COLPM3 = A

DTR_GO:         sta WSYNC               ; start on a fresh line

ldx #$0C                ; 12 scan lines

jmp TITLE_KERNEL        ; into the RAM kernel

.org $0400                      ; --- block runs in RAM at $0400 (copied there from ROM $9630) ---

;-----------------------------------------------------------------------------

;  TITLE_KERNEL - copied to RAM at $0400 and run there (the code below is assembled for

;  $0400). Cycle-counted: after the first WSYNC each pass, plus the cycles ANTIC

;  steals for DMA, is meant to fill one scan line; the delays tune it to the fine scroll.

;  The "LDA #" operands are patched by KERNEL_COLS_TOP/BOTTOM (the STA high bytes by

;  KERNEL_COLS_TOP alone), and

;  the two delay opcodes by SET_FINE_SCROLL.

;-----------------------------------------------------------------------------

TITLE_KERNEL:   .byte $8D,$0A,$D4       ; wait for the start of the next line

KERNEL_DELAY1:  .byte $E4,$B1           ; delay: CPX/CMP,X/ASL = 3/4/5 cycles (patched)

KERNEL_LINE:    .byte $A9,$00,$8D,$16,$D0,$A9,$00,$8D,$17,$D0,$A9,$00,$8D,$18,$D0,$A9; colours for the left group of

;   lights

.byte $00,$8D,$19,$D0,$A9,$00,$8D,$16,$D0,$A9,$00,$8D,$17,$D0,$A9,$00

.byte $8D,$18,$D0,$A9,$00,$8D,$19,$D0,$01,$B1,$01,$B1,$01,$B1

KERNEL_DELAY2:  .byte $E4,$B1,$CA,$F0,$03,$4C,$05,$04; delay (patched like KERNEL_DELAY1)

KERNEL_DONE:    .byte $4C,$6E,$96       ; back to the cartridge

.org $966E                      ; --- back to cartridge addresses ---

;-----------------------------------------------------------------------------

;  DLI_AFTER_ROW - continues after the kernel. After the top row: colour the

;  "HIGH SCORES:" line and arm DLI_TEXT_LINES. After the bottom row: prepare the top row's

;  colours for the next frame.

;-----------------------------------------------------------------------------

DLI_AFTER_ROW:  ldx T_LINE              ; top row or bottom row?

bne DAR_BOTTOM

lda T_TEXTCOL           ; top row done: "HIGH SCORES:" colour

sta (pCOLPF1),y         ; COLPF1 = A

ldx #>DLI_TEXT_LINES    ; next DLI = DLI_TEXT_LINES

ldy #<DLI_TEXT_LINES

jsr SET_DLI             ; install a display-list-interrupt handler

inc T_LINE

lda #$00

tay

sta (pCOLPF2),y         ; COLPF2 = A

beq DAR_EXIT

DAR_BOTTOM:     ldx #$00                ; bottom row done: start over

stx T_LINE

jsr KERNEL_COLS_TOP     ; colours for next frame's top row

DAR_EXIT:       jmp NMI_EXIT            ; common tail of every interrupt handler

;-----------------------------------------------------------------------------

;  DLI_TEXT_LINES - one DLI per text line between the two rows. It colours the two '5's of

;  the side columns that share this band (COLPM0 left, COLPM3 right) and gives each

;  high-score line the next rainbow colour. After the 16th it switches back to

;  DLI_TOP_ROW for the bottom row and prepares the kernel for it.

;-----------------------------------------------------------------------------

DLI_TEXT_LINES: pha                     ; save A, X, Y

txa

pha

tya

pha

ldy #$00

ldx T_LINE              ; which light (if any) is level with this line?

lda LIGHT_ROW,x         ; WSYNC = LIGHT_ROW[X]

sta WSYNC

beq DTL_RAINBOW         ; none

ldx T_SCROLL            ; phase 10-15: lights have moved a slot

cpx #$0A                ; if X < $0A goto DTL_LOW

bcc DTL_LOW

cmp #$0A                ; if A < $0A goto DTL_RAINBOW

bcc DTL_RAINBOW

ldx #$00

stx T_CNT2

sbc #$0A

bcs DTL_COLOUR

DTL_LOW:        cmp #$0A                ; if A >= $0A goto DTL_RAINBOW

bcs DTL_RAINBOW

ldx #$01

stx T_CNT2

DTL_COLOUR:     sta T_CNT               ; colour of the left light

clc

adc T_ROT

tax

lda CHASE_COLOURS+4,x   ; COLPM0 = CHASE_COLOURS+4[X]

sta (pCOLPM0),y         ; COLPM0 = A

lda #$05                ; colour of the right light (the ring runs the other way)

sec

sbc T_CNT

clc

adc T_CNT2

adc T_ROT

tax

lda CHASE_COLOURS-1,x   ; COLPM3 = CHASE_COLOURS-1[X]

sta (pCOLPM3),y         ; COLPM3 = A

DTL_RAINBOW:    lda T_LINE              ; the "HIGH SCORES:" line is not rainbowed

cmp #$02                ; if A == 2 goto DTL_NEXT

beq DTL_NEXT

lda T_COLIX             ; next rainbow colour (index counts down, 15 colours)

sec

sbc #$01

bcs DTL_RAIN_SET

lda #$0E

DTL_RAIN_SET:   sta T_COLIX

tax

lda RAINBOW,x           ; COLPF0 = RAINBOW[X]

sta (pCOLPF0),y         ; COLPF0 = A

DTL_NEXT:       ldx T_LINE              ; the "Press START" line is white

cpx #$0E                ; if X <> $0E goto DTL_COUNT

bne DTL_COUNT

lda #$0F                ; COLPF1 = $0F

sta (pCOLPF1),y         ; COLPF1 = A

DTL_COUNT:      inc T_LINE

ldx #$10                ; 16 lines done?

cpx T_LINE              ; if X <> T_LINE goto DTL_EXIT

bne DTL_EXIT

ldy #<DLI_TOP_ROW       ; yes: next DLI is DLI_TOP_ROW again, for the bottom row

ldx #>DLI_TOP_ROW

jsr SET_DLI             ; install a display-list-interrupt handler

lda #$10                ; the bottom row scrolls the other way: 16 - phase

sec

sbc T_SCROLL

and #$0F

jsr SET_FINE_SCROLL     ; set HSCROL and patch the kernel's delay to match

jsr KERNEL_COLS_BOTTOM  ; colours for the bottom row

DTL_EXIT:       jmp NMI_EXIT            ; common tail of every interrupt handler

;-----------------------------------------------------------------------------

;  CHASE_COLOURS - the ten colours of the chasing lights, repeated so that any run of ten

;  can be read from any start. Each value is hue (high nibble) and brightness (low nibble).

;-----------------------------------------------------------------------------

CHASE_COLOURS:  .byte $0E,$84,$C6,$48,$BA,$E4,$56,$32,$6A,$16

.byte $0E,$84,$C6,$48,$BA,$E4,$56,$32,$6A,$16

.byte $0E,$84,$C6,$48

;-----------------------------------------------------------------------------

;  LIGHT_ROW - for each title DLI line: which light of the side columns sits in it

;  (0 = none). Values above 10 belong to the second half of the scroll.

;-----------------------------------------------------------------------------

LIGHT_ROW:      .byte $BA,$05,$0E,$04,$0D,$00,$03,$00,$0C,$00,$02,$00,$0B,$00,$01,$0A

;-----------------------------------------------------------------------------

;  RAINBOW - 15 colours for the high-score lines.

;-----------------------------------------------------------------------------

RAINBOW:        .byte $1A,$EA,$2A,$FA,$3A,$4A,$5A,$6A,$7A,$8A,$9A,$AA,$BA,$CA,$DA

;-----------------------------------------------------------------------------

;  TITLE_INIT - starting values for T_SCROLL ($B0) .. T_VOLUME ($CA).

;-----------------------------------------------------------------------------

TITLE_INIT:     .byte $00,$00,$00,$00,$D0,$E1,$FF,$00,$00,$00,$36,$02,$03,$02

.byte $02,$04,$00,$00,$00,$02,$00,$FF,$28,$3C,$1E,$01,$60

;-----------------------------------------------------------------------------

;  START_GAME - start a game (DEMO_PTR+1 = 0) or the demo (DEMO_PTR+1 <> 0).

;  A real game starts in station 1 of the zone chosen on the title screen (START_ZONE,

;  1 unless a digit key was used). The demo plays station 1 from a recording.

;-----------------------------------------------------------------------------

START_GAME:     lda #$00                ; screen and NMIs off

sta NMIEN

sta DMACTL

jsr INIT_PLAYERS        ; fresh player variables for both players

ldy DEMO_PTR+1          ; demo?

bne START_DEMO

ldy START_ZONE          ; real game: both players start in START_ZONE

sty ZONE

sty P2_DATA+1           ; (player 2's copy)

jmp PLAY_STATION        ; build the station and play

START_DEMO:     ldx #$01                ; demo: station 1, recording 0

stx STATION

dex

jmp SET_DEMO

;-----------------------------------------------------------------------------

;  DEMO_TABLE - addresses of the recorded demos: 0 = station 1 (after the high-score

;  screen), 2 = the opening credits on station 11.

;-----------------------------------------------------------------------------

DEMO_TABLE:     .word DEMO_STATION1,DEMO_CREDITS

;-----------------------------------------------------------------------------

;  Recorded demo input. Each pair is (stick+button, frames): bits 0-3 are the joystick

;  (PORTA format, 0 = pushed: 1 up, 2 down, 4 left, 8 right), bit 4 is the button

;  (0 = pressed). $1F = nothing, $17 = right, $1B = left, $1E = up, $1D = down,

;  $0F = jump on the spot, $0B/$07 = jump left/right.

;-----------------------------------------------------------------------------

DEMO_STATION1:  .byte $1F,$51,$17,$0B,$1F,$06,$0F,$09,$1F,$1D,$17,$FF,$17,$26,$1F,$16

.byte $0B,$05,$0F,$04,$1F,$1E,$17,$08,$1F,$02,$1B,$AD,$1A,$08,$1E,$36

.byte $1F,$01,$16,$02,$17,$27,$1F,$03,$1B,$5E,$1F,$02,$17,$0B,$1F,$11

.byte $0B,$05,$0F,$04,$1F,$28,$17,$06,$1F,$04,$1B,$25,$1F,$02,$17,$63

.byte $16,$02,$1E,$3E,$16,$02,$17,$B3,$1F,$02,$1B,$7F,$1A,$05,$1B,$01

.byte $1A,$0B,$1E,$23,$16,$01,$17,$7C,$1F,$04,$1A,$01,$1B,$0E,$1A,$09

.byte $1E,$3A,$16,$03,$17,$20,$1F,$02,$1B,$BE,$1F,$58,$1B,$22,$0B,$0A

.byte $1B,$01,$1F,$26,$1B,$08,$1F,$03,$17,$47,$1F,$01,$1D,$08,$19,$02

.byte $1D,$3C,$19,$02,$1B,$3F,$1F,$02,$17,$9C,$1F,$57

DEMO_CREDITS:   .byte $1F,$5E,$17,$FF,$17,$21,$1F,$0D,$1B,$05,$1F,$19,$0F,$12,$1F,$25

.byte $17,$16,$1F,$03,$1B,$0A,$19,$09,$1D,$3D,$19,$11,$1B,$CC,$1F,$1F

.byte $17,$03,$1F,$1A,$0F,$16,$1F,$2C,$0F,$10,$1F,$2A,$0B,$0B,$0F,$03

.byte $1F,$28,$1B,$23,$1F,$03,$17,$07,$1D,$6C,$15,$02,$17,$0C,$1F,$12

.byte $1B,$01,$1F,$1E,$07,$0A,$0F,$03,$1F,$2D,$17,$DE,$1F,$03,$1B,$05

.byte $1F,$19,$0F,$11,$1F,$27,$0F,$19,$1F,$1F,$0F,$12,$1F,$1E,$17,$0D

.byte $1F,$06,$1B,$01,$19,$09,$1D,$45,$19,$2C,$1B,$02,$1F,$08,$17,$05

.byte $1F,$15,$0B,$07,$0F,$07,$1F,$22,$1B,$C5,$1F,$03,$17,$04,$15,$03

.byte $1D,$5C,$15,$02,$17,$FF,$17,$2B

;-----------------------------------------------------------------------------

;  DRAW_SLIDES - draw the station's slides (chutes) from the list at SLIDES.

;  Entry: rows, x (P/M), y (Bob's y at the top), step (signed x shift per row).

;  Each row is 8 pixels of colour 2 (PLOT_COLOR left at 4 by DRAW_LADDERS); the band moves "step" pixels

;  sideways per row, so it slants. The same entries tell MOVE_BOB where slides start.

;-----------------------------------------------------------------------------

DRAW_SLIDES:    ldy #$00

sty CNT1

DS_NEXT:        ldy CNT1

lda (SLIDES),y          ; rows (0 = end of list)

bne DS_ENTRY

rts

DS_ENTRY:       sta SLIDE_LEN

iny                     ; x: P/M coordinates -> pixel column

lda (SLIDES),y          ; PLOT_X = (SLIDES)[Y] - $30

sec

sbc #$30

sta PLOT_X

iny                     ; y: Bob's top -> the line under his feet

lda (SLIDES),y          ; PLOT_Y = (SLIDES)[Y] + $15

clc

adc #$15

sta PLOT_Y

iny                     ; step per row

lda (SLIDES),y          ; CNT3 = (SLIDES)[Y]

sta CNT3

iny

sty CNT1

DS_ROW:         lda #$08                ; 8 pixels per row

sta CNT2

DS_PIXEL:       ldy PLOT_X

ldx PLOT_Y

jsr PLOT_IF_EMPTY       ; set pixel (Y = pixel x 0-159, X = line y 8-191) to colour PLOT_COLOR,

inc PLOT_X

dec CNT2                ; CNT2 -= 1, until 0

bne DS_PIXEL

inc PLOT_Y              ; next line down

lda PLOT_X              ; x = x + 8 - 9 + step + 1 (carry set) = x + step

sec

sbc #$09

adc CNT3

sta PLOT_X

dec SLIDE_LEN           ; SLIDE_LEN -= 1, until 0

bne DS_ROW

beq DS_NEXT

;-----------------------------------------------------------------------------

;  INIT_LIFTS - copy the moving platforms (6-byte records, list at LIFTS) to LIFT_TAB and

;  draw them. Record: 0 timer, 1 width, 2 x, 3 y, 4 travel, 5 speed (signed).

;  Each platform is "width" columns of 3 pixels in colour 3, like a girder (colour 1 on

;  station 8, where it is the lift).

;-----------------------------------------------------------------------------

INIT_LIFTS:     ldy #$FF                ; Y = -1: CP_BYTE pre-increments

sty CNT1

ldy #$08                ; colour 3 ...

lda STATION

cmp #$08                ; if A <> 8 goto IL_SET

bne IL_SET

ldy #$00                ; ... except station 8 (colour 1)

IL_SET:         sty PLOT_COLOR

IL_NEXT:        ldy CNT1

jsr CP_BYTE             ; timer (0 = end of list)

beq CP_RTS

jsr CP_BYTE             ; width

sta CNT0

jsr CP_BYTE             ; x

sta PLOT_X

jsr CP_BYTE             ; y

sta PLOT_Y

jsr CP_BYTE             ; travel (copied only)

jsr CP_BYTE             ; speed (copied only)

sty CNT1

IL_COLUMN:      ldy #$03                ; 3 pixels down

sty CNT3

IL_PIXEL:       ldx PLOT_Y

ldy PLOT_X

jsr PLOT_XY             ; set pixel (x = Y, line = X) to PLOT_COLOR

inc PLOT_Y

dec CNT3                ; CNT3 -= 1, until 0

bne IL_PIXEL

lda PLOT_Y              ; back up and one column right

sec

sbc #$03

sta PLOT_Y

inc PLOT_X

dec CNT0                ; CNT0 -= 1, until 0

bne IL_COLUMN

beq IL_NEXT

CP_BYTE:        iny                     ; copy the next byte of the list into LIFT_TAB

lda (LIFTS),y           ; LIFT_TAB[Y] = (LIFTS)[Y]

sta LIFT_TAB,y

CP_RTS:         rts

;-----------------------------------------------------------------------------

;  DRAW_LADDERS - draw the station's ladders (list at LADDERS) in colour 2.

;  Entry: height, x (P/M), y, style. A ladder is 8 pixels wide: a 2-pixel rail, a

;  4-pixel rung or gap, a 2-pixel rail, with a rung every 4 lines. Style bit 7 makes a

;  "ladder top" that starts 5 lines higher; the low bits set the rung phase.

;-----------------------------------------------------------------------------

DRAW_LADDERS:   lda #$04                ; colour 2

sta PLOT_COLOR

ldy #$00

sty CNT1

DL_NEXT:        ldy CNT1

lda (LADDERS),y         ; height (0 = end of list)

bne DL_ENTRY

rts

DL_ENTRY:       sta CNT0

iny                     ; x -> pixel column

lda (LADDERS),y         ; PLOT_X = (LADDERS)[Y] - $30

sec

sbc #$30

sta PLOT_X

iny                     ; y

lda (LADDERS),y         ; PLOT_Y = (LADDERS)[Y]

sta PLOT_Y

iny                     ; style

lda (LADDERS),y         ; if (LADDERS)[Y] bit 7 clear goto DL_PHASE

bpl DL_PHASE

and #$7F                ; bit 7: extend 5 lines upwards

pha

lda PLOT_Y              ; PLOT_Y = PLOT_Y - 5

sec

sbc #$05

sta PLOT_Y

lda CNT0                ; CNT0 = CNT0 + 5

clc

adc #$05

sta CNT0

pla

DL_PHASE:       sta CNT4                ; rung counter

iny

sty CNT1

DL_LINE:        jsr PLOT2               ; left rail

lda CNT4                ; rung on this line?

bne DL_GAP

jsr PLOT4               ; yes: 4 pixels

jmp DL_RAIL

DL_GAP:         lda PLOT_X              ; no: skip 4 pixels

clc

adc #$04

sta PLOT_X

DL_RAIL:        jsr PLOT2               ; right rail

lda PLOT_X              ; back to the left edge, next line

sec

sbc #$08

sta PLOT_X

inc PLOT_Y

dec CNT4                ; a rung every 4th line

bpl DL_COUNT

ldx #$03

stx CNT4

DL_COUNT:       dec CNT0                ; CNT0 -= 1, goto DL_NEXT if 0

beq DL_NEXT

bne DL_LINE

PLOT4:          ldx #$04                ; 4 pixels in a row

bne PLOT_RUN

PLOT2:          ldx #$02                ; 2 pixels in a row

PLOT_RUN:       stx CNT2

PLOT_RUN_LOOP:  ldx PLOT_Y

ldy PLOT_X

jsr PLOT_IF_EMPTY       ; set pixel (Y = pixel x 0-159, X = line y 8-191) to colour PLOT_COLOR,

inc PLOT_X

dec CNT2                ; CNT2 -= 1, until 0

bne PLOT_RUN_LOOP

rts

;-----------------------------------------------------------------------------

;  DRAW_HLINE - horizontal line of A pixels: line y = X, from pixel x = Y rightwards.

;  (DRAW_HLINE_A takes the line in PLOT_X.) Pixels already drawn are left alone.

;  Note the swapped use of PLOT_X (here the line) and PLOT_Y (here the x position).

;-----------------------------------------------------------------------------

DRAW_HLINE:     stx PLOT_X              ; line y

DRAW_HLINE_A:   sta CNT1                ; length

sty PLOT_Y              ; first x

DH_LOOP:        ldx PLOT_X

ldy PLOT_Y

jsr PLOT_IF_EMPTY       ; set pixel (Y = pixel x 0-159, X = line y 8-191) to colour PLOT_COLOR,

inc PLOT_Y              ; next pixel to the right

dec CNT1                ; CNT1 -= 1, until 0

bne DH_LOOP

rts

;-----------------------------------------------------------------------------

;  DRAW_GIRDERS - draw the station's girders (floors) and count their sections.

;  In: Y/X = list address, A = slope. The list holds three sub-lists, each ended by 0:

;  sloping down (A = 3), sloping up (DRAW_GIRDERS_MORE with A = 5) and flat (A = 0).

;  Entry: sections, x (pixel), y. A section is 4 pixels wide and is the unit Bob

;  claims; every section drawn is added to SECTIONS. A girder is drawn as three thin

;  stripes in colour 3; claiming fills the gaps so the section turns solid. Flat girders alternate two stripe

;  patterns so they

;  look like brickwork, sloped ones move one line per section (y + 4 - slope).

;-----------------------------------------------------------------------------

DRAW_GIRDERS:   stx SPR_PTR+1           ; list address

sty SPR_PTR

ldy #$00

sty CNT1

sty SECTIONS            ; no sections counted yet

ldy #$08                ; colour 3

sty PLOT_COLOR

DRAW_GIRDERS_MORE:

sta CNT4                ; slope for this sub-list

DG_NEXT:        ldy CNT1

lda (SPR_PTR),y         ; sections (0 = end of this sub-list)

bne DG_ENTRY

iny                     ; step past the 0 for the next call

sty CNT1

rts

DG_ENTRY:       sta CNT0

clc                     ; count the sections to be claimed

adc SECTIONS

sta SECTIONS

iny                     ; x

lda (SPR_PTR),y         ; PLOT_X = (SPR_PTR)[Y]

sta PLOT_X

iny                     ; y

lda (SPR_PTR),y         ; PLOT_Y = (SPR_PTR)[Y]

sta PLOT_Y

iny

sty CNT1

DG_SECTION:     lda CNT4                ; flat or sloped?

bne DG_SLOPED

jsr DG_THICK            ; flat: a section with a thick top stripe ...

lda PLOT_Y              ; PLOT_Y = PLOT_Y - 5

sec

sbc #$05

sta PLOT_Y

dec CNT0                ; CNT0 -= 1, goto DG_NEXT if 0

beq DG_NEXT

jsr DG_STRIPES          ; ... then one with thin stripes

ldy #$04

bne DG_RISE

DG_SLOPED:      jsr DG_STRIPES          ; sloped: one section, then move y by (slope - 4)

ldy CNT4

DG_RISE:        dec PLOT_Y

dey                     ; loop until Y = 0

bne DG_RISE

dec CNT0                ; CNT0 -= 1, until 0

bne DG_SECTION

beq DG_NEXT

DG_THICK:       jsr DG_ROW4             ; an extra stripe line, then the three normal stripes

jsr DG_BACK             ; x back 4 pixels, one line down

DG_STRIPES:     ldx #$03                ; three stripes, two lines apart

stx CNT3

DG_STRIPE:      jsr DG_ROW4             ; 4 pixels at (PLOT_X, PLOT_Y), moving right

dec CNT3                ; CNT3 -= 1, goto PIE_RTS if 0

beq PIE_RTS

jsr DG_BACK             ; x back 4 pixels, one line down

inc PLOT_Y              ; PLOT_Y += 1, until 0

bne DG_STRIPE

DG_ROW4:        ldx #$04                ; 4 pixels from (PLOT_X, PLOT_Y), PLOT_X advances

stx CNT2

DG_ROW4_LOOP:   ldy PLOT_X

ldx PLOT_Y

jsr PLOT_IF_EMPTY       ; set pixel (Y = pixel x 0-159, X = line y 8-191) to colour PLOT_COLOR,

inc PLOT_X

dec CNT2                ; CNT2 -= 1, until 0

bne DG_ROW4_LOOP

rts

DG_BACK:        lda PLOT_X              ; x back 4 pixels, one line down

sec

sbc #$04

sta PLOT_X

inc PLOT_Y

rts

;-----------------------------------------------------------------------------

;  PLOT_IF_EMPTY - set pixel (Y = pixel x 0-159, X = line y 8-191) to colour PLOT_COLOR,

;  but only if it is still background. Mode E packs 4 pixels per byte, 2 bits each,

;  leftmost pixel in bits 7-6:

;    byte = LINE_LO/HI[y] + x/4,  bit pair = x and 3.

;  PIXEL_MASK[x and 3] isolates the pair; PIXEL_SET[colour + (x and 3)] supplies it.

;-----------------------------------------------------------------------------

PLOT_IF_EMPTY:  lda LINE_LO,x           ; address of line y

sta PTR

lda LINE_HI,x           ; PTR+1 = LINE_HI[X]

sta PTR+1

tya                     ; X = x and 3 (pixel within the byte)

and #$03

tax

tya                     ; Y = x / 4 (byte within the line)

lsr a

lsr a

tay

lda PIXEL_MASK,x        ; already drawn?

and (PTR),y

bne PIE_RTS             ; yes: leave it

txa                     ; pick the bit pattern for this colour and position

clc

adc PLOT_COLOR

tax

lda PIXEL_SET,x

ora (PTR),y

sta (PTR),y

PIE_RTS:        rts

;-----------------------------------------------------------------------------

;  PLOT - set pixel (PLOT_X, PLOT_Y) to colour PLOT_COLOR, replacing what was there.

;  Entry points: PLOT (from PLOT_X/PLOT_Y), PLOT_YX_Y (x already in Y),

;  PLOT_XY (Y = x, X = y), PLOT_PTR (line address already in PTR).

;-----------------------------------------------------------------------------

PLOT:           ldy PLOT_X

PLOT_YX_Y:      ldx PLOT_Y

PLOT_XY:        lda LINE_LO,x           ; address of line y

sta PTR

lda LINE_HI,x           ; PTR+1 = LINE_HI[X]

sta PTR+1

PLOT_PTR:       tya                     ; X = x and 3

and #$03

tax

tya                     ; Y = x / 4

lsr a

lsr a

tay

lda PIXEL_CLEAR,x       ; clear the old pixel

and (PTR),y

pha

txa

clc

adc PLOT_COLOR

tax

pla

ora PIXEL_SET,x         ; and set the new colour

sta (PTR),y

rts

;-----------------------------------------------------------------------------

;  READ_PIXEL - colour of the pixel at P/M x = PLOT_X, line y = PLOT_Y (0 = background,

;  1 = items and text, 2 = ladders and slides, 3 = girders and platforms).

;  Out: A = colour, Z set if it is colour 3. Lines outside 8-191 read as line 8.

;  READ_PIXEL_XY takes X = y, Y = x; READ_PIXEL_PTR uses the line already in PIX_PTR.

;-----------------------------------------------------------------------------

READ_PIXEL:     ldx PLOT_Y

ldy PLOT_X

READ_PIXEL_XY:  cpx #$C0                ; clamp y into the bitmap

bcs RP_CLAMP

cpx #$08                ; if X >= 8 goto RP_LINE

bcs RP_LINE

RP_CLAMP:       ldx #$08

RP_LINE:        lda LINE_LO,x           ; PIX_PTR = LINE_LO[X]

sta PIX_PTR

lda LINE_HI,x           ; PIX_PTR+1 = LINE_HI[X]

sta PIX_PTR+1

READ_PIXEL_PTR: tya                     ; P/M x -> pixel x

sec

sbc #$30

tay

and #$03                ; X = x and 3

tax

tya                     ; Y = x / 4

lsr a

lsr a

tay

lda (PIX_PTR),y

RP_SHIFT:       cpx #$03                ; shift the pair down to bits 1-0

beq RP_DONE

lsr a

lsr a

inx

bne RP_SHIFT

RP_DONE:        and #$03

cmp #$03                ; Z = colour 3?

RP_RTS:         rts

;-----------------------------------------------------------------------------

;  Station 11 (the opening credits) has no status line: these 4 bytes replace the top of

;  the display list with "8 blank lines + DLI" and "mode E, LMS $2100".

;-----------------------------------------------------------------------------

DL_CREDITS_TOP: .byte $F0,$4E,$00,$21

;-----------------------------------------------------------------------------

;  BUILD_DLIST - build the game screen's display list at $2000 and the line tables.

;    $2000  3 x "8 blank lines"                      (top border)

;    $2003  mode 2 + LMS $20D8 + DLI                  status line (40 text characters)

;    $2006  184 x mode E (160 x 1, 4 colours)        the playfield bitmap at $2100

;           bitmap lines 46, 93 and 140 also request a DLI (mutant bands, see DLI_MUTANTS)

;           line 96 reloads the address ($3000): ANTIC cannot cross a 4K boundary

;    $20C0  JVB $2000

;  Then LINE_LO/LINE_HI[y] = $2100 + 40 * (y - 8) for y = 8..191, and the bitmap is cleared.

;-----------------------------------------------------------------------------

BUILD_DLIST:    lda #$70                ; 3 lines of 8 blank scan lines

sta DLIST

sta DLIST+1

sta DLIST+2

lda #$C2                ; status line: mode 2 + LMS + DLI

sta DLIST+3

lda #$D8                ; at $20D8

sta DLIST+4

lda #$20                ; DLIST+5 = $20

sta DLIST+5

lda #$0E                ; 186 x mode E (two are overwritten by the LMS below)

ldx #$BA

BD_FILL:        sta DLIST+5,x

dex                     ; loop until X = 0

bne BD_FILL

lda STATION             ; the credits screen has no status line

cmp #$0B                ; if A == $0B goto BD_CREDITS

beq BD_CREDITS

lda #$8E                ; DLIs for the mutant bands

sta DLIST+52

sta $2063

sta $2094

bne BD_LMS

BD_CREDITS:     ldy #$04

BD_CREDITS_LOOP:

lda RP_RTS,y            ; DLIST+2[Y] = RP_RTS[Y]

sta DLIST+2,y

dey                     ; loop until Y = 0

bne BD_CREDITS_LOOP

BD_LMS:         lda #$4E                ; line 96: mode E + LMS $3000 (across the 4K boundary)

sta $2066

lda #$00                ; $2067 = 0

sta $2067

lda #$30                ; $2068 = $30

sta $2068

lda #$41                ; JVB $2000

sta $20C0

lda #$00                ; $20C1 = 0

sta $20C1

lda #$20                ; $20C2 = $20

sta $20C2

ldx #$00                ; line tables for y = 8 ... 191

stx PTR

txa

ldy #$21

BD_TABLE:       sta LINE_LO+8,x

tya

sta LINE_HI+8,x

lda PTR                 ; + 40 bytes per line

clc

adc #$28

sta PTR

bcc BD_TABLE_NEXT

iny

BD_TABLE_NEXT:  inx

cpx #$B8                ; if X <> $B8 goto BD_TABLE

bne BD_TABLE

ldx #$20                ; clear $20D8 onwards: status line + bitmap ($1D28 bytes)

stx PTR+1

ldx #$D8

stx PTR

ldx #$E3

ldy #$18

;-----------------------------------------------------------------------------

;  ZERO_MEM - zero memory from (PTR) for -(X:Y) bytes.

;-----------------------------------------------------------------------------

ZERO_MEM:       stx CNT1

sty CNT0

lda #$00

tay

ZM_LOOP:        sta (PTR),y

inc PTR                 ; PTR += 1, until 0

bne ZM_COUNT

inc PTR+1

ZM_COUNT:       inc CNT0                ; CNT0 += 1, until 0

bne ZM_LOOP

inc CNT1                ; CNT1 += 1, until 0

bne ZM_LOOP

rts

;-----------------------------------------------------------------------------

;  PIXEL_SET - bits to OR in, indexed by PLOT_COLOR + (x and 3):

;    +0 colour 1 ($40 $10 $04 $01)   +4 colour 2 ($80 $20 $08 $02)   +8 colour 3 (PIXEL_MASK)

;   +12 background (the zeros after PIXEL_MASK).

;  PIXEL_MASK - the two bits of each pixel ($C0 $30 $0C $03); doubles as colour 3.

;  PIXEL_CLEAR - the complement, to clear one pixel.

;-----------------------------------------------------------------------------

PIXEL_SET:      .byte $40,$10,$04,$01

.byte $80,$20,$08,$02

PIXEL_MASK:     .byte $C0,$30,$0C,$03

.byte $00,$00,$00,$00

PIXEL_CLEAR:    .byte $3F,$CF,$F3,$FC

;-----------------------------------------------------------------------------

;  SETUP_PULVERIZERS - station 9 ("The Pulverizers").

;  Draws the 3-line-thick beam the pulverizers hang from (lines $9D-$9F, x 40-131), gives

;  the four pulverizers a random starting delay, draws their heads in the raised position

;  (y = $A0), and draws the two 2-pixel posts at the ends of the frame.

;-----------------------------------------------------------------------------

SETUP_PULVERIZERS:

ldx #$9D                ; first line of the beam

stx PLOT_X

ldx #$03                ; 3 lines thick

stx CNT3

ldx #$08                ; colour 3

stx PLOT_COLOR

SP_BEAM:        ldy #$28                ; from x = 40 ...

lda #$5C                ; ... 92 pixels long

jsr DRAW_HLINE_A        ; horizontal line: line = PLOT_X, from x = Y, A pixels

inc PLOT_X

dec CNT3                ; CNT3 -= 1, until 0

bne SP_BEAM

ldx #$04                ; 4 pulverizers

stx CNT6

ldy #$00

SP_INIT:        lda (pAUDCTL,x)         ; X = 4 here, so this reads (pSKRES) = RANDOM: random start delay

sta PULV_TAB,y

lda #$01                ; moving down when it starts

sta PULV_TAB+1,y

lda #$A0                ; at the top (y = $A0)

sta PULV_TAB+2,y

iny                     ; next 3-byte record

iny

iny

dec CNT6                ; CNT6 -= 1, until 0

bne SP_INIT

lda #$0B                ; draw the four heads at byte columns $0B, $11, $17, $1D

SP_HEADS:       ldx #$A0

tay

pha

jsr DRAW_PULV           ; draw a pulverizer head with its top line at y = X, byte column Y

pla

clc

adc #$06

cmp #$1E                ; if A < $1E goto SP_HEADS

bcc SP_HEADS

ldx #$28                ; the two posts, at pixel x = $28 and $82

jsr DRAW_POST           ; 2-pixel-wide post, 31 lines, at pixel x = X

ldx #$82

DRAW_POST:      stx PLOT_X              ; 2 pixels wide, 31 lines from y = $A0, colour 2

ldy #$A0

sty PLOT_Y

lda #$04                ; PLOT_COLOR = 4

sta PLOT_COLOR

lda #$1F                ; CNT3 = $1F

sta CNT3

DP_LOOP:        jsr PLOT                ; set pixel (PLOT_X, PLOT_Y) to colour PLOT_COLOR, replacing what was th

inc PLOT_X

jsr PLOT                ; set pixel (PLOT_X, PLOT_Y) to colour PLOT_COLOR, replacing what was th

dec PLOT_X

inc PLOT_Y

dec CNT3                ; CNT3 -= 1, until 0

bne DP_LOOP

DP_RTS:         rts

;-----------------------------------------------------------------------------

;  PULV_SHAPE - a pulverizer head, read backwards by DRAW_PULV: 6 lines of 3 bytes

;  (12 pixels). The top line is a thin rod: as the head moves down, each frame leaves

;  that line behind, so the rod grows from the beam. The blank bottom line erases the

;  head's old bottom line as it rises.

;-----------------------------------------------------------------------------

PULV_SHAPE:     .byte $00,$00,$00       ; .... .... ....

.byte $55,$55,$55       ; 1111 1111 1111

.byte $55,$55,$55       ; 1111 1111 1111

.byte $50,$55,$05       ; 11.. 1111 ..11

.byte $50,$55,$05       ; 11.. 1111 ..11

.byte $00,$AA,$00       ; .... 2222 ....

;-----------------------------------------------------------------------------

;  DRAW_PULV - draw a pulverizer head with its top line at y = X, byte column Y.

;-----------------------------------------------------------------------------

DRAW_PULV:      lda LINE_LO,x           ; line address

sta PTR

lda LINE_HI,x           ; PTR+1 = LINE_HI[X]

sta PTR+1

ldx #$12                ; 18 bytes, last first

DPV_LINE:       lda #$03                ; 3 bytes per line

sta CNT2

DPV_BYTE:       lda DP_RTS,x            ; (PTR)[Y] = DP_RTS[X]

sta (PTR),y

iny

dex

dec CNT2                ; CNT2 -= 1, until 0

bne DPV_BYTE

dey                     ; back to the first byte, next line

dey

dey

lda PTR                 ; PTR = PTR + $28

clc

adc #$28

sta PTR

bcc DPV_NEXT

inc PTR+1

DPV_NEXT:       inx

dex                     ; loop until X = 0

bne DPV_LINE

rts

;-----------------------------------------------------------------------------

;  MOVE_PULVERIZERS - station 9, every frame. Each head waits, drops 22 lines to y = $B6,

;  rises back to $A0, then waits again. The wait gets shorter as the zone rises:

;    delay = (18 - 2 * min(zone, 9)) * 8 + 40 frames

;-----------------------------------------------------------------------------

MOVE_PULVERIZERS:

lda STATION             ; only on station 9

cmp #$09                ; if A <> 9 goto MP_RTS

bne MP_RTS

ldy #$0B                ; Y = byte column of the first head

ldx #$00

MP_LOOP:        lda PULV_TAB,x          ; waiting?

bne MP_WAIT

lda PULV_TAB+1,x        ; move by the direction (+1 / -1)

clc

adc PULV_TAB+2,x

sta PULV_TAB+2,x

cmp #$B6                ; at the bottom: turn round

beq MP_BOTTOM

cmp #$A0                ; back at the top?

bne MP_DRAW

lda #$01                ; yes: moving down next time ...

sta PULV_TAB+1,x

lda ZONE                ; ... after a delay that depends on the zone

cmp #$0A                ; if A < $0A goto MP_DELAY

bcc MP_DELAY

lda #$09

MP_DELAY:       asl a

sta PLOT_X

lda #$12

sec

sbc PLOT_X

asl a

asl a

asl a

clc

adc #$28

sta PULV_TAB,x

lda #$A0                ; redraw at the top

bne MP_DRAW

MP_BOTTOM:      lda #$FF                ; direction = up

sta PULV_TAB+1,x

lda #$B6

MP_DRAW:        stx PLOT_X              ; redraw the head at its new height

sty PLOT_Y

tax

jsr DRAW_PULV           ; draw a pulverizer head with its top line at y = X, byte column Y

ldx PLOT_X

ldy PLOT_Y              ; if PLOT_Y <> 0 goto MP_NEXT

bne MP_NEXT

MP_WAIT:        dec PULV_TAB,x          ; count the delay down

MP_NEXT:        inx                     ; next record and next column (+6 bytes = 24 pixels)

inx

inx

tya

clc

adc #$06

tay

cmp #$1E                ; if A < $1E goto MP_LOOP

bcc MP_LOOP

MP_RTS:         rts

;-----------------------------------------------------------------------------

;  GAME_OVER_TUNE - plays "Oh My Darling, Clementine" ("...dwelt a miner, forty-niner")

;  three times: on voice 1, then on voice 2, then on both. START, the button, a digit key

;  or OPTION cuts it short; at the end the high-score screen appears.

;

;  TUNE holds (note, length) pairs; a note is an AUDF1 divider (smaller = higher), $FE ends

;  a phrase and $FF the tune. Voice 2 does not read the table: its divider is worked out

;  from the melody note, (note - $1D) * 4 + $79 ($6C for note $1A), which puts it about

;  two octaves lower. Each note starts at volume 15 (pure tone, AUDC $AF), holds for

;  length+2 frames, then fades one step per frame to volume 6.

;-----------------------------------------------------------------------------

GAME_OVER_TUNE: ldx #$01                ; pass 1: voice 1 only

GOT_PASS:       stx MOVE_B              ; voice mask: 1 = voice 1, 2 = voice 2, 3 = both

ldx #$00

GOT_NOTE:       lda TUNE,x              ; next byte of the tune

bpl GOT_PLAY            ; a note?

ldy #$A5                ; end of phrase: fade from volume 5 down to 0 ...

GOT_FADE:       jsr SET_TUNE_VOL        ; write AUDC value Y to the voices selected in MOVE_B (bit 0 = voice 1,

jsr WAIT_FRAME_KEYS     ; ... two frames per step

jsr WAIT_FRAME_KEYS     ; wait for the next frame, checking OPTION, digit keys and START while

dey

cpy #$9F                ; if Y <> $9F goto GOT_FADE

bne GOT_FADE

lda TUNE,x              ; end of the tune?

cmp #$FF                ; if A <> $FF goto GOT_NEXT

bne GOT_NEXT

ldy #$14                ; yes: 20 frames of silence

GOT_REST:       jsr WAIT_FRAME_KEYS     ; wait for the next frame, checking OPTION, digit keys and START while

dey                     ; loop until Y = 0

bne GOT_REST

ldx MOVE_B              ; next pass

inx

cpx #$04                ; if X <> 4 goto GOT_PASS

bne GOT_PASS

jmp TITLE_SCREEN        ; three passes played: the high-score screen

GOT_PLAY:       ldy #$00                ; melody on voice 1

sta (pAUDF1),y          ; work out voice 2's note from the melody note

cmp #$1A                ; if A <> $1A goto GOT_BASS

bne GOT_BASS

lda #$6C

bne GOT_SET2

GOT_BASS:       sec

sbc #$1D

asl a

asl a

clc

adc #$79

GOT_SET2:       sta (pAUDF2),y          ; AUDF2 = A

ldy #$AF                ; attack: pure tone, volume 15

jsr SET_TUNE_VOL        ; write AUDC value Y to the voices selected in MOVE_B (bit 0 = voice 1,

inx                     ; length

lda TUNE,x

tay                     ; + 2 frames

iny

iny

GOT_HOLD:       jsr WAIT_FRAME_KEYS     ; wait for the next frame, checking OPTION, digit keys and START while

dey                     ; loop until Y = 0

bne GOT_HOLD

ldy #$AE                ; decay: volume 14 down to 6, one per frame

GOT_DECAY:      jsr SET_TUNE_VOL        ; write AUDC value Y to the voices selected in MOVE_B (bit 0 = voice 1,

jsr WAIT_FRAME_KEYS     ; wait for the next frame, checking OPTION, digit keys and START while

dey

cpy #$A5                ; if Y <> $A5 goto GOT_DECAY

bne GOT_DECAY

GOT_NEXT:       inx

bne GOT_NOTE

;-----------------------------------------------------------------------------

;  WAIT_FRAME_KEYS - wait for the next frame, checking OPTION, digit keys and START while

;  waiting. START/button/digit start a game at once (abandoning the caller).

;-----------------------------------------------------------------------------

WAIT_FRAME_KEYS:

lda FRAMES              ; FRAMES changes once per frame

sty SAVE_Y

WFK_LOOP:       pha

jsr CHECK_OPTION        ; OPTION: high-score screen

jsr CHECK_ZONE_KEY      ; digit: start in that zone

jsr CHECK_START_ANY     ; START or button?

bcc WFK_WAIT

jmp START_GAME          ; start a game (DEMO_PTR+1 = 0) or the demo (DEMO_PTR+1 <> 0)

WFK_WAIT:       pla

cmp FRAMES              ; if A == FRAMES goto WFK_LOOP

beq WFK_LOOP

ldy SAVE_Y

rts

;-----------------------------------------------------------------------------

;  SET_TUNE_VOL - write AUDC value Y to the voices selected in MOVE_B (bit 0 = voice 1,

;  bit 1 = voice 2).

;-----------------------------------------------------------------------------

SET_TUNE_VOL:   lda MOVE_B

lsr a

bcc STV_VOICE2

pha

tya

ldy #$00

sta (pAUDC1),y          ; AUDC1 = A

tay

pla

STV_VOICE2:     lsr a

bcc STV_RTS

tya

ldy #$00

sta (pAUDC2),y          ; AUDC2 = A

tay

STV_RTS:        rts

;-----------------------------------------------------------------------------

;  TUNE - Clementine: (POKEY divider, frames) pairs. $28 = C, $35 = G below it,

;  $1F = E, $1A = G, $1D = F, $23 = D, $2A = B: "In a cav-ern, in a can-yon,

;  ex-ca-vat-ing for a mine ..."

;-----------------------------------------------------------------------------

TUNE:           .byte $28,$06,$28,$02,$28,$04,$35,$04,$1F,$06,$1F,$02,$1F,$04,$28,$04

.byte $28,$06,$1F,$02,$1A,$04,$1A,$04,$1D,$06,$1F,$02,$23,$08,$FE,$23

.byte $06,$1F,$02,$1D,$04,$1D,$04,$1F,$06,$23,$02,$1F,$04,$28,$04,$28

.byte $06,$1F,$02,$23,$04,$35,$04,$2A,$06,$23,$02,$28,$08,$FF

;-----------------------------------------------------------------------------

;  DRAW_TRANSPORTER - stations 3 and 7: draw the column of four numbered transporter

;  booths at x = A (SPECIAL_X; 0 = this station has none).

;  The digits 1-4 are drawn with the font renderer; the booths come from BOOTH_SHAPE,

;  each bit drawn as 2 pixels.

;-----------------------------------------------------------------------------

DRAW_TRANSPORTER:

sta SPECIAL_X

bne DT_DIGITS

rts

DT_DIGITS:      sec                     ; text x = column - $33

sbc #$33

sta TXT_X

ldx #$00                ; colour 1, normal height

stx TXT_COLOR

inx

stx TXT_VSCALE

stx TRANS_LOCK

ldx #$9D                ; first digit ('1') at the bottom, y = $9D

stx TXT_Y

lda #$11                ; CNT1 = $11

sta CNT1

DT_DIGIT:       jsr DRAW_CHAR           ; draw the digit

lda TXT_Y               ; next booth up: 47 lines higher (DRAW_CHAR left y 8 lower)

sec

sbc #$37

sta TXT_Y

inc CNT1                ; digits '1' to '4'

lda CNT1

cmp #$15                ; if A <> $15 goto DT_DIGIT

bne DT_DIGIT

ldx #$04                ; 4 booths

stx CNT0

ldx #$0E                ; starting at y = 14

stx PLOT_Y

DT_BOOTH:       ldy #$00

sty CNT1

sty PLOT_COLOR          ; colour 1

DT_RUN:         lda BOOTH_SHAPE,y       ; lines in this run (0 = booth done)

beq DT_NEXT_BOOTH

sta CNT2

iny

sty CNT1

DT_LINE:        lda SPECIAL_X           ; x = column - $37

sec

sbc #$37

sta PLOT_X

lda BOOTH_SHAPE,y       ; bit pattern for this run

ldy #$08                ; 8 bits

sty CNT3

DT_BIT:         asl a

bcc DT_BIT_NEXT

pha                     ; bit set: 2 pixels

jsr PLOT                ; set pixel (PLOT_X, PLOT_Y) to colour PLOT_COLOR, replacing what was th

ldx PLOT_Y

ldy PLOT_X

iny

jsr PLOT_XY             ; set pixel (x = Y, line = X) to PLOT_COLOR

pla

DT_BIT_NEXT:    inc PLOT_X

inc PLOT_X

dec CNT3                ; CNT3 -= 1, until 0

bne DT_BIT

inc PLOT_Y              ; next line

ldy CNT1

dec CNT2                ; CNT2 -= 1, until 0

bne DT_LINE

iny

bne DT_RUN

DT_NEXT_BOOTH:  dec CNT0                ; CNT0 -= 1, until 0

bne DT_BOOTH

rts

;-----------------------------------------------------------------------------

;  DRAW_CHAR - draw character A (screen code $00-$7F) into the bitmap from the machine's

;  ROM font, at TXT_X/TXT_Y, in colour TXT_COLOR, each font row repeated TXT_VSCALE

;  times. 1 bits are drawn in the ink colour and 0 bits are cleared, so text erases what

;  was under it. On return TXT_X is unchanged and TXT_Y has moved down 8 * TXT_VSCALE

;  lines; DRAW_STRING moves on to the next character position itself.

;  The font address is FONTPAGE*256 + A*8, so the 400/800 and the 5200 each use their own

;  ROM font.

;-----------------------------------------------------------------------------

DRAW_CHAR:      ldy FONTPAGE            ; font page

DC_PAGE:        cmp #$20                ; 32 characters per page of font

bcc DC_FOUND

iny

sec

sbc #$20

jmp DC_PAGE

DC_FOUND:       sty TXT_FONT+1

ldy #$00

sty TXT_FONT

asl a                   ; offset of the character in its page

asl a

asl a

sta TXT_ROW

ldx #$08                ; 8 font rows

stx TXT_ROWS

lda TXT_VSCALE

DC_ROW:         pha                     ; repeat count for this row

ldx #$08                ; 8 pixels

stx TXT_BITS

ldy TXT_ROW

lda (TXT_FONT),y

DC_BIT:         asl a                   ; next bit into carry

pha

bcs DC_INK

lda TXT_COLOR           ; 0 bit: draw it as background (TXT_COLOR 12) ...

cmp #$0C                ; if A == $0C goto DC_BIT_NEXT

beq DC_BIT_NEXT

pha

lda #$0C                ; TXT_COLOR = $0C

sta TXT_COLOR

jsr TXT_PLOT            ; PLOT for the text renderer

pla

sta TXT_COLOR

jmp DC_BIT_NEXT

DC_INK:         jsr TXT_PLOT            ; 1 bit: ink

DC_BIT_NEXT:    inc TXT_X

pla

dec TXT_BITS            ; TXT_BITS -= 1, until 0

bne DC_BIT

inc TXT_Y               ; next line

lda TXT_X               ; TXT_X = TXT_X - 8

sec

sbc #$08

sta TXT_X

pla                     ; same font row again?

sec

sbc #$01

bne DC_ROW

lda TXT_VSCALE          ; next font row

inc TXT_ROW

dec TXT_ROWS            ; TXT_ROWS -= 1, until 0

bne DC_ROW

rts

;-----------------------------------------------------------------------------

;  TXT_PLOT - PLOT for the text renderer: pixel (TXT_X - no offset, TXT_Y), colour

;  TXT_COLOR. Uses its own pointer (TXT_DST) so it can run inside other drawing.

;-----------------------------------------------------------------------------

TXT_PLOT:       ldy TXT_X

ldx TXT_Y

lda LINE_LO,x           ; TXT_DST = LINE_LO[X]

sta TXT_DST

lda LINE_HI,x           ; TXT_DST+1 = LINE_HI[X]

sta TXT_DST+1

tya

and #$03

tax

tya

lsr a

lsr a

tay

lda PIXEL_CLEAR,x

and (TXT_DST),y

pha

txa

clc

adc TXT_COLOR

tax

pla

ora PIXEL_SET,x

sta (TXT_DST),y

rts

;-----------------------------------------------------------------------------

;  BOOTH_SHAPE - transporter booth as (lines, 8-bit pattern) runs, 0 = end:

;    2 x 01111110   top of the number window

;    8 x 01000010   sides of the window (the digit goes inside)

;    1 x 01111110   bottom of the window

;    2 x 11111111   roof

;   23 x 10000001   walls

;   11 x 00000000   gap to the next booth

;-----------------------------------------------------------------------------

BOOTH_SHAPE:    .byte $02,$7E,$08,$42,$01,$7E,$02,$FF,$17,$81,$0B,$00,$00

;-----------------------------------------------------------------------------

;  CHECK_OPTION - OPTION (5200: the * key) abandons everything for the high-score screen.

;-----------------------------------------------------------------------------

CHECK_OPTION:   lda IS5200              ; if IS5200 == 0 goto CO_800

beq CO_800

lda KEYCODE

cmp #$02

jmp CO_CHK

CO_800:         lda CONSOL              ; OPTION = CONSOL bit 2

and #$04

CO_CHK:         bne CS_RTS

jmp TITLE_SCREEN        ; the high-score screen with its ring of "chasing lights"

;-----------------------------------------------------------------------------

;  CHECK_START - START (or the button, during the demo) pressed?

;  Out: carry set = start a new game (DEMO_PTR+1 is set to 0 on the way).

;  CHECK_START_ANY skips the "only during the demo" test. START_DLY makes sure START has

;  been released for a few frames first.

;-----------------------------------------------------------------------------

CHECK_START:    lda DEMO_PTR+1          ; in a real game only START counts

beq CS_START

CHECK_START_ANY:

lda IS5200              ; the button counts too (not on the 5200)

bne CS_5200

ldy #$00

lda (pTRIG0),y          ; A = TRIG0

and #$01                ; test bits $01

beq CS_PRESSED

CS_START:       lda IS5200              ; if IS5200 == 0 goto CS_800

beq CS_800

CS_5200:        lda KEYCODE             ; 5200 START key

cmp #$03

jmp CS_CHK

CS_800:         lda CONSOL              ; 400/800 START key

and #$01

CS_CHK:         beq CS_PRESSED

lda START_DLY           ; not pressed: count the release time

beq CS_NO

dec START_DLY

CS_NO:          clc

rts

CS_PRESSED:     lda #$FF                ; forget the key

sta KEYCODE

lda START_DLY           ; released long enough before?

beq CS_YES

lda #$05                ; no: start the release count again

sta START_DLY

bne CS_NO

CS_YES:         sta DEMO_PTR+1          ; A = 0: DEMO_PTR+1 = 0, a real game

sec

CS_RTS:         rts

;-----------------------------------------------------------------------------

;  CANNON_WALLS - station 10: vertical 2-pixel walls (lines, y, x), 0 = end.

;-----------------------------------------------------------------------------

CANNON_WALLS:   .byte $09,$5B,$0D

.byte $09,$5B,$29

.byte $59,$66,$00

.byte $43,$66,$34

;-----------------------------------------------------------------------------

;  CANNON_SHAPE - the cannon, 35 lines of (missile byte, player 3 byte): the four missiles

;  (as a "fifth player") form the left 8 pixels and player 3 the right 8. Read backwards, indexed

;  from 1 (CANNON_SHAPE,y with y = $46..1), so the pairs start at $9EF3 and $9EF2 is unused.

;-----------------------------------------------------------------------------

CANNON_SHAPE:   .byte $00

.byte $0F,$F0           ; ....#### ####....

.byte $0F,$F0           ; ....#### ####....

.byte $0F,$F0           ; ....#### ####....

.byte $0F,$F0           ; ....#### ####....

.byte $0F,$F0           ; ....#### ####....

.byte $0F,$F0           ; ....#### ####....

.byte $1F,$F8           ; ...##### #####...

.byte $1F,$F8           ; ...##### #####...

.byte $1F,$F8           ; ...##### #####...

.byte $1F,$F8           ; ...##### #####...

.byte $1F,$F8           ; ...##### #####...

.byte $1F,$F8           ; ...##### #####...

.byte $1F,$F8           ; ...##### #####...

.byte $1F,$F8           ; ...##### #####...

.byte $1F,$F8           ; ...##### #####...

.byte $3F,$FC           ; ..###### ######..

.byte $3F,$FC           ; ..###### ######..

.byte $3F,$FC           ; ..###### ######..

.byte $3F,$FC           ; ..###### ######..

.byte $FF,$FF           ; ######## ########

.byte $FF,$FF           ; ######## ########

.byte $BF,$FD           ; #.###### ######.#

.byte $BF,$FD           ; #.###### ######.#

.byte $BF,$FD           ; #.###### ######.#

.byte $9F,$F9           ; #..##### #####..#

.byte $8F,$F1           ; #...#### ####...#

.byte $83,$C1           ; #.....## ##.....#

.byte $C1,$83           ; ##.....# #.....##

.byte $61,$86           ; .##....# #....##.

.byte $30,$0C           ; ..##.... ....##..

.byte $18,$18           ; ...##... ...##...

.byte $1F,$F8           ; ...##### #####...

.byte $1F,$F8           ; ...##### #####...

.byte $1E,$78           ; ...####. .####...

.byte $1E,$78           ; ...####. .####...

;-----------------------------------------------------------------------------

;  SETUP_CANNON - station 10 ("The Cannon"): put the cannon into player 3 + missiles at

;  x = $A1, write "TNT Charge=   0 Tons", and draw the TNT store (shelves, walls and its

;  "TNT" sign) and the cannon's track.

;-----------------------------------------------------------------------------

SETUP_CANNON:   lda #$A1                ; cannon starts at x = $A1

sta CANNON_X

ldy #$46                ; copy the shape, 35 lines, into P/M rows $B9-$DB

ldx #$23

SC_COPY:        lda CANNON_SHAPE,y      ; player 3 byte

sta $07B8,x

dey

lda CANNON_SHAPE,y      ; missile byte

sta $03B8,x

dey

dex                     ; loop until X = 0

bne SC_COPY

lda #$00                ; colour 1 for text and lines

sta TXT_COLOR

sta PLOT_COLOR

ldx #$A5                ; "TNT Charge=   0 Tons"

stx TXT_IDX

lda #$01                ; TXT_VSCALE = 1

sta TXT_VSCALE

ldx #$00                ; at x = 0, y = 13, 20 characters

ldy #$0D

lda #$14

jsr DRAW_STRING         ; draw A characters of TEXT_POOL, starting at TXT_IDX, at x = X, y = Y

ldy #$0D                ; shelves of the TNT store: lines $59/$5A and $64/$65

ldx #$59

lda #$1E

jsr DRAW_HLINE          ; horizontal line of A pixels

ldy #$0D

ldx #$5A

lda #$1E

jsr DRAW_HLINE          ; horizontal line of A pixels

ldy #$00

ldx #$64

lda #$36

jsr DRAW_HLINE          ; horizontal line of A pixels

ldy #$00

ldx #$65

lda #$36

jsr DRAW_HLINE          ; horizontal line of A pixels

ldy #$00                ; the store's walls from CANNON_WALLS

SC_WALL:        sty CNT1

lda CANNON_WALLS,y      ; lines (0 = end)

beq SC_SIGN

sta CNT4

lda CANNON_WALLS+1,y    ; y

sta PLOT_Y

lda CANNON_WALLS+2,y    ; x

sta PLOT_X

SC_WALL_LINE:   jsr PLOT                ; 2 pixels wide

inc PLOT_X

jsr PLOT                ; set pixel (PLOT_X, PLOT_Y) to colour PLOT_COLOR, replacing what was th

dec PLOT_X

inc PLOT_Y

dec CNT4                ; CNT4 -= 1, until 0

bne SC_WALL_LINE

ldy CNT1

iny

iny

iny

bne SC_WALL

SC_SIGN:        lda #$04                ; colour 2 ...

sta TXT_COLOR

sta PLOT_COLOR

ldx #$A5                ; ... "TNT" (the first 3 characters of the same string)

stx TXT_IDX

lda #$01                ; TXT_VSCALE = 1

sta TXT_VSCALE

ldx #$10                ; at x = $10, y = $5C

ldy #$5C

lda #$03

jsr DRAW_STRING         ; draw A characters of TEXT_POOL, starting at TXT_IDX, at x = X, y = Y

lda #$40                ; the cannon's track starts at x = $40

;-----------------------------------------------------------------------------

;  DRAW_TRACK - rails on lines $BC/$BD from pixel x = A, 62 pixels long, plus sleepers

;  (2 pixels on, 2 off) on line $BE. Also used for the lift's track on station 8.

;-----------------------------------------------------------------------------

DRAW_TRACK:     pha

tay

ldx #$BC

lda #$3E

jsr DRAW_HLINE          ; horizontal line of A pixels

pla

pha

tay

ldx #$BD

lda #$3E

jsr DRAW_HLINE          ; horizontal line of A pixels

ldy #$BE                ; sleepers on line $BE

sty PLOT_Y

pla

sta PLOT_X

lda #$10                ; 16 sleepers

sta CNT3

DT_SLEEPER:     jsr PLOT                ; set pixel (PLOT_X, PLOT_Y) to colour PLOT_COLOR, replacing what was th

inc PLOT_X

jsr PLOT                ; set pixel (PLOT_X, PLOT_Y) to colour PLOT_COLOR, replacing what was th

inc PLOT_X              ; 4 pixels per sleeper

inc PLOT_X

inc PLOT_X

dec CNT3                ; CNT3 -= 1, until 0

bne DT_SLEEPER

rts

;-----------------------------------------------------------------------------

;  DRAW_WASTE - station 6 ("Radioactive Waste"): a solid box (lines $9E-$BE, x 20-139)

;  with 2-pixel edges in colour 2, and "DANGER / Radioactive / Waste" cut into it in the

;  background colour.

;-----------------------------------------------------------------------------

DRAW_WASTE:     ldx #$00                ; colour 1

stx PLOT_COLOR

ldx #$9E                ; 33 lines from $9E ...

lda #$21                ; CNT2 = $21

sta CNT2

DW_FILL:        ldy #$14                ; ... each from x = 20, 120 pixels long

lda #$78

jsr DRAW_HLINE          ; horizontal line of A pixels

ldx PLOT_X

inx

dec CNT2                ; CNT2 -= 1, until 0

bne DW_FILL

ldy #$00                ; colour 2 edges at the x positions in WASTE_EDGES

sty CNT2

ldy #$04

sty PLOT_COLOR

DW_EDGE:        ldy CNT2

lda WASTE_EDGES,y       ; if WASTE_EDGES[Y] == 0 goto DW_TEXT

beq DW_TEXT

sta PLOT_X

iny

sty CNT2

lda #$21                ; 33 lines

sta CNT3

ldx #$9E

stx PLOT_Y

DW_EDGE_LINE:   jsr PLOT                ; set pixel (PLOT_X, PLOT_Y) to colour PLOT_COLOR, replacing what was th

inc PLOT_Y

dec CNT3                ; CNT3 -= 1, until 0

bne DW_EDGE_LINE

beq DW_EDGE

DW_TEXT:        ldx #$0C                ; text in the background colour (12)

stx TXT_COLOR

ldx #$00                ; string 0 of TEXT_POOL

stx TXT_IDX

lda #$01                ; normal height

sta TXT_VSCALE

ldx #$38                ; "DANGER" at ($38, $A2)

ldy #$A2

lda #$06

jsr DRAW_STRING         ; draw A characters of TEXT_POOL, starting at TXT_IDX, at x = X, y = Y

ldx #$24                ; "Radioactive" at ($24, $AD)

ldy #$AD

lda #$0B

jsr DRAW_STRING         ; draw A characters of TEXT_POOL, starting at TXT_IDX, at x = X, y = Y

ldx #$3C                ; "Waste" at ($3C, $B5)

ldy #$B5

lda #$05

;-----------------------------------------------------------------------------

;  DRAW_STRING - draw A characters of TEXT_POOL, starting at TXT_IDX, at x = X, y = Y.

;  DRAW_STRING_Y is the same with TXT_X already set. Characters are 8 pixels apart;

;  TXT_IDX is left pointing after the string, so strings can be drawn one after another.

;-----------------------------------------------------------------------------

DRAW_STRING:    sta TXT_LEN

stx TXT_X

DRAW_STRING_Y:  sty TXT_Y

DSTR_LOOP:      ldy TXT_IDX             ; next character

lda TEXT_POOL,y

jsr DRAW_CHAR           ; draw character A (screen code $00-$7F) into the bitmap from the machin

lda TXT_X               ; move right 8 pixels ...

clc

adc #$08

sta TXT_X

lda TXT_Y               ; ... and back up to the same top line (8 or 16 lines)

sec

sbc #$08

ldx TXT_VSCALE

dex

beq DSTR_NEXT

sbc #$08

DSTR_NEXT:      sta TXT_Y

inc TXT_IDX

dec TXT_LEN             ; TXT_LEN -= 1, until 0

bne DSTR_LOOP

rts

WASTE_EDGES:    .byte $12,$13,$8C,$8D,$00

;-----------------------------------------------------------------------------

;  GET_BOB_FRAME - SPR_PTR = address of Bob's animation frame BOB_FRAME.

;-----------------------------------------------------------------------------

GET_BOB_FRAME:  ldx BOB_FRAME

lda BOB_FRAME_HI,x      ; SPR_PTR+1 = BOB_FRAME_HI[X]

sta SPR_PTR+1

lda BOB_FRAME_LO,x      ; SPR_PTR = BOB_FRAME_LO[X]

sta SPR_PTR

rts

;-----------------------------------------------------------------------------

;  DRAW_BOB - copy Bob's current frame into players 0, 1 and 2 at height BOB_Y.

;  Bob is three overlaid single-colour players, which gives him three colours (hat and

;  coat, face and hands, legs and boots ...). A frame holds the three layers back to back

;  (20, 17 and 17 lines) with neighbouring layers sharing a byte, 51 bytes in all. Copying

;  backwards with the extra blank lines at the ends also erases his previous position.

;-----------------------------------------------------------------------------

DRAW_BOB:       jsr GET_BOB_FRAME       ; SPR_PTR = address of Bob's animation frame BOB_FRAME

lda BOB_Y               ; player 0: 20 lines, bottom at row BOB_Y + $32

clc

adc #$32

tax

ldy #$14

DB_P0:          lda (SPR_PTR),y         ; TITLE_KERNEL[X] = (SPR_PTR)[Y]

sta TITLE_KERNEL,x

dex

dey                     ; loop until Y = 0

bne DB_P0

lda #$13                ; next layer starts 19 bytes on

jsr ADD_SPR_PTR         ; SPR_PTR = SPR_PTR + A

txa                     ; player 1: 17 lines, bottom at row BOB_Y + $2F

clc

adc #$11

tax

ldy #$11

DB_P1:          lda (SPR_PTR),y         ; PM_P1[X] = (SPR_PTR)[Y]

sta PM_P1,x

dex

dey                     ; loop until Y = 0

bne DB_P1

txa                     ; player 2: 17 lines, bottom at row BOB_Y + $35

clc

adc #$17

tax

ldy #$11

lda #$10                ; next layer starts 16 bytes on

jsr ADD_SPR_PTR         ; SPR_PTR = SPR_PTR + A

DB_P2:          lda (SPR_PTR),y         ; PM_P2[X] = (SPR_PTR)[Y]

sta PM_P2,x

dex

dey                     ; loop until Y = 0

bne DB_P2

rts

;-----------------------------------------------------------------------------

;  SHOW_BOB - draw Bob (upside down once he has been fired from the cannon), and on

;  station 10 hide his legs while he stands inside the loaded cannon.

;-----------------------------------------------------------------------------

SHOW_BOB:       lda CANNON_DONE         ; fired from the cannon?

bne DRAW_BOB_FLIPPED

jsr DRAW_BOB            ; copy Bob's current frame into players 0, 1 and 2 at height BOB_Y

HIDE_IN_CANNON: lda STATION             ; station 10 only

cmp #$0A                ; if A <> $0A goto HIC_RTS

bne HIC_RTS

lda CANNON_ST           ; already inside?

bne HIC_HIDE

lda TNT                 ; TNT loaded?

beq HIC_RTS

lda BOB_FRAME           ; walking or standing?

cmp #$07                ; if A >= 7 goto HIC_RTS

bcs HIC_RTS

lda CANNON_X            ; level with the cannon's mouth ...

clc

adc #$04

cmp BOB_X               ; if A <> BOB_X goto HIC_RTS

bne HIC_RTS

lda BOB_Y               ; ... and at its height?

cmp #$7C                ; if A < $7C goto HIC_RTS

bcc HIC_RTS

cmp #$92                ; if A >= $92 goto HIC_RTS

bcs HIC_RTS

HIC_HIDE:       ldy #$0B                ; blank P/M rows $B9-$C3 of players 0-2 (the cannon covers him)

lda #$00

HIC_LOOP:       sta $04B8,y

sta $05B8,y

sta $06B8,y

dey                     ; loop until Y = 0

bne HIC_LOOP

HIC_RTS:        rts

;-----------------------------------------------------------------------------

;  DRAW_BOB_FLIPPED - as DRAW_BOB but copying forwards, so Bob is upside down (flying

;  out of the cannon head first). Blank lines are written at both ends.

;-----------------------------------------------------------------------------

DRAW_BOB_FLIPPED:

jsr GET_BOB_FRAME       ; SPR_PTR = address of Bob's animation frame BOB_FRAME

lda BOB_Y

clc

adc #$1F

tax

ldy #$03                ; 3 blank lines above

lda #$00

DBF_CLR0:       sta TITLE_KERNEL,x

inx

dey                     ; loop until Y = 0

bne DBF_CLR0

ldy #$14                ; then player 0's 20 lines, top to bottom

DBF_P0:         lda (SPR_PTR),y         ; TITLE_KERNEL[X] = (SPR_PTR)[Y]

sta TITLE_KERNEL,x

inx

dey                     ; loop until Y = 0

bne DBF_P0

lda #$13

jsr ADD_SPR_PTR         ; SPR_PTR = SPR_PTR + A

txa                     ; player 1

sec

sbc #$17

tax

ldy #$06                ; 6 blank lines

lda #$00

DBF_CLR1:       sta PM_P1,x

inx

dey                     ; loop until Y = 0

bne DBF_CLR1

ldy #$11

DBF_P1:         lda (SPR_PTR),y         ; PM_P1[X] = (SPR_PTR)[Y]

sta PM_P1,x

inx

dey                     ; loop until Y = 0

bne DBF_P1

txa                     ; player 2

sec

sbc #$17

tax

ldy #$11

lda #$10

jsr ADD_SPR_PTR         ; SPR_PTR = SPR_PTR + A

DBF_P2:         lda (SPR_PTR),y         ; PM_P2[X] = (SPR_PTR)[Y]

sta PM_P2,x

inx

dey                     ; loop until Y = 0

bne DBF_P2

ldy #$08                ; 8 blank lines below

lda #$00

DBF_CLR2:       sta PM_P2,x

inx

dey                     ; loop until Y = 0

bne DBF_CLR2

jmp HIDE_IN_CANNON      ; station 10: hide Bob's legs inside the loaded cannon

;-----------------------------------------------------------------------------

;  SPARKLE_BOB - draw Bob, then AND every line of his three players with a random number,

;  so he appears as a cloud of flickering dots (transporter beam, death).

;-----------------------------------------------------------------------------

SPARKLE_BOB:    jsr DRAW_BOB            ; copy Bob's current frame into players 0, 1 and 2 at height BOB_Y

ldx #$15                ; 21 lines

stx CNT5

ldx BOB_Y

ldy #$00

SB_LOOP:        lda KERNEL_LINE+27,x

and (pSKRES),y          ; A &= RANDOM

sta KERNEL_LINE+27,x

lda PM_P1+32,x

and (pSKRES),y          ; A &= RANDOM

sta PM_P1+32,x

lda PM_P2+32,x

and (pSKRES),y          ; A &= RANDOM

sta PM_P2+32,x

inx

dec CNT5                ; CNT5 -= 1, until 0

bne SB_LOOP

rts

;-----------------------------------------------------------------------------

;  ADD_SPR_PTR - SPR_PTR = SPR_PTR + A.

;-----------------------------------------------------------------------------

ADD_SPR_PTR:    clc

adc SPR_PTR

sta SPR_PTR

bcc ASP_RTS

inc SPR_PTR+1

ASP_RTS:        rts

;-----------------------------------------------------------------------------

;  COLD_START - power-up / RESET entry (cartridge init vector at $BFFE).

;

;  The cartridge's option byte ($BFFD) is $FF. On the 400/800 its bit 7 makes this a

;  "diagnostic" cartridge: the OS jumps here straight after RESET, before initialising

;  anything, so the game owns the whole machine. On the 5200 the same $FF tells the BIOS

;  to skip its title screen and jump through the same address. One ROM image therefore

;  boots on both machines, and the code below works out which one it is on:

;

;    $F800-$F80F all $00 or $18?  -> those are the first two glyphs (space and '!') of the

;    5200 BIOS font, so this is a 5200. On the 400/800 that address holds OS code.

;

;  It then builds the hardware pointer table at $10-$6F:

;    $10+2r -> GTIA register r     ($D000+r on the 400/800, $C000+r on the 5200)

;    $50+2r -> POKEY register r    ($D200+r on the 400/800, $E800+r on the 5200)

;  and from then on every GTIA/POKEY access goes "LDA (pTRIG0),Y"-style through it. ANTIC

;  sits at $D400 on both machines and is addressed directly.

;-----------------------------------------------------------------------------

COLD_START:     ldy #$01                ; starting zone 1

sty START_ZONE

dey                     ; Y = 0

sty NMIEN               ; no NMIs

sei                     ; no IRQs

sty DMACTL              ; no DMA

sty CHEAT_IDX           ; phone-number code: nothing typed yet

cld

ldx #$0F                ; look at $F800-$F80F

PWR_DETECT:     lda FONT_5200,x         ; if FONT_5200[X] == 0 goto PWR_DETECT_NEXT

beq PWR_DETECT_NEXT

cmp #$18                ; if A <> $18 goto PWR_MACHINE

bne PWR_MACHINE         ; something else: a 400/800

PWR_DETECT_NEXT:

dex

bpl PWR_DETECT

sty WARMST              ; 5200: there is no warm-start flag, so force "cold" (clear the high scores)

iny                     ; Y = 1

PWR_MACHINE:    tya

sta IS5200              ; 0 = 400/800, 1 = 5200

beq PWR_800

ldy #$C0                ; 5200: GTIA page $C0, POKEY page $E8, font at $F800

lda #$E8

ldx #$F8

bne PWR_TABLE

PWR_800:        ldy #$D0                ; 400/800: GTIA page $D0, POKEY page $D2, font at $E000

lda #$D2

ldx #$E0

PWR_TABLE:      stx FONTPAGE

pha

lda #$00                ; 32 pointers to GTIA registers 0-31 at $10-$4F

tax

PWR_GTIA:       sta pHPOSP0,x

sty pHPOSP0+1,x

clc

adc #$01

inx

inx

cpx #$40                ; if X <> $40 goto PWR_GTIA

bne PWR_GTIA

pla                     ; 16 pointers to POKEY registers 0-15 at $50-$6F

tay

lda #$00

PWR_POKEY:      sta pHPOSP0,x

sty pHPOSP0+1,x

clc

adc #$01

inx

inx

cpx #$60                ; if X <> $60 goto PWR_POKEY

bne PWR_POKEY

lda WARMST              ; power-on (not RESET): clear the high-score table

bne PWR_TEXT

ldx #$C8

PWR_CLEAR_HI:   sta HISCORES-1,x

dex                     ; loop until X = 0

bne PWR_CLEAR_HI

nop                     ; three NOPs: room left for a patch

nop

nop

PWR_TEXT:       ldx #$28                ; copy "Press START For a 1 Miner Game" to TEXT_RAM

PWR_TEXT_LOOP:  lda TITLE_TEXT-1,x      ; TEXT_RAM-1[X] = TITLE_TEXT-1[X]

sta TEXT_RAM-1,x

dex                     ; loop until X = 0

bne PWR_TEXT_LOOP

stx TWO_PLAYER          ; one player

;-----------------------------------------------------------------------------

;  RESTART - the cartridge's start address ($BFFA). Starts the attract sequence: the

;  opening credits on station 11, played from the DEMO_CREDITS recording.

;-----------------------------------------------------------------------------

RESTART:        jsr HW_RESET            ; silence and blank everything

jsr INIT_PLAYERS        ; fresh player variables

lda #$0B                ; station 11 = the credits screen

sta STATION

ldx #$02                ; recording 2 (DEMO_CREDITS)

SET_DEMO:       lda DEMO_TABLE,x        ; DEMO_PTR = DEMO_TABLE[X]

sta DEMO_PTR

lda DEMO_TABLE+1,x      ; DEMO_PTR+1 = DEMO_TABLE+1[X]

sta DEMO_PTR+1

;-----------------------------------------------------------------------------

;  PLAY_STATION - build STATION and run it until the turn ends.

;-----------------------------------------------------------------------------

PLAY_STATION:   jsr HW_RESET            ; silence and blank everything

jsr BUILD_STATION       ; build the station (prepare screen, drawing, start-up)

lda STATION             ; the credits screen has its own loop

cmp #$0B                ; if A <> $0B goto GAME_LOOP

bne GAME_LOOP

jmp CREDITS_LOOP        ; station 11, the opening credits

;-----------------------------------------------------------------------------

;  GAME_LOOP - foreground loop while a station is played. All the action happens in the

;  vertical-blank interrupt (GAME_VBI); this loop only watches the keys and waits for the

;  turn to end (ALIVE = 0) or the station's end-of-station wipe to finish (WIPE_LINE = 0).

;-----------------------------------------------------------------------------

GAME_LOOP:      lda ALIVE               ; turn over?

beq TURN_OVER

lda DEMO_PTR+1          ; demo: a digit key starts a real game

beq GL_KEYS

jsr CHECK_ZONE_KEY      ; on the title screens a digit key starts a one-player game at that zone

GL_KEYS:        jsr SHOW_TNT            ; station 10: show the TNT charge

lda IS5200              ; if IS5200 <> 0 goto GL_START

bne GL_START

jsr CHECK_OPTION        ; OPTION -> high-score screen (400/800)

GL_START:       jsr CHECK_START         ; START (or the button during the demo) -> new game

bcc GL_WIPE

jmp START_GAME          ; start a game (DEMO_PTR+1 = 0) or the demo (DEMO_PTR+1 <> 0)

GL_WIPE:        lda WIPE_LINE           ; station cleared and the screen wiped away?

bne GAME_LOOP

sta NMIEN               ; next station: NMIs off (A = 0)

ldx STATION

inx

cpx #$0B                ; after station 10 ...

bcc GL_STATION

ldx #$01                ; ... back to station 1 of the next zone

ldy ZONE

iny

beq GL_STATION          ; (the zone stops at 255)

sty ZONE

GL_STATION:     stx STATION

lda DEMO_PTR+1          ; demo: back to the credits

beq PLAY_STATION

jmp RESTART             ; the cartridge's start address ($BFFA)

;-----------------------------------------------------------------------------

;  TURN_OVER - Bob has died. If that was his last life, show "Game Over / Miner n" in a

;  box with a little siren of sound; then hand over to the next player, or end the game.

;-----------------------------------------------------------------------------

TURN_OVER:      lda LIVES               ; lives left?

beq GAME_OVER_BOX

jmp TURN_NEXT

GAME_OVER_BOX:  ldx #$28                ; fill a box (x 40-119, lines $5C-$6B) in colour 3, column by column

stx PLOT_X

jsr SILENCE             ; all four voices off (SILENCE_234 leaves voice 1 alone; A must be 0)

ldx #$08

stx PLOT_COLOR

lda #$AF                ; voices 1 and 2: pure tone, volume 15 (X = 8 makes these AUDC1/AUDC2)

sta (pGRACTL,x)

sta (pCONSOL,x)

GOB_COLUMN:     ldx #$5C

stx PLOT_Y

lda FRAMES              ; one column per frame

GOB_WAIT:       cmp FRAMES              ; if A == FRAMES goto GOB_WAIT

beq GOB_WAIT

GOB_PIXEL:      lda PLOT_X              ; two tones that rise with the column

sec

sbc #$1E

ldy #$00

sta (pAUDF1),y          ; AUDF1 = A

adc #$6D

sta (pAUDF2),y          ; AUDF2 = A

jsr PLOT                ; set pixel (PLOT_X, PLOT_Y) to colour PLOT_COLOR, replacing what was th

ldx #$00                ; status line stops blinking

stx CHACTL

dex

stx BLINK_TMR

inc PLOT_Y

ldx PLOT_Y

cpx #$6C                ; if X <> $6C goto GOB_PIXEL

bne GOB_PIXEL

inc PLOT_X

ldy PLOT_X

cpy #$78                ; if Y <> $78 goto GOB_COLUMN

bne GOB_COLUMN

lda #$00                ; silence voice 2; voice 1 to a low hum

tay

sta (pAUDC2),y          ; AUDC2 = A

lda #$6F                ; AUDC1 = $6F

sta (pAUDC1),y          ; AUDC1 = A

sta PLOT_Y

lda #$FF                ; AUDF1 = $FF

sta (pAUDF1),y          ; AUDF1 = A

lda #$32                ; PLOT_X = $32

sta PLOT_X

lda #$0C                ; text in the background colour, cut into the box

sta TXT_COLOR

ldx #$97                ; "Game Over" (TEXT_POOL + $97)

stx TXT_IDX

lda #$01                ; TXT_VSCALE = 1

sta TXT_VSCALE

ldx #$2C

ldy #$5C

lda #$09

jsr DRAW_STRING         ; draw A characters of TEXT_POOL, starting at TXT_IDX, at x = X, y = Y

ldx #$34                ; "Miner" below it

ldy #$64

lda #$05

jsr DRAW_STRING         ; draw A characters of TEXT_POOL, starting at TXT_IDX, at x = X, y = Y

ldx #$64                ; then the player's number

stx TXT_X

lda PLAYER

clc

adc #$11

jsr DRAW_CHAR           ; draw character A (screen code $00-$7F) into the bitmap from the machin

lda FRAMES              ; wait 3 seconds while the hum fades

adc #$B4

tax

GOB_FADE:       lda FRAMES

GOB_FADE_WAIT:  cmp FRAMES              ; if A == FRAMES goto GOB_FADE_WAIT

beq GOB_FADE_WAIT

dec PLOT_X              ; every 4 frames ...

bne GOB_FADE_NEXT

lda #$04                ; PLOT_X = 4

sta PLOT_X

lda PLOT_Y              ; ... one step quieter (down to volume 0)

cmp #$60                ; if A == $60 goto GOB_FADE_NEXT

beq GOB_FADE_NEXT

sec

sbc #$01

sta PLOT_Y

ldy #$00

sta (pAUDC1),y          ; AUDC1 = A

GOB_FADE_NEXT:  cpx FRAMES              ; if X <> FRAMES goto GOB_FADE

bne GOB_FADE

TURN_NEXT:      jsr NEXT_PLAYER         ; after a turn

jmp PLAY_STATION        ; build STATION and run it until the turn ends

;-----------------------------------------------------------------------------

;  NEXT_PLAYER - after a turn: in the demo go back to the credits; in a two-player game

;  swap to the other player if he has lives left; otherwise carry on, or if nobody has

;  lives left, enter the high scores and show the high-score screen.

;-----------------------------------------------------------------------------

NEXT_PLAYER:    lda DEMO_PTR+1          ; demo?

beq NP_REAL

jmp RESTART             ; the cartridge's start address ($BFFA)

NP_REAL:        lda a:$00C3             ; the other player still in the game?

beq NP_ALONE

ldx #$0A                ; swap STATION..SCORE ($B7-$C0) with the other player's copy ($C1-$CA)

NP_SWAP:        lda PLAYER+1,x

ldy a:$00C0,x

sty PLAYER+1,x

sta a:$00C0,x

dex                     ; loop until X = 0

bne NP_SWAP

lda #$01                ; PLAYER = 1 - PLAYER

sec

sbc PLAYER

sta PLAYER

NP_GO:          lda #$00                ; a life will be taken when the turn starts

sta LIFE_USED

rts

NP_ALONE:       lda LIVES               ; this player still has lives?

bne NP_GO

jsr HISCORE_CHECK       ; everybody is out: high-score table ...

jmp TITLE_SCREEN        ; ... and the high-score screen

;-----------------------------------------------------------------------------

;  CREDITS_LOOP - station 11, the opening credits. The credits ("MINER 2049er / By /

;  Bill Hogue / Starring "Bounty Bob" / Copyright (C) 1982 By / Big Five Software / ...

;  (213) 782-6861") were drawn at the start in pixel colour 1, whose colour register is

;  black, so they are invisible. As Bob (driven by DEMO_CREDITS) walks along the girders,

;  GAME_VBI repaints every colour-1 pixel in a band around him in colour 2, so the words

;  appear in his wake. When he reaches the bottom right corner the rest is revealed, a

;  DLI rainbow starts on the copyright lines, and the game-over tune plays.

;-----------------------------------------------------------------------------

CREDITS_LOOP:   lda #$05                ; START_DLY = 5

sta START_DLY

CL_LOOP:        jsr CHECK_ZONE_KEY      ; digit: start a game in that zone

jsr CHECK_START_ANY     ; START or the button: start a game

bcc CL_WAIT

jmp START_GAME          ; start a game (DEMO_PTR+1 = 0) or the demo (DEMO_PTR+1 <> 0)

CL_WAIT:        jsr CHECK_OPTION        ; OPTION: high-score screen

ldx BOB_X               ; has Bob reached the bottom right?

cpx #$C8                ; if X < $C8 goto CL_LOOP

bcc CL_LOOP

ldy BOB_Y

cpy #$9F                ; if Y < $9F goto CL_LOOP

bcc CL_LOOP

lda VCOUNT              ; wait until the beam is half way down the screen

cmp #$3F                ; if A < $3F goto CL_LOOP

bcc CL_LOOP

lda #$00                ; stop the VBI, footsteps off

tay

sta NMIEN

sta (pAUDC3),y          ; AUDC3 = A

lda COLPF1_V            ; colour 1 = the revealed-text colour: everything not yet uncovered appears

;   ...

sta (pCOLPF0),y         ; COLPF0 = A

jsr DRAW_CREDITS        ; ... and the credits are drawn again on top

lda COLPF1_V            ; colour for the rainbow DLI

sta PLOT_COLOR

ldx #$C2

stx CNT3

ldx #$AB                ; DLI = DLI_RAINBOW

ldy #$B3

jsr SET_DLI             ; install a display-list-interrupt handler

lda #$80                ; DLI only (no VBI)

sta NMIEN

jmp GAME_OVER_TUNE      ; play Clementine, then the high-score screen

ST_RTS:         rts

;-----------------------------------------------------------------------------

;  SHOW_TNT - station 10: print the TNT charge ("TNT Charge=  nn Tons") on the status

;  text, and print "DANGER" (at pixel $48, line $35) once 4 or more tons are loaded.

;-----------------------------------------------------------------------------

SHOW_TNT:       lda STATION

cmp #$0A                ; if A <> $0A goto ST_RTS

bne ST_RTS

lda SECTIONS            ; station finished?

beq ST_RTS

lda TNT                 ; 4 tons or more?

cmp #$04                ; if A < 4 goto ST_NUMBER

bcc ST_NUMBER

ldy #$04                ; "DANGER" in colour 2 at ($48, $35)

sty TXT_COLOR

ldx #$01

stx CNT4

dex

stx TXT_IDX

ldx #$48

ldy #$35

lda #$06

jsr DRAW_STRING         ; draw A characters of TEXT_POOL, starting at TXT_IDX, at x = X, y = Y

ST_NUMBER:      ldy #$00                ; colour 1

sty TXT_COLOR

lda CANNON_ST           ; fired, or nothing loaded: show 0

cmp #$02                ; if A == 2 goto ST_ZERO

beq ST_ZERO

lda TNT                 ; if TNT == 0 goto ST_ZERO

beq ST_ZERO

ldx #$00                ; tens and units

ST_TENS:        cmp #$0A                ; if A < $0A goto ST_DIGITS

bcc ST_DIGITS

sbc #$0A

inx

bne ST_TENS

ST_DIGITS:      pha

txa                     ; tens digit (blank if 0)

beq ST_TENS_OUT

clc

adc #$10

ST_TENS_OUT:    jsr ST_TENS_X           ; draw the tens digit A at x = $60

pla                     ; units digit

clc

adc #$10

ST_UNITS:       ldx #$68                ; at x = $68

ST_DRAW:        stx TXT_X

ldy #$0D                ; y = 13

sty TXT_Y

jmp DRAW_CHAR           ; draw character A (screen code $00-$7F) into the bitmap from the machin

ST_ZERO:        lda #$00                ; "0"

jsr ST_UNITS            ; draw the units digit A at x = $68

lda #$00                ; and a blank tens digit

ST_TENS_X:      ldx #$60                ; at x = $60

bne ST_DRAW

;-----------------------------------------------------------------------------

;  INIT_PLAYERS - both players to station 1, zone 1, 3 lives, extra life pending, score 0

;  (from PLAYER_INIT). In a one-player game player 2 gets no lives.

;-----------------------------------------------------------------------------

INIT_PLAYERS:   ldx #$0A

IP_LOOP:        lda IP_RTS,x            ; PLAYER+1[X] = IP_RTS[X]

sta PLAYER+1,x

sta a:$00C0,x

dex                     ; loop until X = 0

bne IP_LOOP

stx PLAYER              ; player 1 first

stx LIFE_USED

lda TWO_PLAYER          ; if TWO_PLAYER <> 0 goto IP_RTS

bne IP_RTS

sta a:$00C3             ; one player: the other player has no lives

IP_RTS:         rts

;-----------------------------------------------------------------------------

;  PLAYER_INIT - STATION, ZONE, LIVES, XLIFE, SCORE (6 digits).

;-----------------------------------------------------------------------------

PLAYER_INIT:    .byte $01,$01,$03,$0A,$00,$00,$00,$00,$00,$00

;-----------------------------------------------------------------------------

;  HISCORE_CHECK - after a game, offer the high-score table to both players' scores,

;  the higher score first (so the two never compete for the same slot out of order).

;-----------------------------------------------------------------------------

HISCORE_CHECK:  lda #$00                ; screen off

sta NMIEN

tay

sta DMACTL

sta (pGRACTL),y         ; GRACTL = A

jsr HIDE_PLAYERS        ; players to x = 0 (off screen)

ldy #$0A                ; Y = 10 = player 2's offset if his score is higher

ldx #$00

HC_COMPARE:     lda SCORE,x             ; compare the two scores digit by digit

cmp P2_DATA+4,x         ; ; player 2's digit

bcc HC_ORDER

bne HC_P1

inx

cpx #$06                ; if X <> 6 goto HC_COMPARE

bne HC_COMPARE

HC_P1:          ldy #$00                ; player 1 is higher

HC_ORDER:       sty PTR

jsr INSERT_SCORE        ; the higher score ...

lda #$0A                ; ... then the other one

sec

sbc PTR

sta PTR

jmp INSERT_SCORE        ; put the score at SCORE+CNT1(=PTR) (player 1

;-----------------------------------------------------------------------------

;  INSERT_SCORE - put the score at SCORE+CNT1(=PTR) (player 1: offset 0, player 2: 10)

;  into the high-score table if it beats an entry, and ask for a name.

;  Table lines are 20 screen-code characters; the score sits at columns 11-17 as

;  "nnn,nnn" with leading zeros blank. The top line is shown in ANTIC mode 7, whose

;  characters carry their colour in bits 6-7, so its digits are $50-$59, not $10-$19.

;-----------------------------------------------------------------------------

INSERT_SCORE:   ldx PTR

ldy #$06                ; a score of zero is never entered

lda #$00

IS_ZERO:        ora SCORE,x

inx

dey                     ; loop until Y = 0

bne IS_ZERO

cmp #$00                ; if A == 0 goto IS_RTS

beq IS_RTS

lda #$50                ; line 0 digits are $50-$59

sta PLOT_COLOR

lda #$0A                ; 10 lines

sta PTR+1

lda #$00

IS_LINE:        sta CNT0

tay

lda #$06                ; 6 digits

sta CNT1

ldx PTR

IS_DIGIT:       lda HISCORES+11,y       ; table digit (blank counts as 0)

sec

sbc PLOT_COLOR

cmp #$0A                ; if A < $0A goto IS_CMP

bcc IS_CMP

lda #$00

IS_CMP:         cmp SCORE,x             ; if A < SCORE[X] goto IS_INSERT

bcc IS_INSERT           ; the new score is higher: insert here

bne IS_NEXT_LINE        ; lower: try the next line

inx                     ; equal: next digit

iny

dec CNT1                ; CNT1 -= 1, goto IS_NEXT_LINE if 0

beq IS_NEXT_LINE

lda CNT1

cmp #$03                ; if A <> 3 goto IS_DIGIT

bne IS_DIGIT

iny                     ; skip the comma

bne IS_DIGIT

IS_NEXT_LINE:   lda #$10                ; lines 1-9 use plain digits

sta PLOT_COLOR

lda CNT0

clc

adc #$14

dec PTR+1               ; PTR+1 -= 1, until 0

bne IS_LINE

IS_RTS:         rts

IS_INSERT:      lda #$B4                ; move the lines below down by one (20 bytes)

sec

sbc CNT0

beq IS_CLEAR

tax

ldy #$B3

IS_MOVE:        lda HISCORES,y          ; HISCORES+20[Y] = HISCORES[Y]

sta HISCORES+20,y

dey

dex                     ; loop until X = 0

bne IS_MOVE

IS_CLEAR:       ldx #$14                ; clear the new line

lda #$00

ldy CNT0

IS_CLEAR_LOOP:  sta HISCORES,y

iny

dex                     ; loop until X = 0

bne IS_CLEAR_LOOP

ldx PTR                 ; write the score into it

ldy CNT0

lda #$06                ; CNT1 = 6

sta CNT1

lda #$00                ; no digit written yet (leading zeros stay blank)

sta PTR+1

IS_WRITE:       lda SCORE,x             ; if SCORE[X] <> 0 goto IS_DIGIT_OUT

bne IS_DIGIT_OUT

lda PTR+1               ; if PTR+1 == 0 goto IS_WRITE_NEXT

beq IS_WRITE_NEXT

lda #$00

IS_DIGIT_OUT:   clc                     ; digit -> screen code

adc #$10

sta HISCORES+11,y

sta PTR+1

IS_WRITE_NEXT:  iny

inx

dec CNT1                ; CNT1 -= 1, goto IS_NAME if 0

beq IS_NAME

lda CNT1                ; after 3 digits: comma (if any digit has been written)

cmp #$03                ; if A <> 3 goto IS_WRITE

bne IS_WRITE

lda PTR+1               ; if PTR+1 == 0 goto IS_COMMA

beq IS_COMMA

lda #$0C

IS_COMMA:       sta HISCORES+11,y

iny

bne IS_WRITE

IS_NAME:        lda CNT0

pha                     ; ask for the name

jsr NAME_ENTRY          ; "MINER 1 / great score! / enter your name

pla                     ; new top score?

bne IS_RTS2

ldx #$10                ; yes: line 0 to the mode 7 colour ...

IS_RECOLOUR0:   lda HISCORES+1,x        ; HISCORES+1[X] = HISCORES+1[X] + $40

clc

adc #$40

sta HISCORES+1,x

dex                     ; loop until X = 0

bne IS_RECOLOUR0

ldx #$10                ; ... and the old top line, now line 1, back to plain codes

IS_RECOLOUR1:   lda HISCORES+21,x       ; HISCORES+21[X] = HISCORES+21[X] - $40

sec

sbc #$40

sta HISCORES+21,x

dex                     ; loop until X = 0

bne IS_RECOLOUR1

IS_RTS2:        rts

;-----------------------------------------------------------------------------

;  BUILD_STATION - show the "prepare for station" screen, then draw STATION from its

;  description and set everything up for play.

;

;  STATION_TABLE (8 bytes per station, indexed by station number) gives the addresses of

;  four lists: ladders, slides, moving platforms and girders. Three more tables indexed by

;  station give Bob's start position and the transporter column, and two word tables

;  give the item list and the mutant list. Stations with machinery (6, 8, 9, 10) get extra

;  set-up code.

;-----------------------------------------------------------------------------

BUILD_STATION:  jsr PREPARE_SCREEN      ; prepare screen (also builds the empty game screen)

lda STATION             ; X = station * 8

asl a

asl a

asl a

tax

BS_FILLER:      lda #$4C                ; two instructions with no effect (5 bytes of filler; $4C is the JMP opcode,

lda BS_FILLER           ; so this may be space reserved for a patch)

lda STATION_TABLE,x     ; ladder list

sta LADDERS

lda STATION_TABLE+1,x   ; LADDERS+1 = STATION_TABLE+1[X]

sta LADDERS+1

lda STATION_TABLE+2,x   ; slide list

sta SLIDES

lda STATION_TABLE+3,x   ; SLIDES+1 = STATION_TABLE+3[X]

sta SLIDES+1

lda STATION_TABLE+4,x   ; moving-platform list

sta LIFTS

lda STATION_TABLE+5,x   ; LIFTS+1 = STATION_TABLE+5[X]

sta LIFTS+1

lda STATION_TABLE+6,x   ; girder list (Y/X = lo/hi)

tay

lda STATION_TABLE+7,x

tax

lda #$03                ; girders sloping down ...

jsr DRAW_GIRDERS        ; draw the station's girders (floors) and count their sections

lda #$05                ; ... sloping up ...

jsr DRAW_GIRDERS_MORE   ; continue drawing girders with slope A

lda #$00                ; (lift x = 0: no lift)

sta LIFT_X

jsr DRAW_GIRDERS_MORE   ; ... and flat

jsr DRAW_LADDERS        ; draw the station's ladders (list at LADDERS) in colour 2

jsr DRAW_SLIDES         ; draw the station's slides (chutes) from the list at SLIDES

jsr INIT_LIFTS          ; copy the moving platforms (6-byte records, list at LIFTS) to LIFT_TAB

ldx STATION             ; Bob's starting position

lda BOB_START_X,x       ; BOB_X = BOB_START_X[X]

sta BOB_X

lda BOB_START_Y,x       ; BOB_Y = BOB_START_Y[X]

sta BOB_Y

lda TRANS_COLUMN,x      ; transporter column (0 = none)

jsr DRAW_TRANSPORTER    ; stations 3 and 7

lda STATION             ; word tables: X = station * 2

asl a

tax

lda MUTANT_LISTS,x      ; mutant list

sta MUTANTS

lda MUTANT_LISTS+1,x    ; MUTANTS+1 = MUTANT_LISTS+1[X]

sta MUTANTS+1

lda ITEM_LISTS,x        ; item list

sta PTR

lda ITEM_LISTS+1,x      ; PTR+1 = ITEM_LISTS+1[X]

sta PTR+1

jsr SETUP_ITEMS         ; draw the items

lda #$FF                ; mask = $FF: copy the mutants ...

ldx STATION

cpx #$0B                ; if X < $0B goto BS_MUTANTS

bcc BS_MUTANTS

lda #$00                ; ... except on the credits screen (mask 0: no mutants)

BS_MUTANTS:     sta START_DLY

ldy #$00

lda #$08                ; 8 mutant records

sta CNT3

BS_MUT_REC:     ldx #$0B                ; 11 bytes each

BS_MUT_BYTE:    lda (MUTANTS),y

and START_DLY

sta MUT_TAB,y

iny

dex                     ; loop until X = 0

bne BS_MUT_BYTE

lda (pSKRES,x)          ; X = 0, so (pSKRES,x) reads RANDOM: a random colour, never too dark

and #$F6

ora #$02

sta ITEMS+38,y          ; byte 3 of the record just copied = current colour

sta ITEMS+36,y          ; byte 1 = its normal colour

dec CNT3                ; CNT3 -= 1, until 0

bne BS_MUT_REC

lda STATION             ; station 6: the radioactive waste box

cmp #$06                ; if A <> 6 goto BS_SPECIAL

bne BS_SPECIAL

jsr DRAW_WASTE          ; station 6 ("Radioactive Waste")

BS_SPECIAL:     lda STATION             ; credits screen: nothing more

cmp #$0B                ; if A <> $0B goto BS_SPECIAL2

bne BS_SPECIAL2

jmp BS_FLOOR

BS_SPECIAL2:    cmp #$08                ; if A == 8 goto BS_LIFT

beq BS_LIFT             ; station 8: the lift

cmp #$0A                ; if A <> $0A goto BS_SPECIAL3

bne BS_SPECIAL3

jsr SETUP_CANNON        ; station 10: the cannon and the TNT store

jmp BS_BONUS

BS_SPECIAL3:    cmp #$09                ; if A <> 9 goto BS_SPECIAL_DONE

bne BS_SPECIAL_DONE

jsr SETUP_PULVERIZERS   ; station 9: the pulverizers

BS_SPECIAL_DONE:

jmp BS_BONUS

BS_LIFT:        ldx #$07                ; station 8: the lift's cart (7 lines from LIFT_CART into player 3)

BS_LIFT_CART:   lda LIFT_CART-1,x       ; $07D4[X] = LIFT_CART-1[X]

sta $07D4,x

dex                     ; loop until X = 0

bne BS_LIFT_CART

lda #$96                ; lift at x = $96, platform on line $A0

sta LIFT_X

lda #$A0                ; LIFT_Y = $A0

sta LIFT_Y

lda #$04                ; colour 2 for its track

sta PLOT_COLOR

ldx #$02                ; the scissor arms: P/M rows $A1-$B4 of player 3, from LIFT_SCISSOR

ldy #$B4

BS_SCISSOR:     lda LIFT_SCISSOR,x      ; PM_P3+32[Y] = LIFT_SCISSOR[X]

sta PM_P3+32,y

dey

dex

bpl BS_SCISSOR_NEXT

ldx #$06

BS_SCISSOR_NEXT:

cpy #$A1                ; if Y <> $A1 goto BS_SCISSOR

bne BS_SCISSOR

inx                     ; animation phase it was left at

stx LIFT_ANIM

lda #$FF                ; two solid lines for the platform

sta PM_P3+32,y

sta PM_P3+31,y

lda #$3A                ; its track at x = $3A

jsr DRAW_TRACK          ; rails and sleepers from x = A

BS_BONUS:       lda #$10                ; bonus "0000" on the status line

ldx #$04

BS_BONUS_ZERO:  sta STATUS+17,x

dex                     ; loop until X = 0

bne BS_BONUS_ZERO

ldx STATION             ; bonus (hundreds) = BONUS_BASE[station] + 5 * zone, at most 99

lda BONUS_BASE,x

ldx ZONE

BS_BONUS_ADD:   clc

adc #$05

bcs BS_BONUS_MAX

dex                     ; loop until X = 0

bne BS_BONUS_ADD

cmp #$64                ; if A < $64 goto BS_BONUS_TENS

bcc BS_BONUS_TENS

BS_BONUS_MAX:   lda #$63

BS_BONUS_TENS:  cmp #$0A                ; tens digit

bcc BS_BONUS_UNITS

sec

sbc #$0A

inc STATUS+18           ; STATUS+18 += 1, until 0

bne BS_BONUS_TENS

BS_BONUS_UNITS: adc #$10                ; units digit (carry is clear: + $10)

sta STATUS+19

BS_FLOOR:       ldx #$F0                ; bonus timer: first tick after 240 frames

stx BONUS_TMR

ldx #$08                ; the bottom floor: one line, colour 3, the full 160 pixels

stx PLOT_COLOR

ldy #$00

ldx #$BF

lda #$A0

jsr DRAW_HLINE          ; horizontal line of A pixels

jmp START_STATION       ; carry on: colours, variables, NMIs

;-----------------------------------------------------------------------------

;  LIFT_SCISSOR - the lift's scissor arms: a 7-line cycle drawn down the lift's column of

;  player 3, starting at the phase LIFT_ANIM so the arms seem to fold and unfold.

;-----------------------------------------------------------------------------

LIFT_SCISSOR:   .byte $18,$24,$42,$81,$42,$24,$18; ...##... ..#..#.. .#....#. #......# .#....#. ..#..#.. ...##...

;-----------------------------------------------------------------------------

;  LIFT_CART - the lift's cart (7 lines). Its last line, the wheels, is the first byte of

;  BONUS_BASE, which no station uses: the two tables overlap by one byte.

;-----------------------------------------------------------------------------

LIFT_CART:      .byte $7E,$7E,$FF,$FF,$FF,$66; .######. .######. ######## ######## ######## .##..##.

;-----------------------------------------------------------------------------

;  BONUS_BASE - starting bonus for each station, in hundreds, before adding 5 per zone.

;  Index 0 is the cart's wheels; station 10 reads one past the end, the $41 at the start

;  of TRANS_COLUMN (so its bonus starts at 6500 + 500 per zone).

;-----------------------------------------------------------------------------

BONUS_BASE:     .byte $66,$19,$1E,$23,$28,$2D,$32,$37,$28,$3C

;-----------------------------------------------------------------------------

;  TRANS_COLUMN - x of the transporter column per station (0 = none); stations 3 and 7.

;  (Station 11 reads one past the end: BOB_START_X's first byte, 0.)

;-----------------------------------------------------------------------------

TRANS_COLUMN:   .byte $41,$00,$00,$7A,$00,$00,$00,$43,$00,$00,$00

;-----------------------------------------------------------------------------

;  BOB_START_X / BOB_START_Y - where Bob starts on each station (index 1-11). Station 11

;  reads BOB_START_X one past the end, i.e. BOB_START_Y's unused first byte, $30.

;-----------------------------------------------------------------------------

BOB_START_X:    .byte $00,$34,$8A,$AF,$3A,$30,$39,$30,$B1,$C8,$C3

BOB_START_Y:    .byte $30,$AA,$AA,$AA,$AA,$AA,$AA,$AA,$8B,$05,$AA,$0F

;-----------------------------------------------------------------------------

;  SETUP_ITEMS - copy the station's items to ITEMS and draw them.

;  List entry: shape (bit 7 set = present), y, x (pixels). The shape number selects an

;  entry of ITEM_SHAPES: width, height, points, then the picture as whole bytes (4 pixels

;  per byte, 2 bits per pixel), copied straight into the bitmap. x is rounded down to a

;  multiple of 4 so items sit on byte boundaries.

;-----------------------------------------------------------------------------

SETUP_ITEMS:    ldy #$00

SI_LOOP:        lda (PTR),y             ; shape (0 = end of list)

sta ITEMS,y

beq BL_RTS

and #$7F

asl a

tax

SI_FILLER:      lda NMI_EXIT            ; three instructions with no effect (8 bytes of filler)

lda #$4C

lda SI_FILLER

lda ITEM_SHAPES,x       ; address of the shape

sta CNT3

lda ITEM_SHAPES+1,x     ; CNT4 = ITEM_SHAPES+1[X]

sta CNT4

iny                     ; y

lda (PTR),y             ; ITEMS[Y] = (PTR)[Y]

sta ITEMS,y

tax                     ; line address

lda LINE_LO,x           ; CNT0 = LINE_LO[X]

sta CNT0

lda LINE_HI,x           ; CNT1 = LINE_HI[X]

sta CNT1

iny                     ; x, rounded to a byte

lda (PTR),y

and #$FC

sta ITEMS,y

iny

sty PLOT_X

jsr ADD_X_BYTES         ; + x/4

jsr BLIT_SHAPE          ; copy the picture

ldy PLOT_X              ; if PLOT_X <> 0 goto SI_LOOP

bne SI_LOOP

;-----------------------------------------------------------------------------

;  BLIT_SHAPE - copy the item picture at (CNT3) to the bitmap at (CNT0), byte by byte.

;-----------------------------------------------------------------------------

BLIT_SHAPE:     ldy #$00                ; width in pixels -> bytes

lda (CNT3),y

jsr WIDTH_BYTES         ; PLOT_Y = number of bytes covering A pixels = (A - 1) / 4 + 1

iny                     ; height

lda (CNT3),y            ; CNT2 = (CNT3)[Y]

sta CNT2

iny                     ; skip the points byte

iny

BL_LINE:        ldx PLOT_Y

lda #$00                ; PIX_PTR+1 = 0

sta PIX_PTR+1

BL_BYTE:        lda (CNT3),y

sty PIX_PTR

ldy PIX_PTR+1

sta (CNT0),y

inc PIX_PTR+1

ldy PIX_PTR

iny

dex                     ; loop until X = 0

bne BL_BYTE

jsr NEXT_LINE_PTR       ; next line

bne BL_LINE

BL_RTS:         rts

;-----------------------------------------------------------------------------

;  ITEM_ADDR - CNT0/CNT1 = bitmap address of item X (offset into ITEMS).

;  ADD_X_BYTES adds A/4 to CNT0.

;-----------------------------------------------------------------------------

ITEM_ADDR:      lda ITEMS+1,x           ; line

tay

lda LINE_LO,y           ; CNT0 = LINE_LO[Y]

sta CNT0

lda LINE_HI,y           ; CNT1 = LINE_HI[Y]

sta CNT1

lda ITEMS+2,x           ; + x/4

ADD_X_BYTES:    lsr a

lsr a

clc

adc CNT0

bcc AXB_STORE

inc CNT1

AXB_STORE:      sta CNT0

rts

;-----------------------------------------------------------------------------

;  WIDTH_BYTES - PLOT_Y = number of bytes covering A pixels = (A - 1) / 4 + 1.

;-----------------------------------------------------------------------------

WIDTH_BYTES:    sec

sbc #$01

lsr a

lsr a

clc

adc #$01

sta PLOT_Y

rts

;-----------------------------------------------------------------------------

;  NEXT_LINE_PTR - CNT0/CNT1 += 40 (one bitmap line), CNT2 -= 1.

;-----------------------------------------------------------------------------

NEXT_LINE_PTR:  lda #$28                ; CNT0 = $28 + CNT0

clc

adc CNT0

sta CNT0

bcc NLP_COUNT

inc CNT1

NLP_COUNT:      dec CNT2

rts

;-----------------------------------------------------------------------------

;  UPDATE_ITEMS - every frame (from GAME_VBI): pick up items Bob touches.

;  ITEMS[n+0]: bit 7 set = item present. When Bob touches it, the item is replaced by its

;  points value ("100" .. "500", "1100" for the BF logo), the score goes up by

;  points x 100, mutants become edible (ITEM_GOT), and the byte becomes a countdown from

;  $55: while it runs a falling whistle plays on voice 4; at 1 the points are erased.

;  On station 10 the points of a TNT bundle are also added to the charge (1-3 tons).

;  On station 5 the goblet (shape $21) is poisoned: touching it kills Bob.

;-----------------------------------------------------------------------------

UPDATE_ITEMS:   ldx #$80                ; a meaningless sequence: compares a ROM byte with itself minus 5 and

;   ignores the

stx PTR+1               ; result; the NOPs suggest a check that was patched out

lda (PTR),y

sbc #$05

nop

nop

cmp (PTR),y             ; if A <> (PTR)[Y] goto UI_START

bne UI_START

nop

nop

UI_START:       ldx #$00

UI_LOOP:        lda ITEMS,x             ; 0 = end of list

bne UI_ENTRY

rts

UI_ENTRY:       bmi UI_PRESENT          ; present?

sec                     ; counting down after a pick-up

sbc #$01

beq UI_NEXT             ; (stays at 1 = gone)

sta ITEMS,x

cmp #$01                ; at 1: erase the points

bne UI_WHISTLE

jmp UI_ERASE_POINTS

UI_WHISTLE:     ldy SECTIONS            ; the whistle (not after the station is cleared)

beq UI_NEXT

ldy #$00

sec                     ; only during the first 31 frames

sbc #$36

bcc UI_NEXT

beq UI_VOL

asl a                   ; pitch falls as the count goes down

asl a

asl a

sta CNT1

lda #$FF                ; AUDF4 = $FF - CNT1

sec

sbc CNT1

sta (pAUDF4),y          ; AUDF4 = A

ldy #$A8                ; pure tone, volume 8

UI_VOL:         tya

ldy #$00

sta (pAUDC4),y          ; AUDC4 = A

UI_NEXT:        inx                     ; next item

inx

inx

bne UI_LOOP

UI_PRESENT:     ldy DYING               ; not while Bob is dying or in the transporter

bne UI_NEXT

ldy SPECIAL             ; if SPECIAL <> 0 goto UI_NEXT

bne UI_NEXT

and #$7F                ; shape

jsr SHAPE_ADDR          ; CNT3/CNT4 = address of item shape A

ldy #$00                ; horizontal: centre of the item within 8 of Bob's x?

lda (CNT3),y

lsr a

clc

adc ITEMS+2,x

adc #$30

sec

sbc BOB_X

cmp #$08                ; if A >= 8 goto UI_NEXT

bcs UI_NEXT

iny                     ; vertical: does Bob's body overlap the item's lines?

lda (CNT3),y

clc

adc ITEMS+1,x

adc #$20

sta CNT1

lda BOB_Y

adc #$20

cmp CNT1                ; if A >= CNT1 goto UI_NEXT

bcs UI_NEXT

sta CNT1

lda ITEMS+1,x

adc #$0C

cmp CNT1                ; if A >= CNT1 goto UI_NEXT

bcs UI_NEXT

lda ITEMS,x             ; touched: the goblet on station 5 kills

cmp #$A1                ; if A <> $A1 goto UI_COLLECT

bne UI_COLLECT

lda STATION

cmp #$05                ; if A <> 5 goto UI_COLLECT

bne UI_COLLECT

sta KILLED              ; KILLED = 5

UI_COLLECT:     lda #$55                ; start the countdown

sta ITEMS,x

dey                     ; width, height and points of the shape

lda (CNT3),y

pha

iny

lda (CNT3),y

pha

iny

lda (CNT3),y

pha

jsr SHAPE_ADDR          ; the points value is itself a shape number: draw it over the item

jsr ITEM_ADDR           ; CNT0/CNT1 = bitmap address of item X (offset into ITEMS)

jsr BLIT_SHAPE          ; copy the item picture at (CNT3) to the bitmap at (CNT0), byte by byte

lda STATION             ; station 10: TNT bundles add their tons

cmp #$0A                ; if A <> $0A goto UI_SCORE

bne UI_SCORE

pla

pha

clc

adc TNT

sta TNT

UI_SCORE:       pla                     ; score += points * 100 (digit 4 = hundreds)

ldx #$04

stx ITEM_GOT            ; and the mutants turn edible

jsr ADD_SCORE           ; add A to score digit X (1 = 100,000s

pla                     ; height

tay

pla                     ; width

cpy #$06                ; taller than the 5-line number? erase the rest of the item below it

bcc CB_RTS

jsr WIDTH_BYTES         ; PLOT_Y = number of bytes covering A pixels = (A - 1) / 4 + 1

tya

sec

sbc #$05

CLEAR_BLOCK:    sta CNT2                ; lines

CB_LINE:        ldx PLOT_Y

ldy #$00

tya

CB_BYTE:        sta (CNT0),y

iny

dex                     ; loop until X = 0

bne CB_BYTE

jsr NEXT_LINE_PTR       ; CNT0/CNT1 += 40 (one bitmap line), CNT2 -= 1

bne CB_LINE

CB_RTS:         rts

UI_ERASE_POINTS:

txa                     ; erase the 3-byte, 5-line points number

pha

jsr ITEM_ADDR           ; CNT0/CNT1 = bitmap address of item X (offset into ITEMS)

lda #$03                ; PLOT_Y = 3

sta PLOT_Y

lda #$05

jsr CLEAR_BLOCK         ; clear CNT2 lines of PLOT_Y bytes at (CNT0)

pla

tax

lda SECTIONS            ; whistle off (unless the station is over: the bonus count owns voice 4)

beq UI_ERASE_DONE

lda #$00

tay

sta (pAUDC4),y          ; AUDC4 = A

UI_ERASE_DONE:  jmp UI_NEXT

;-----------------------------------------------------------------------------

;  SHAPE_ADDR - CNT3/CNT4 = address of item shape A.

;-----------------------------------------------------------------------------

SHAPE_ADDR:     asl a

tay

lda ITEM_SHAPES,y       ; CNT3 = ITEM_SHAPES[Y]

sta CNT3

lda ITEM_SHAPES+1,y     ; CNT4 = ITEM_SHAPES+1[Y]

sta CNT4

rts

;-----------------------------------------------------------------------------

;  PREP_VBI - vertical blank of the "MINER 1 / PREPARE FOR / STATION 1 / ZONE 1" screen.

;  Every 30 frames FRAMES counts down (it starts at 6: three seconds). The background

;  flashes red/black with a two-tone siren, and the station line flickers through random

;  colours. The station is drawn behind this screen meanwhile.

;-----------------------------------------------------------------------------

PREP_VBI:       lda #$22                ; DMACTL: normal playfield, display list on, no P/M

sta DMACTL

lda #$09                ; display list at $0978 (built by PREPARE_SCREEN)

sta DLISTH

lda #$78                ; DLISTL = $78

sta DLISTL

ldx #$04                ; fine scroll used to centre some lines

stx HSCROL

dec BCD_100             ; every 30 frames ...

bne PV_COLOURS

ldx #$1E

stx BCD_100

dec FRAMES              ; ... count down

PV_COLOURS:     lda FRAMES              ; odd or even second?

and #$01                ; test bits $01

beq PV_EVEN

ldx #$42                ; red background, siren note $96, zone line black

ldy #$00

lda #$96

bne PV_SET

PV_EVEN:        ldx #$00                ; black background, siren note $4B, zone line orange

ldy #$48

lda #$4B

PV_SET:         pha

tya

ldy #$00

sta (pCOLPF3),y         ; COLPF3: "ZONE n"

txa

sta (pCOLBK),y          ; COLBK

pla

sta (pAUDF1),y          ; siren

lda #$AF                ; AUDC1 = $AF

sta (pAUDC1),y          ; AUDC1 = A

lda #$88                ; COLPF0: "MINER n"

sta (pCOLPF0),y         ; COLPF0 = A

lda #$C4                ; COLPF1: "PREPARE FOR"

sta (pCOLPF1),y         ; COLPF1 = A

lda (pSKRES),y          ; COLPF2: "STATION n", a random colour every frame

sta (pCOLPF2),y         ; COLPF2 = A

jmp NMI_EXIT            ; common tail of every interrupt handler

;-----------------------------------------------------------------------------

;  NAME_ENTRY - "MINER 1 / great score! / enter your name. / >>        <<"

;  The player types up to 8 characters (400/800 keyboard), or on the 5200 picks letters

;  with the stick and enters them with the fire button. RETURN / START / the 5200's lower

;  button ends entry, and so does a time limit (60 s; 120 s on the 5200).

;  Each character goes both into the name on screen ($095E) and straight into the

;  high-score line at HISCORES + CNT0 + 2. The whole thing runs in NAME_VBI; this routine

;  sets up and waits for CNT4 to become non-zero.

;-----------------------------------------------------------------------------

NAME_ENTRY:     ldy #$54                ; copy the screen text (84 bytes from NAME_TEXT)

NE_TEXT:        lda NAME_TEXT-1,y       ; $0917[Y] = NAME_TEXT-1[Y]

sta $0917,y

dey                     ; loop until Y = 0

bne NE_TEXT

tya                     ; clear the missiles ($0300-$03FF): they form the cursor

NE_CLEAR:       sta PM_MISSILES,y

iny

bne NE_CLEAR

lda IS5200              ; 5200: copy its ROM font to RAM at $1C00 so one glyph can be changed

beq NE_PLAYER

sty PLOT_Y

sty CNT3

lda #$F8                ; CNT2 = $F8

sta CNT2

lda #$1C                ; CNT4 = $1C

sta CNT4

sta CHBASE

ldx #$02                ; 512 bytes = the first 64 characters

NE_FONT:        lda (PLOT_Y),y          ; (CNT3)[Y] = (PLOT_Y)[Y]

sta (CNT3),y

iny

bne NE_FONT

inc CNT2

inc CNT4

dex                     ; loop until X = 0

bne NE_FONT

ldx #$07                ; character $3E (the "delete" choice) gets the glyph at $FAF0

NE_GLYPH:       lda $FAF0,x             ; $1DF0[X] = $FAF0[X]

sta $1DF0,x

dex

bpl NE_GLYPH

NE_PLAYER:      lda PLAYER              ; "MINER n": which player's score is this?

ldx PTR                 ; if PTR == 0 goto NE_PLAYER_SET

beq NE_PLAYER_SET

lda #$01

sec

sbc PLAYER

NE_PLAYER_SET:  clc                     ; digit with colour bits ($91 = '1' in colour 2)

adc #$91

sta $0926

ldx #$08                ; X = 8: (pAUDF4,x) is (pSKRES) = RANDOM: a random hue for the rainbow

lda (pAUDF4,x)

and #$F0

ora #$0A

sta PLOT_COLOR

stx PTR+1

stx PLOT_Y

lda #$00                ; name length 0, no repeat, not finished

sta CNT1

sta CNT2

sta CNT4

sta COLBK_V

sta COLPF0_V

sta NAME_RPT

jsr SILENCE_234         ; silence voices 2-4

ldx #$21                ; 5200 letter starts at 'A'

stx LASTCHAR

ldx #$FF                ; cursor: two solid missile rows

stx $03CF

stx $03D0

stx KEYCODE             ; no key

ldx #$C2

stx CNT3

lda #$02                ; opening beep sweep

sta PLOT_X

sta START_DLY           ; START must be released first

tax                     ; X = 2: (pCONSOL,x) = (pAUDF1): AUDF1 = 2

sta (pCONSOL,x)

lda #$8F                ; and (pAUDF1,x) = (pAUDC1): pure tone, volume 15

sta (pAUDF1,x)

lda #$FF                ; no key held

sta COLPF1_V

lda #$3C                ; one second ...

sta NAME_DLY

sta COLPF2_V

ldx IS5200              ; ... and 60 of them (120 on the 5200) before giving up

beq NE_LIMIT

asl a

NE_LIMIT:       sta BLINK_TMR

ldx #$A9                ; VBI = NAME_VBI

ldy #$A9

jsr SET_VBI             ; install a vertical-blank interrupt handler

ldx #$AB                ; DLI = DLI_NAME

ldy #$78

jsr SET_DLI             ; install a display-list-interrupt handler

lda #$C0                ; NMIEN = $C0

sta NMIEN

NE_WAIT:        lda CNT4                ; wait for NAME_VBI to finish

beq NE_WAIT

lda #$00                ; NMIEN = 0

sta NMIEN

NE_RTS:         rts

;-----------------------------------------------------------------------------

;  NAME_DLIST - display list of the name-entry screen, also copied to $0978 for the

;  "prepare" screen: 7 x 8 blank lines (the 5th requests the DLI), then four mode 7 text

;  lines with blank lines between them, starting at $0918, then JVB $0978.

;-----------------------------------------------------------------------------

NAME_DLIST:     .byte $70               ; 8 blank lines

.byte $70               ; 8 blank lines

.byte $70               ; 8 blank lines

.byte $70               ; 8 blank lines

.byte $F0               ; 8 blank lines +DLI

.byte $70               ; 8 blank lines

.byte $70               ; 8 blank lines

.byte $57,$18,$09       ; mode 7 LMS $0918 +HSCROL

.byte $70               ; 8 blank lines

.byte $70               ; 8 blank lines

.byte $70               ; 8 blank lines

.byte $70               ; 8 blank lines

.byte $70               ; 8 blank lines

.byte $70               ; 8 blank lines

.byte $07               ; mode 7

.byte $70               ; 8 blank lines

.byte $70               ; 8 blank lines

.byte $07               ; mode 7

.byte $70               ; 8 blank lines

.byte $70               ; 8 blank lines

.byte $07               ; mode 7

.byte $41,$78,$09       ; JVB $0978

;-----------------------------------------------------------------------------

;  NAME_VBI - runs the whole name entry, once per frame.

;-----------------------------------------------------------------------------

NAME_VBI:       jsr POLL_KEYS           ; called at the start of every VBI handler

lda #$A9                ; display list

sta DLISTH

lda #$8F                ; DLISTL = $8F

sta DLISTL

lda #$36                ; DMACTL: normal playfield, missiles on, single-line P/M

sta DMACTL

jsr READ_POTS           ; 5200: read the analog stick

jsr READ_STICK          ; stick, 400/800 or 5200 format

ldy #$FF                ; 5200 letter choice: left = -1 ...

lsr a

lsr a

lsr a

bcc NV_REPEAT

ldy #$01                ; ... right = +1

lsr a

bcc NV_REPEAT

lda #$00                ; centred: reset the auto-repeat

sta NAME_RPT

beq NV_COLOURS

NV_REPEAT:      inc NAME_RPT            ; first frame moves at once, then after 40 frames every 8

lda NAME_RPT

cmp #$01                ; if A == 1 goto NV_MOVE

beq NV_MOVE

cmp #$28                ; if A < $28 goto NV_COLOURS

bcc NV_COLOURS

lda #$20                ; NAME_RPT = $20

sta NAME_RPT

NV_MOVE:        tya                     ; new letter, kept within $00-$3E

clc

adc LASTCHAR

bmi NV_COLOURS

cmp #$3F                ; if A >= $3F goto NV_COLOURS

bcs NV_COLOURS

sta LASTCHAR

NV_COLOURS:     lda #$00                ; black background, playfield colour 2 black

tay

sta (pCOLBK),y          ; COLBK = A

sta (pCOLPF2),y         ; COLPF2 = A

lda #$01                ; missile DMA on (the cursor)

sta (pGRACTL),y         ; GRACTL = A

lda #$46                ; text colours

sta (pCOLPF0),y         ; COLPF0 = A

lda #$86                ; cursor colour (missiles use COLPF3 in fifth-player mode)

sta (pCOLPF3),y         ; COLPF3 = A

ldx COLBK_V             ; key click: switch off after 5 frames

beq NV_RAINBOW

dec COLBK_V             ; COLBK_V -= 1, until 0

bne NV_RAINBOW

dex

txa

sta (pAUDC2,x)          ; X = 0 here: AUDC2 = 0

NV_RAINBOW:     dec PTR+1               ; every 15 frames move the rainbow on one hue (skipping grey)

bne NV_KEYS

ldx #$03

stx PTR+1

dec CNT3                ; CNT3 -= 1, until 0

bne NV_KEYS

ldx #$05

stx CNT3

lda PLOT_COLOR

clc

adc #$10

cmp #$0A                ; if A <> $0A goto NV_RAINBOW_SET

bne NV_RAINBOW_SET

lda #$1A

NV_RAINBOW_SET: sta PLOT_COLOR

NV_KEYS:        jsr CHECK_START_ANY     ; START (or the button on the 400/800) finishes

bcs NV_FINISH

ldy #$00

lda IS5200              ; which machine?

beq NV_800_KEY

lda (pSKCTL),y          ; 5200: lower button (SKSTAT bit 3) finishes, once NAME_DLY has run out

and #$08                ; test bits $08

bne NV_5200_FIRE

lda NAME_DLY            ; if NAME_DLY == 0 goto NV_FINISH

beq NV_FINISH

lda #$3C                ; NAME_DLY = $3C

sta NAME_DLY

NV_5200_FIRE:   lda (pTRIG0),y          ; 5200: fire button enters the chosen letter

bne NV_RELEASED

ldx #$21                ; (code of the space key, $21, or of DELETE, $34, if the "delete" glyph is

;   chosen)

lda LASTCHAR

cmp #$3E                ; if A <> $3E goto NV_5200_RPT

bne NV_5200_RPT

ldx #$34

NV_5200_RPT:    inc COLPF0_V            ; only on the first frame of the press

lda COLPF0_V

cmp #$01                ; if A <> 1 goto NV_REPEAT_CHK

bne NV_REPEAT_CHK

beq NV_CHAR

NV_800_KEY:     lda KEYCODE             ; 400/800: a new key?

cmp #$FF                ; if A <> $FF goto NV_NEW_KEY

bne NV_NEW_KEY

lda (pSKCTL),y          ; none: is the last key still held down?

and #$04                ; test bits $04

beq NV_HELD

NV_RELEASED:    jmp NV_NO_REPEAT

NV_HELD:        ldx COLPF1_V            ; held: auto-repeat after 40 frames

cpx #$FF                ; if X <> $FF goto NV_REPEAT_KEY

bne NV_REPEAT_KEY

NV_TAIL_J:      jmp NV_CURSOR

NV_REPEAT_KEY:  inc COLPF0_V

lda COLPF0_V

NV_REPEAT_CHK:  cmp #$28                ; if A <> $28 goto NV_TAIL_J

bne NV_TAIL_J

lda #$21                ; COLPF0_V = $21

sta COLPF0_V

bne NV_RETURN

NV_NEW_KEY:     and #$3F                ; new key: remember it

sta COLPF1_V

tax

lda #$FF                ; KEYCODE = $FF

sta KEYCODE

lda #$00                ; COLPF0_V = 0

sta COLPF0_V

NV_RETURN:      cpx #$0C                ; RETURN finishes

bne NV_CHAR

NV_FINISH:      ldx #$0C

jmp NV_SET_DONE

NV_CHAR:        cpx #$34                ; DELETE?

beq NV_DELETE

ldy CNT1                ; name full (8 characters)? low buzz

lda #$FA

cpy #$08                ; if Y == 8 goto NV_BEEP

beq NV_BEEP

lda KEY_TO_SCREEN,x     ; key code -> screen code

beq NV_STORE

cmp #$1C                ; digits and punctuation: with SHIFT use the shifted symbol

bcs NV_STORE

pha

ldy #$00

lda (pSKCTL),y          ; A = SKSTAT

and #$08

tax

pla

dex

inx

bne NV_STORE

tax

lda SHIFT_MAP-1,x

NV_STORE:       ldy CNT1

pha

lda IS5200              ; on the 5200 the character is the chosen letter

beq NV_STORE2

pla

lda LASTCHAR

pha

NV_STORE2:      pla                     ; into the name on screen

sta $095E,y

inc CNT1

pha                     ; cursor visible

lda #$FF                ; CNT2 = $FF

sta CNT2

lda #$0A                ; PLOT_Y = $0A

sta PLOT_Y

pla                     ; and into the high-score line

ldy CNT0

sta HISCORES+2,y

inc CNT0

NV_CLICK:       lda #$32                ; click

NV_BEEP:        ldy #$00

sta (pAUDF2),y          ; AUDF2 = A

lda #$05                ; COLBK_V = 5

sta COLBK_V

lda #$AA                ; AUDC2 = $AA

sta (pAUDC2),y          ; AUDC2 = A

bne NV_CURSOR

NV_DELETE:      lda #$FA                ; delete: nothing to delete? buzz

ldx CNT1                ; if CNT1 == 0 goto NV_BEEP

beq NV_BEEP

lda #$00                ; remove the last character from both places

sta $095E,x

dec CNT1

dec CNT0

ldx CNT0

lda #$00                ; HISCORES+2[X] = 0

sta HISCORES+2,x

beq NV_CLICK

NV_NO_REPEAT:   lda #$00                ; COLPF0_V = 0

sta COLPF0_V

beq NV_CURSOR

NV_SET_DONE:    stx CNT4                ; finished

NV_CURSOR:      dec PLOT_Y              ; cursor blinks every 15 frames

bne NV_CURSOR_POS

lda #$0F                ; PLOT_Y = $0F

sta PLOT_Y

lda CNT2

eor #$FF

sta CNT2

NV_CURSOR_POS:  lda CNT1                ; four missiles side by side under the next character

asl a

asl a

asl a

adc #$61

and CNT2

ldx #$06

clc

NV_CURSOR_LOOP: sta (pHPOSM0,x)         ; X = 6,4,2,0: HPOSM3 ... HPOSM0

adc #$02

dex

dex

bpl NV_CURSOR_LOOP

ldy #$00                ; the character at the cursor: blank, or on the 5200 the letter being chosen

tya

ldx IS5200              ; if IS5200 == 0 goto NV_PREVIEW

beq NV_PREVIEW

lda LASTCHAR

NV_PREVIEW:     ldx CNT1

sta $095E,x

lda PLOT_X              ; opening beep: rising sweep until it wraps to 0

beq NV_TIMEOUT

inc PLOT_X

inc PLOT_X

lda PLOT_X              ; AUDF1 = PLOT_X

sta (pAUDF1),y          ; AUDF1 = A

bne NV_TIMEOUT

sta (pAUDC1),y          ; AUDC1 = A

NV_TIMEOUT:     dec COLPF2_V            ; time limit

bne NV_EXIT

ldx #$3C

stx COLPF2_V

dec BLINK_TMR           ; BLINK_TMR -= 1, until 0

bne NV_EXIT

stx CNT4

NV_EXIT:        lda NAME_DLY            ; if NAME_DLY == 0 goto NV_EXIT2

beq NV_EXIT2

dec NAME_DLY

NV_EXIT2:       jmp NMI_EXIT            ; common tail of every interrupt handler

;-----------------------------------------------------------------------------

;  BLANK_VBI - used while the credits screen (station 11) is being built: screen, players

;  and sound off, FRAMES held at 0.

;-----------------------------------------------------------------------------

BLANK_VBI:      .byte $20,$18,$92,$98,$8D,$00,$D4,$91,$4A,$91,$44,$91,$52,$85,$9D,$F0

.byte $E6

;-----------------------------------------------------------------------------

;  DLI_NAME - the name-entry screen's background: 170 scan lines of colour bands, the

;  hue moving on every 5 lines, while the text brightness ripples line by line.

;-----------------------------------------------------------------------------

DLI_NAME:       pha

txa

pha

tya

pha

ldy #$AA                ; 170 lines

sty LINECNT

ldy #$00

ldx CNT3

lda PLOT_COLOR          ; start colour (moves every 15 frames)

sta (pCOLPF1),y         ; COLPF1 = A

DN_LINE:        sta WSYNC

sta (pCOLBK),y          ; background colour for this line

pha

lda LINECNT             ; text brightness

clc

adc #$03

and #$0F

sta (pCOLPF1),y         ; COLPF1 = A

pla

dex                     ; every 5 lines the next hue (skipping grey)

bne DN_NEXT

ldx #$05

adc #$10

cmp #$0A                ; if A <> $0A goto DN_NEXT

bne DN_NEXT

lda #$1A

DN_NEXT:        dec LINECNT             ; LINECNT -= 1, until 0

bne DN_LINE

DN_END:         sty WSYNC

tya                     ; background back to black

sta (pCOLBK),y          ; COLBK = A

jmp NMI_EXIT            ; common tail of every interrupt handler

;-----------------------------------------------------------------------------

;  DLI_RAINBOW - the end of the credits: the only interrupt left running (there is no

;  VBI), so it also reads the keys and advances FRAMES. 180 lines of colour 1 change

;  hue every 16 lines, so the credits text is striped with a scrolling rainbow.

;-----------------------------------------------------------------------------

DLI_RAINBOW:    pha

txa

pha

tya

pha

jsr POLL_KEYS           ; called at the start of every VBI handler

lda PLOT_COLOR

dec CNT3                ; every 16 frames the starting hue moves on

bne DR_GO

ldx #$10

stx CNT3

clc

adc #$10

cmp #$10                ; if A >= $10 goto DR_START

bcs DR_START

adc #$10

DR_START:       sta PLOT_COLOR

DR_GO:          ldx CNT3

inc FRAMES              ; a frame has passed

ldy #$B4                ; 180 lines

sty LINECNT

ldy #$00

DR_LINE:        sta WSYNC

sta (pCOLPF0),y         ; COLPF0 = A

dex                     ; loop until X = 0

bne DR_NEXT

ldx #$10

clc

adc #$10

cmp #$10                ; if A >= $10 goto DR_NEXT

bcs DR_NEXT

adc #$10

DR_NEXT:        dec LINECNT             ; LINECNT -= 1, until 0

bne DR_LINE

beq DN_END

;-----------------------------------------------------------------------------

;  CREDITS_TEXT - (characters, x, y) for each line of the credits, drawn from TEXT_POOL

;  starting at "MINER 2049er" (the first line double height). 0 ends the list.

;-----------------------------------------------------------------------------

CREDITS_TEXT:   .byte $0C,$24,$10

.byte $02,$48,$2D

.byte $0A,$2C,$35

.byte $08,$34,$55

.byte $0C,$24,$5D

.byte $0D,$02,$80

.byte $07,$1A,$88

.byte $11,$10,$9D

.byte $11,$10,$A5

.byte $11,$10,$AD

.byte $0E,$10,$B5

.byte $00

;-----------------------------------------------------------------------------

;  HIDE_PLAYERS - zero GTIA registers 0-12: positions of players and missiles, sizes.

;-----------------------------------------------------------------------------

HIDE_PLAYERS:   ldx #$18

lda #$00

HP_LOOP:        sta (pHPOSP0,x)

dex

dex

bpl HP_LOOP

rts

;-----------------------------------------------------------------------------

;  PREPARE_SCREEN - show "MINER n / PREPARE FOR / STATION n / ZONE n" (built in RAM from

;  the text at PREP_TEXT and the display list NAME_DLIST), start its VBI and a 3-second

;  countdown, clear P/M memory and build the (still invisible) game screen.

;  The credits screen instead gets BLANK_VBI.

;-----------------------------------------------------------------------------

PREPARE_SCREEN: jsr HIDE_PLAYERS        ; zero GTIA registers 0-12

lda STATION

cmp #$0B                ; if A <> $0B goto PS_TEXT

bne PS_TEXT

ldx #$AB

ldy #$67

jmp PS_START

PS_TEXT:        ldy #$30                ; "MINER 1" / "Prepare for" text

PS_TEXT_LOOP:   lda PREP_TEXT-2,y       ; $0917[Y] = PREP_TEXT-2[Y]

sta $0917,y

dey                     ; loop until Y = 0

bne PS_TEXT_LOOP

ldy #$1A                ; copy the display list to $0978

PS_DL_LOOP:     lda NE_RTS,y            ; $0977[Y] = NE_RTS[Y]

sta $0977,y

dey                     ; loop until Y = 0

bne PS_DL_LOOP

ldx #$17                ; and adjust it: the station line gets fine scrolling (centring)

stx $0988

lda #$70                ; $097C = $70

sta $097C

lda PLAYER              ; player number

clc

adc #$11

sta $0926

lda STATION             ; two-digit station?

cmp #$0A                ; if A < $0A goto PS_ONE_DIGIT

bcc PS_ONE_DIGIT

ldy #$2C                ; "STATION 10" layout

PS_TEXT10:      lda PREP_TEXT+48,y      ; $0947[Y] = PREP_TEXT+48[Y]

sta $0947,y

dey                     ; loop until Y = 0

bne PS_TEXT10

lda STATION             ; the units digit, in colour 2

clc

adc #$86

sta $0956

ldx #$09                ; zone digits will go at $095C

ldy #$5C

jmp PS_ZONE

PS_ONE_DIGIT:   stx $098B               ; one-digit station: centre that line too

ldy #$30

PS_TEXT1:       lda PREP_TEXT+46,y      ; $0947[Y] = PREP_TEXT+46[Y]

sta $0947,y

dey                     ; loop until Y = 0

bne PS_TEXT1

lda STATION             ; the digit, in colour 2

clc

adc #$90

sta $0957

ldx #$09                ; zone digits will go at $0960

ldy #$60

PS_ZONE:        stx PTR+1

sty PTR

ldx #$D0                ; zone in decimal, digits in colour 3 ($D0 = '0')

stx BCD_100

stx SEC_TMR

lda ZONE

PS_HUNDREDS:    cmp #$64                ; if A < $64 goto PS_TENS

bcc PS_TENS

sbc #$64

inc BCD_100             ; BCD_100 += 1, until 0

bne PS_HUNDREDS

PS_TENS:        cmp #$0A                ; if A < $0A goto PS_UNITS

bcc PS_UNITS

sbc #$0A

inc SEC_TMR             ; SEC_TMR += 1, until 0

bne PS_TENS

PS_UNITS:       adc #$D0

sta BONUS_TMR

ldx #$04                ; "Zone" (4 characters)

stx MOVE_OK

ldx #$00

ldy #$06                ; how many digits, and where to start (centred)

lda ZONE

cmp #$64                ; if A >= $64 goto PS_WRITE

bcs PS_WRITE

iny

ldx #$02

cmp #$0A                ; if A < $0A goto PS_WRITE

bcc PS_WRITE

iny

dex

lda #$17                ; $098E = $17

sta $098E

PS_WRITE:       stx KILLED

ldx #$00

PS_ZONE_WORD:   lda ZONE_TEXT,x         ; (PTR)[Y] = ZONE_TEXT[X]

sta (PTR),y

iny

inx

dec MOVE_OK             ; MOVE_OK -= 1, until 0

bne PS_ZONE_WORD

iny                     ; a space

ldx KILLED

PS_ZONE_DIGIT:  lda BCD_100,x           ; (PTR)[Y] = BCD_100[X]

sta (PTR),y

inx

iny

cpx #$03                ; if X <> 3 goto PS_ZONE_DIGIT

bne PS_ZONE_DIGIT

ldx #$1E                ; countdown step: 30 frames

stx BCD_100

ldx #$A8                ; VBI = PREP_VBI

ldy #$81

PS_START:       jsr SET_VBI             ; install a vertical-blank interrupt handler

ldx #$06                ; 6 x 30 frames = 3 seconds

stx FRAMES

lda #$40                ; VBI only

sta NMIEN

jsr SILENCE             ; silence

lda IS5200              ; 400/800: PIA port A all inputs (joysticks)

bne PS_PM

ldx #$38                ; PACTL bit 2 = 0: PORTA addresses the direction register

stx PACTL

sta PORTA               ; all inputs

lda #$3C                ; back to the data register

sta PACTL

PS_PM:          ldx #$03                ; PMBASE = 3; in single-line mode ANTIC ignores the low 3 bits, so P/M

;   memory

stx PMBASE              ; starts at $0000: missiles $0300, players $0400 $0500 $0600 $0700

stx PTR+1               ; clear $0300-$07FF

ldx #$00

stx PTR

ldx #$FB

ldy #$00

jsr ZERO_MEM            ; zero memory from (PTR) for -(X:Y) bytes

jmp BUILD_DLIST         ; build the game display list and clear the screen

;-----------------------------------------------------------------------------

;  DRAW_CREDITS - draw the credits text from CREDITS_TEXT in colour 1.

;-----------------------------------------------------------------------------

DRAW_CREDITS:   lda #$00                ; TXT_COLOR = 0

sta TXT_COLOR

ldx #$16

stx TXT_IDX

ldx #$02                ; the first line double height

DCR_LINE:       stx TXT_VSCALE

tax

lda CREDITS_TEXT,x      ; characters (0 = end)

bne DCR_DRAW

rts

DCR_DRAW:       sta TXT_LEN

lda CREDITS_TEXT+1,x    ; x

sta TXT_X

lda CREDITS_TEXT+2,x    ; y

tay

txa

clc

adc #$03

pha

jsr DRAW_STRING_Y       ; draw A characters of TEXT_POOL at TXT_X, line Y

pla

ldx #$01                ; the rest normal height

bne DCR_LINE

;-----------------------------------------------------------------------------

;  START_STATION - choose the station's colours, reset the per-turn variables, wait for

;  the prepare countdown, then switch on the game interrupts and let Bob go.

;

;  Colours: three random hues for pixel colours 1-3, all different, and three random

;  brightnesses from {4, 6, 8}, also all different. Different brightness keeps items,

;  ladders and girders distinguishable on a black-and-white TV.

;-----------------------------------------------------------------------------

START_STATION:  ldx #$03

stx CNT7

SS_CLEAR:       lda COLBK_V,x           ; keep only the hues of the old colours (so they can be compared)

and #$F0

sta COLBK_V,x

dex                     ; loop until X = 0

bne SS_CLEAR

SS_HUE:         ldy #$00                ; random hue ...

lda (pSKRES),y          ; A = RANDOM

and #$F0

sta MOVE_A

ldx #$03

SS_HUE_CHK:     lda COLBK_V,x           ; ... not used by another colour

and #$F0

cmp MOVE_A              ; if A == MOVE_A goto SS_HUE

beq SS_HUE

dex                     ; loop until X = 0

bne SS_HUE_CHK

SS_LUM:         lda (pSKRES),y          ; random brightness 4, 6 or 8 ...

and #$06                ; test bits $06

beq SS_LUM

clc

adc #$02

sta MOVE_B

ldx #$03

SS_LUM_CHK:     lda COLBK_V,x           ; ... not used by another colour

and #$0F

cmp MOVE_B              ; if A == MOVE_B goto SS_LUM

beq SS_LUM

dex                     ; loop until X = 0

bne SS_LUM_CHK

lda MOVE_A

ora MOVE_B

ldy CNT7

sta a:$008A,y

dec CNT7                ; CNT7 -= 1, until 0

bne SS_HUE

lda STATION             ; credits: colour 1 black (the text starts hidden) and draw the text

cmp #$0B                ; if A <> $0B goto SS_PRIOR

bne SS_PRIOR

ldx #$00

stx COLPF0_V

jsr DRAW_CREDITS        ; draw the credits text from CREDITS_TEXT in colour 1

SS_PRIOR:       lda #$31                ; X = 8: (pCOLPF1,x) = (pPRIOR): PRIOR = $31 = players in front, fifth

;   player

ldx #$08                ; (missiles take COLPF3), multicolour players

sta (pCOLPF1,x)

lda STATION

cmp #$0B                ; if A == $0B goto SS_VARS

beq SS_VARS

lda #$FF                ; missile rows $20-$27: the box behind the bonus on the status line

SS_BOX:         sta PM_MISSILES+31,x

dex                     ; loop until X = 0

bne SS_BOX

SS_VARS:        lda #$1E                ; status line steady for 30 frames

sta BLINK_TMR

lda #$00                ; clear START_DLY .. HIT_P3

ldx #$0A

SS_CLR:         sta START_DLY,x

dex                     ; loop until X = 0

bne SS_CLR

tax                     ; per-turn state = 0

stx ITEM_GOT

stx CANNON_DONE

stx CANNON_ST

stx TNT

stx LIFT_DLY

stx LIFT_ON

stx KILLED

stx BLINK_ON

stx CHACTL

stx SND3_VOL

stx FALL_LEN

stx SPECIAL

stx BOB_FALL

stx SLIDE_LEN

stx DYING

stx COLBK_V

inx                     ; = 1

stx ON_LIFT

stx TICK_SND

stx BOB_FRAME

stx WALK_STEP

inx                     ; = 2

stx ALIVE

stx PAUSE_DLY

ldx #$0A

stx JUMP_LOCK

ldx #$3C

stx SEC_TMR

ldx #$B8                ; the end-of-station wipe starts at display-list line 184

stx WIPE_LINE

ldy #$01                ; first demo input lasts this long

lda (DEMO_PTR),y        ; DEMO_CNT = (DEMO_PTR)[Y]

sta DEMO_CNT

jsr BOB_COLOURS         ; Bob's colours

jsr DRAW_STATUS         ; status line

SS_WAIT:        lda FRAMES              ; wait for the prepare countdown

bne SS_WAIT

sta MOVE_OK             ; Bob may not move yet

sta NMIEN               ; NMIs off while the vectors change

ldx #$B7                ; DLI = DLI_MUTANTS

ldy #$0E

jsr SET_DLI             ; install a display-list-interrupt handler

ldy #$B0                ; VBI = GAME_VBI

ldx #$BB

jsr SET_VBI             ; install a vertical-blank interrupt handler

ldy #$00                ; clear collisions

sta (pHITCLR),y         ; HITCLR = A

ldx #$C0                ; DLI + VBI on

stx NMIEN

ldx #$3C                ; one second of quiet (GAME_VBI counts FRAMES up)

SS_PAUSE:       jsr SILENCE             ; all four voices off (SILENCE_234 leaves voice 1 alone; A must be 0)

cpx FRAMES              ; if X <> FRAMES goto SS_PAUSE

bne SS_PAUSE

lda #$07                ; go!

sta MOVE_OK

ldx LIFE_USED           ; take a life at the start of the turn (once)

bne SS_RTS

inc LIFE_USED

dec LIVES

SS_RTS:         rts

;-----------------------------------------------------------------------------

;  DRAW_STATUS - the status line: player 1's score and lives at the left; in a two-player

;  game player 2's score and lives at the right. The current player's score is written with bit 7 set,

;  so that CHACTL bit 0 (blank such characters) makes it blink.

;  Lives are shown as balls (internal code $54, ATASCII ball).

;-----------------------------------------------------------------------------

DRAW_STATUS:    lda STATION             ; nothing on the credits screen

cmp #$0B                ; if A == $0B goto DS_RTS

beq DS_RTS

lda #$00                ; player 1's score at column 1

ldy #$01

jsr DRAW_SCORE          ; player A's score on the status line from column Y

lda #$00                ; his lives after it, left to right

ldx #$01

jsr DRAW_LIVES          ; lives of player A as balls, from column Y, step X

lda TWO_PLAYER          ; if TWO_PLAYER == 0 goto DS_RTS

beq DS_RTS

lda #$01                ; player 2's score from column 32, right-aligned

ldy #$20

jsr DRAW_SCORE          ; player A's score on the status line from column Y

ldy PLOT_X              ; his lives to the left of it, right to left

dey

lda #$01

ldx #$FF

DRAW_LIVES:     stx PLOT_X

cmp PLAYER              ; lives of this player

beq DLV_CURRENT

ldx P2_LIVES

jmp DLV_START

DLV_CURRENT:    ldx LIVES

DLV_START:      inx                     ; a blank, then one ball per life, then a blank

lda #$00

beq DLV_PUT

DLV_BALL:       lda #$44

clc

adc CNT1

DLV_PUT:        sta STATUS,y

tya

clc

adc PLOT_X

tay

dex                     ; loop until X = 0

bne DLV_BALL

txa

sta STATUS,y

DS_RTS:         rts

;-----------------------------------------------------------------------------

;  DRAW_SCORE - player A's score on the status line from column Y: six digits, leading

;  zeros blank (player 1 left-justified, player 2 right-justified), a comma after the

;  thousands, "0" for no score. Leaves PLOT_X = column of the first character printed.

;-----------------------------------------------------------------------------

DRAW_SCORE:     sta PLOT_Y

cmp PLAYER              ; current player: bit 7 set ($90), blinked off by CHACTL bit 0 ...

beq DSC_CURRENT

ldx #$C5                ; ... the other: normal ($10)

lda #$10

bne DSC_GO

DSC_CURRENT:    lda #$90

ldx #$BB

DSC_GO:         sta CNT1

lda #$06                ; CNT3 = 6

sta CNT3

lda #$00                ; no digit printed yet

sta CNT2

DSC_DIGIT:      lda 0,x                 ; digit X of zero page (X = SCORE or P2_DATA+4)

bne DSC_OUT

bit CNT2

bmi DSC_OUT

lda CNT3                ; always print the last digit

cmp #$01                ; if A == 1 goto DSC_ZERO

beq DSC_ZERO

tya                     ; leading zero: player 2 still moves along

clc

adc PLOT_Y

tay

bne DSC_NEXT

DSC_ZERO:       lda #$00

DSC_OUT:        clc

adc CNT1

sta STATUS,y

lda CNT2                ; first digit printed?

bne DSC_ADV

sty PLOT_X

lda #$FF                ; CNT2 = $FF

sta CNT2

DSC_ADV:        iny

DSC_NEXT:       inx

dec CNT3                ; CNT3 -= 1, goto DS_RTS if 0

beq DS_RTS

lda CNT3                ; after the thousands ...

cmp #$03                ; if A <> 3 goto DSC_DIGIT

bne DSC_DIGIT

lda CNT2                ; ... a comma, if any digit has been printed

beq DSC_NO_COMMA

lda #$FC                ; STATUS[Y] = $FC + CNT1

clc

adc CNT1

sta STATUS,y

iny

bne DSC_DIGIT

DSC_NO_COMMA:   tya

clc

adc PLOT_Y

tay

bne DSC_DIGIT

AS_CARRY:       dex                     ; carry into the next digit up (lost beyond the 100,000s)

beq AS_STEADY

;-----------------------------------------------------------------------------

;  ADD_SCORE - add A to score digit X (1 = 100,000s ... 4 = hundreds, 5 = tens,

;  6 = units), carrying upwards. Awards the one extra life when the score first reaches

;  10,000, and makes the status line hold still for two seconds.

;-----------------------------------------------------------------------------

ADD_SCORE:      ldy #$00

clc

adc SCORE-1,x

AS_DIV:         cmp #$0A                ; if A < $0A goto AS_STORE

bcc AS_STORE

sbc #$0A

iny

jmp AS_DIV

AS_STORE:       sta SCORE-1,x

tya

bne AS_CARRY

ldx XLIFE               ; extra life still to come, and 10,000 reached?

cpx #$0A                ; if X <> $0A goto AS_STEADY

bne AS_STEADY

lda SCORE+1             ; if SCORE+1 == 0 goto AS_STEADY

beq AS_STEADY

dex                     ; yes: start the jingle (XLIFE = 9) and add a life

stx XLIFE

inc LIVES

AS_STEADY:      lda #$78                ; BLINK_TMR = $78

sta BLINK_TMR

lda #$00                ; BLINK_ON = 0

sta BLINK_ON

sta CHACTL

rts

;-----------------------------------------------------------------------------

;  TRY_CLIMB_UP - stick pushed up: is Bob standing at the foot of a ladder?

;  A ladder entry is (height, x, y, style). Bob qualifies if his feet (BOB_Y + $15) are

;  level with the ladder's bottom (y + height) or one line above it, and his x within 2 of the ladder's.

;  He is then lined up with the ladder and starts climbing: frame 7, MOVE_A = lines to

;  the top, MOVE_B = lines above the bottom.

;-----------------------------------------------------------------------------

TRY_CLIMB_UP:   ldy #$00

TCU_LOOP:       lda (LADDERS),y         ; end of the ladder list: no ladder, treat as a plain stick movement

bne TCU_ENTRY

jmp NO_LADDER

TCU_ENTRY:      iny                     ; bottom of the ladder = y + height

iny

clc

adc (LADDERS),y

sec                     ; - feet line

sbc BOB_Y

sec

sbc #$15

cmp #$02                ; within 0-1 lines?

bcs TCU_NEXT

sta TMP9B

dey                     ; x within -2..+2?

lda (LADDERS),y

sec

sbc BOB_X

clc

adc #$02

cmp #$05                ; if A >= 5 goto TCU_NEXT_X

bcs TCU_NEXT_X

lda (LADDERS),y         ; line up with the ladder and step on

sta BOB_X

dec BOB_Y

lda #$07                ; climbing frame

sta BOB_FRAME

dey                     ; MOVE_A = height + 4 - offset = lines to the top

lda (LADDERS),y

adc #$04

sec

sbc TMP9B

sta MOVE_A

ldy #$01                ; MOVE_B = 1

sty MOVE_B

dey

LADDER_ON:      tya                     ; footsteps off

sta (pAUDC3),y          ; AUDC3 = A

jmp SHOW_BOB            ; draw Bob (upside down once he has been fired from the cannon), and on

TCU_NEXT_X:     iny

TCU_NEXT:       iny

iny

jmp TCU_LOOP

;-----------------------------------------------------------------------------

;  TRY_CLIMB_DOWN - stick pulled down: is Bob standing at the top of a ladder?

;  (feet within 2 lines of y + 4, x within 2). Then frame 10, MOVE_A = 1,

;  MOVE_B = lines to the bottom.

;-----------------------------------------------------------------------------

TRY_CLIMB_DOWN: ldy #$00

TCD_LOOP:       lda (LADDERS),y         ; if (LADDERS)[Y] == 0 goto NO_LADDER

beq NO_LADDER

iny

iny

lda (LADDERS),y         ; top of the ladder - Bob's feet - 4

sec

sbc BOB_Y

sec

sbc #$19

cmp #$02                ; if A >= 2 goto TCD_NEXT

bcs TCD_NEXT

sta TMP9B

dey

lda (LADDERS),y

sec

sbc BOB_X

clc

adc #$02

cmp #$05                ; if A >= 5 goto TCD_NEXT_X

bcs TCD_NEXT_X

lda (LADDERS),y         ; line up and step down

sta BOB_X

inc BOB_Y

lda #$0A                ; frame 10 (stepping onto the ladder)

sta BOB_FRAME

lda #$01                ; MOVE_A = 1

sta MOVE_A

dey                     ; MOVE_B = height + 3 + offset = lines to the bottom

lda (LADDERS),y

adc #$03

adc TMP9B

sta MOVE_B

ldy #$00

jmp LADDER_ON

TCD_NEXT_X:     iny

TCD_NEXT:       iny

iny

jmp TCD_LOOP

NO_LADDER:      jsr READ_STICK          ; no ladder here: continue with left/right

lsr a

lsr a

jmp MB_STICK_LR

;-----------------------------------------------------------------------------

;  MOVE_BOB - Bob's state machine, run every frame by GAME_VBI.

;

;    SPECIAL = 1 ........ dying: MELT_BOB

;    SPECIAL = 2 ........ dead: nothing (GAME_VBI ends the turn)

;    SPECIAL = 3-7 ...... in the transporter: TRANSPORT

;    BOB_FALL ........... falling: FALLING

;    SLIDE_LEN .......... sliding down a chute: SLIDING

;    on a slide start ... begin a slide

;    BOB_FRAME 11-12 .... jumping: JUMPING

;    BOB_FRAME 7-10 ..... on a ladder: ON_LADDER

;    otherwise .......... on a girder: button = jump, up/down = ladders,

;                         left/right = walk, still in a transporter booth + digit = go

;-----------------------------------------------------------------------------

MOVE_BOB:       lda SPECIAL             ; if SPECIAL == 0 goto MB_NORMAL

beq MB_NORMAL

cmp #$01                ; if A <> 1 goto MB_SPECIAL

bne MB_SPECIAL

jmp MELT_BOB            ; SPECIAL = 1

MB_SPECIAL:     cmp #$03                ; if A < 3 goto MB_DEAD

bcc MB_DEAD

jmp TRANSPORT           ; the transporter, SPECIAL = 3..7, with a random buzz on voice 3:

MB_DEAD:        rts

MB_NORMAL:      lda BOB_FALL            ; if BOB_FALL == 0 goto MB_SLIDING

beq MB_SLIDING

jmp FALLING             ; one line down per frame with a falling whistle; then the landing test

MB_SLIDING:     lda SLIDE_LEN           ; if SLIDE_LEN == 0 goto MB_SLIDE_CHECK

beq MB_SLIDE_CHECK

jmp SLIDING             ; one line down and MOVE_A pixels sideways per frame, with a falling whi

MB_SLIDE_CHECK: ldy #$00                ; is Bob exactly at the top of a slide?

MBS_LOOP:       lda (SLIDES),y          ; rows (0 = end of list)

beq MB_FRAME

tax

iny

lda (SLIDES),y

iny

cmp BOB_X               ; if A <> BOB_X goto MBS_NEXT

bne MBS_NEXT

lda (SLIDES),y

cmp BOB_Y               ; if A <> BOB_Y goto MBS_NEXT

bne MBS_NEXT

stx SLIDE_LEN           ; yes: slide for that many frames

iny

ldx #$0B                ; frame 11 (sliding right) or 12 (left)

lda (SLIDES),y          ; MOVE_A = (SLIDES)[Y]

sta MOVE_A              ; MOVE_A = x step per frame

bpl MBS_FRAME

inx

MBS_FRAME:      stx BOB_FRAME

lda #$00                ; start 2 * step further back

sec

sbc (SLIDES),y

asl a

clc

adc BOB_X

sta BOB_X

jmp FALL_SOUND          ; slide noise

MBS_NEXT:       iny

iny

bne MBS_LOOP

MB_FRAME:       lda BOB_FRAME           ; jumping?

cmp #$0B                ; if A < $0B goto MB_LADDER_Q

bcc MB_LADDER_Q

jmp JUMPING             ; every other frame move MOVE_B sideways (unless blocked); every frame m

MB_LADDER_Q:    cmp #$07                ; on a ladder?

bcc MB_GROUND

jmp ON_LADDER           ; climbing, one line every other frame

MB_GROUND:      jsr READ_BUTTON         ; button: carry clear = pressed

lsr a

bcc MB_BUTTON

ldx JUMP_LOCK           ; not pressed: run the lock-out down

beq MB_STICK

dec JUMP_LOCK

bcs MB_STICK

MB_BUTTON:      ldy #$06                ; pressed: jump only on a fresh press (lock-out was 0)

ldx JUMP_LOCK

sty JUMP_LOCK

bne MB_STICK

lda CANNON_ST           ; and not while inside the cannon

bne MB_STICK

jmp START_JUMP          ; button pressed

MB_STICK:       jsr READ_STICK          ; stick bit 0 = up

lsr a

bcs MB_STICK_DOWN

jmp TRY_CLIMB_UP        ; stick pushed up

MB_STICK_DOWN:  lsr a                   ; bit 1 = down

bcs MB_STICK_LR

jmp TRY_CLIMB_DOWN      ; stick pulled down

MB_STICK_LR:    ldy WALK_STEP           ; bit 2 = left

ldx BOB_X

lsr a

bcs MB_STICK_RIGHT

jmp WALK_LEFT           ; as WALK_RIGHT, frames 4-6, feet tested at x+2..x+4

MB_STICK_RIGHT: lsr a                   ; bit 3 = right

bcc WALK_RIGHT

lda SPECIAL_X           ; standing still: inside a transporter booth?

clc

sbc BOB_X

cmp #$05                ; if A >= 5 goto MB_STILL

bcs MB_STILL

lda #$1D                ; on one of the four floors ($1D + 47 n)?

ldx #$04

MB_BOOTH:       cmp BOB_Y               ; if A <> BOB_Y goto MB_BOOTH_NEXT

bne MB_BOOTH_NEXT

jsr READ_DIGIT          ; a digit key 1-4 ...

beq MB_STILL

cpy #$05                ; if Y >= 5 goto MB_STILL

bcs MB_STILL

sty TRANS_DEST          ; ... that is not this floor ...

cpx TRANS_DEST          ; if X == TRANS_DEST goto MB_STILL

beq MB_STILL

lda TRANS_LOCK          ; ... and the transporter is not cooling down

bne MB_STILL

stx TRANS_SRC           ; go: start the transporter sequence

ldy #$03

sty SPECIAL

ldy #$08

sty MOVE_A

sty MOVE_B

jmp SPARKLE_BOB         ; draw Bob, then AND every line of his three players with a random numbe

MB_BOOTH_NEXT:  clc                     ; next floor up

adc #$2F

dex                     ; loop until X = 0

bne MB_BOOTH

MB_STILL:       jmp FS_QUIET

;-----------------------------------------------------------------------------

;  WALK_RIGHT - walk animation frames 1-3, one pixel every other frame.

;  After moving, look at Bob's feet:

;    girder 1 line above the feet  -> the girder rises: step up one line

;    girder under the feet          -> fine

;    girder 1 line below            -> the girder falls: step down

;    nothing                         -> walk off the edge and start falling

;-----------------------------------------------------------------------------

WALK_RIGHT:     iny                     ; next walk frame (1-3)

cpy #$04                ; if Y < 4 goto WR_FRAME

bcc WR_FRAME

ldy #$01

WR_FRAME:       sty WALK_STEP

sty BOB_FRAME

lda FRAMES              ; move on even frames only

and #$01                ; test bits $01

bne FOOTSTEPS

inx

jsr CHECK_RIGHT         ; wall or screen edge?

beq FOOTSTEPS

stx BOB_X

lda CANNON_ST           ; inside the cannon: no ground check

bne FOOTSTEPS

txa                     ; girder one line above the feet, at x + 5?

clc

adc #$05

tay

lda #$14

jsr PROBE_LINE          ; girder at x = Y, line BOB_Y + A?

bne WR_FEET

dec BOB_Y               ; yes: step up

jmp FOOTSTEPS

WR_FEET:        lda #$03                ; girder under the feet at x+3, x+4 or x+5?

sta TMP9B

jsr PROBE_FEET3         ; is there girder under Bob's feet at x = BOB_X + 3?

WR_FEET_LOOP:   beq FOOTSTEPS

dec TMP9B               ; TMP9B -= 1, goto WR_DOWN if 0

beq WR_DOWN

ldx PROBE_Y

ldy PROBE_X

iny

jsr PROBE_AT_Y          ; girder at x = Y, line PROBE_Y?

jmp WR_FEET_LOOP

WR_DOWN:        inc BOB_Y               ; none: try one line lower

jsr PROBE_FEET3         ; is there girder under Bob's feet at x = BOB_X + 3?

beq FOOTSTEPS           ; found: step down

ldx #$01                ; nothing: walked off, start to fall (frame 1)

START_FALL:     stx BOB_FRAME

ldx #$00

stx FALL_LEN

inx

stx BOB_FALL

FALL_SOUND:     lda #$1E                ; X = $1E: (pCOLPM3,x) = (pAUDF3), (pCOLPF0,x) = (pAUDC3)

tax

sta SND3_F

sta (pCOLPM3,x)

lda #$AA

sta (pCOLPF0,x)

bne FS_DONE

FOOTSTEPS:      ldy CANNON_ST           ; inside the cannon: a low rumble

beq FS_STEP

ldy #$4F

ldx #$1A

bne FS_SET

FS_STEP:        ldy SND3_F              ; 14-frame cycle: thud (0-2), silence, click (7-9), silence

iny

cpy #$0E                ; if Y < $0E goto FS_PHASE

bcc FS_PHASE

ldy #$00

FS_PHASE:       sty SND3_F

cpy #$03                ; if Y < 3 goto FS_THUD

bcc FS_THUD

cpy #$07                ; if Y < 7 goto FS_QUIET

bcc FS_QUIET

cpy #$0A                ; if Y < $0A goto FS_CLICK

bcc FS_CLICK

FS_QUIET:       ldy #$00

beq FS_VOL

FS_THUD:        ldx #$FA

ldy #$CA

bne FS_SET

FS_CLICK:       ldx #$00

ldy #$8A

FS_SET:         txa

ldx #$00

sta (pAUDF3,x)

FS_VOL:         tya

ldy #$00

sta (pAUDC3),y          ; AUDC3 = A

FS_DONE:        jmp SHOW_BOB            ; draw Bob (upside down once he has been fired from the cannon), and on

;-----------------------------------------------------------------------------

;  WALK_LEFT - as WALK_RIGHT, frames 4-6, feet tested at x+2..x+4.

;-----------------------------------------------------------------------------

WALK_LEFT:      iny

cpy #$04                ; if Y < 4 goto WL_FRAME

bcc WL_FRAME

ldy #$01

WL_FRAME:       sty WALK_STEP

tya

clc

adc #$03

sta BOB_FRAME

lda FRAMES

and #$01                ; test bits $01

bne FOOTSTEPS

dex

jsr CHECK_LEFT          ; wall or screen edge?

beq FOOTSTEPS

stx BOB_X

lda CANNON_ST           ; if CANNON_ST <> 0 goto FOOTSTEPS

bne FOOTSTEPS

txa                     ; girder one line above the feet, at x + 2?

tay

iny

iny

lda #$14

jsr PROBE_LINE          ; girder at x = Y, line BOB_Y + A?

bne WL_FEET

dec BOB_Y               ; step up

jmp FOOTSTEPS

WL_FEET:        lda #$03                ; TMP9B = 3

sta TMP9B

ldy BOB_X

iny

iny

jsr PROBE_FEET          ; girder at x = Y, line BOB_Y + $15 (the feet)?

WL_FEET_LOOP:   beq FOOTSTEPS

dec TMP9B               ; TMP9B -= 1, goto WL_DOWN if 0

beq WL_DOWN

ldx PROBE_Y

ldy PROBE_X

iny

jsr PROBE_AT_Y          ; girder at x = Y, line PROBE_Y?

jmp WL_FEET_LOOP

WL_DOWN:        inc BOB_Y

ldy BOB_X

iny

iny

jsr PROBE_FEET          ; girder at x = Y, line BOB_Y + $15 (the feet)?

bne WL_FALL

jmp FOOTSTEPS

WL_FALL:        ldx #$04                ; walked off: fall, facing left (frame 4)

jmp START_FALL

;-----------------------------------------------------------------------------

;  PROBE_FEET - is there girder (colour 3) at x = Y, line = BOB_Y + A?

;  PROBE_FEET3: x = BOB_X + 3, line = feet. PROBE_FEET: x in Y, line = BOB_Y + $15.

;  PROBE_AT: line in X. PROBE_AT_Y: x in Y, line already in PROBE_Y.

;  Out: Z set = solid. Station 8's lift counts as solid where Bob's feet meet its

;  platform; ON_LIFT is set to 0 there.

;-----------------------------------------------------------------------------

PROBE_FEET3:    ldy BOB_X

iny

iny

iny

PROBE_FEET:     lda #$15

PROBE_LINE:     clc

adc BOB_Y

tax

PROBE_AT:       stx PROBE_Y

PROBE_AT_Y:     sty PROBE_X

jsr READ_PIXEL_XY       ; girder?

beq PR_SET

lda BOB_Y               ; no: the lift's platform? (only at the feet line)

clc

adc #$15

cmp PROBE_Y             ; if A <> PROBE_Y goto PR_RTS

bne PR_RTS

cmp LIFT_Y              ; if A <> LIFT_Y goto PR_NONE

bne PR_NONE

lda PROBE_X

cmp LIFT_X              ; if A < LIFT_X goto PR_NONE

bcc PR_NONE

lda LIFT_X

adc #$0E

cmp PROBE_X             ; if A >= PROBE_X goto PR_LIFT

bcs PR_LIFT

PR_NONE:        lda #$02

bne PR_SET

PR_LIFT:        lda #$00                ; on the lift

sta ON_LIFT

lda #$03                ; Z set

cmp #$03

PR_RTS:         rts

PR_SET:         sta ON_LIFT

rts

;-----------------------------------------------------------------------------

;  TRANSPORT - the transporter, SPECIAL = 3..7, with a random buzz on voice 3:

;    3  Bob fades into sparkles while the buzz gets louder

;    4  a short pause

;    5  Bob appears at the chosen floor; the sparkles at the old booth die away

;    6  sparkle for a moment at the new booth

;    7  Bob forms again while the buzz fades; the transporter is then locked for a while

;-----------------------------------------------------------------------------

TRANSPORT:      ldy #$00                ; random buzz

lda (pSKRES),y          ; A = RANDOM

and #$1E

sta (pAUDF3),y          ; AUDF3 = A

ldx SPECIAL

dex

dex

dex

beq TR_FADE

dex

beq TR_PAUSE

dex                     ; loop until X = 0

bne TR_7Q

jmp TR_OLD_BOOTH

TR_7Q:          dex                     ; loop until X = 0

bne TR_ARRIVE

jmp TR_NEW_BOOTH

TR_ARRIVE:      dec MOVE_A              ; state 7: slow down the sparkle, quieter each step

bne TR_SHOW

inc MOVE_B

lda MOVE_B              ; MOVE_A = MOVE_B

sta MOVE_A

lda #$08

sec

sbc MOVE_B

asl a

ora #$A0

ldx #$05                ; X = 5: (pAUDF2+1,x) = (pAUDC3)

sta (pAUDF2+1,x)

cmp #$A0                ; fully formed?

bne TR_SPARKLE

stx TRANS_LOCK          ; lock the transporter (TRANS_LOCK = 5) and return to normal

ldx #$00

stx SPECIAL

TR_SHOW:        jmp SHOW_BOB            ; draw Bob (upside down once he has been fired from the cannon), and on

TR_SPARKLE:     jmp SPARKLE_BOB         ; draw Bob, then AND every line of his three players with a random numbe

TR_FADE:        dec MOVE_A              ; state 3: fade out, louder each step

bne TR_SHOW

dec MOVE_B              ; MOVE_B -= 1, goto TR_GONE if 0

beq TR_GONE

lda MOVE_B              ; MOVE_A = MOVE_B

sta MOVE_A

lda #$08

sec

sbc MOVE_B

asl a

ldy #$00

ora #$A0

sta (pAUDC3),y          ; AUDC3 = A

bne TR_SPARKLE

TR_GONE:        ldx #$04                ; gone: state 4 for 8 frames

stx SPECIAL

ldx #$08

stx MOVE_A

bne TR_SPARKLE

TR_PAUSE:       dec MOVE_A              ; state 4

bne TR_SPARKLE

ldx BOB_Y               ; remember the old height

stx TRANS_Y

ldx #$05                ; state 5

stx SPECIAL

ldx #$0E

stx MOVE_A

ldy TRANS_DEST          ; new height = $D9 - 47 * floor

lda #$D9

TR_FLOOR:       sec

sbc #$2F

dey                     ; loop until Y = 0

bne TR_FLOOR

sta BOB_Y

bne TR_SHOW

TR_OLD_BOOTH:   lda BOB_Y               ; state 5: sparkle at the old booth (BOB_Y briefly set back)

pha

lda TRANS_Y             ; BOB_Y = TRANS_Y

sta BOB_Y

jsr SPARKLE_BOB         ; draw Bob, then AND every line of his three players with a random numbe

pla

sta BOB_Y

lda FRAMES

and #$01                ; test bits $01

bne TR_SPARKLE

dec MOVE_A

bpl TR_SPARKLE

lda TRANS_Y             ; done: wipe the old image from players 0-2

clc

adc #$3C

tax

ldy #$14

lda #$00

TR_WIPE0:       sta $03F6,x

dex

dey                     ; loop until Y = 0

bne TR_WIPE0

ldy #$11

TR_WIPE1:       sta PM_P1+7,x

dex

dey                     ; loop until Y = 0

bne TR_WIPE1

ldy #$11

TR_WIPE2:       sta PM_P2+30,x

dex

dey                     ; loop until Y = 0

bne TR_WIPE2

ldx #$08                ; state 6 for 8 frames

stx MOVE_A

ldx #$06

stx SPECIAL

jsr TR_SPARKLE          ; draw Bob as sparkles

;-----------------------------------------------------------------------------

;  BOB_COLOURS - Bob's three colours: player 0 blue ($8A), player 1 orange ($3A),

;  player 2 white ($0F).

;-----------------------------------------------------------------------------

BOB_COLOURS:    lda #$8A

ldy #$00

sta (pCOLPM0),y         ; COLPM0 = A

lda #$3A                ; COLPM1 = $3A

sta (pCOLPM1),y         ; COLPM1 = A

lda #$0F                ; COLPM2 = $0F

sta (pCOLPM2),y         ; COLPM2 = A

rts

TR_NEW_BOOTH:   dec MOVE_A              ; state 6

beq TR_TO_7

jmp SPARKLE_BOB         ; draw Bob, then AND every line of his three players with a random numbe

TR_TO_7:        ldx #$07                ; state 7

stx SPECIAL

ldx #$01

stx MOVE_A

dex

stx MOVE_B

jmp TR_SPARKLE          ; draw Bob as sparkles

;-----------------------------------------------------------------------------

;  SLIDING - one line down and MOVE_A pixels sideways per frame, with a falling whistle (AUDF3 rises),

;  until SLIDE_LEN runs out.

;-----------------------------------------------------------------------------

SLIDING:        ldy #$00

inc BOB_Y

lda MOVE_A              ; BOB_X = MOVE_A + BOB_X

clc

adc BOB_X

sta BOB_X

lda FRAMES

and #$01                ; test bits $01

bne SL_COUNT

inc SND3_F              ; every second frame the whistle drops a step

lda SND3_F              ; AUDF3 = SND3_F

sta (pAUDF3),y          ; AUDF3 = A

SL_COUNT:       dec SLIDE_LEN           ; SLIDE_LEN -= 1, until 0

bne SL_SHOW

lda #$FF                ; end of the slide: sound to a low hiss

sta (pAUDF3),y          ; AUDF3 = A

lda #$8F                ; AUDC3 = $8F

sta (pAUDC3),y          ; AUDC3 = A

lda BOB_FRAME           ; standing frame facing the same way (1 or 4)

ldx #$01

cmp #$0B                ; if A == $0B goto SL_FRAME

beq SL_FRAME

ldx #$04

SL_FRAME:       stx BOB_FRAME

SL_SHOW:        jmp SHOW_BOB            ; draw Bob (upside down once he has been fired from the cannon), and on

;-----------------------------------------------------------------------------

;  MELT_BOB - SPECIAL = 1: Bob melts. Each frame a random byte is fed in at his feet in

;  each of his three players and the column above is shifted down a line, from the feet

;  up to MOVE_B; MOVE_B moves down one line per frame. When it reaches BOB_Y + $11 (his shins) he is a

;  puddle of noise and SPECIAL becomes 2 (dead).

;-----------------------------------------------------------------------------

MELT_BOB:       lda BOB_Y               ; feet line

clc

adc #$15

sta TMP9B

ldy #$04                ; player 0 ($041F + row)

ldx #$1F

jsr MELT_COLUMN         ; melt one of Bob's players (page Y)

ldy #$05                ; player 1

ldx #$1F

jsr MELT_COLUMN         ; melt one of Bob's players (page Y)

ldy #$06                ; player 2

ldx #$1F

jsr MELT_COLUMN         ; melt one of Bob's players (page Y)

inc MOVE_B

lda BOB_Y

clc

adc #$11

cmp MOVE_B              ; if A <> MOVE_B goto MELT_SHOW

bne MELT_SHOW

lda #$02                ; melted: dead

sta SPECIAL

MELT_SHOW:      jmp HIDE_IN_CANNON      ; station 10: hide Bob's legs inside the loaded cannon

MELT_COLUMN:    stx SPR_PTR

ldx #$00

sty SPR_PTR+1

ldy TMP9B

lda (pSKRES,x)          ; X = 0: (pSKRES,x) = RANDOM

sta (SPR_PTR),y

MELT_SHIFT:     dey

dey

lda (SPR_PTR),y

iny

sta (SPR_PTR),y

cpy MOVE_B              ; if Y <> MOVE_B goto MELT_SHIFT

bne MELT_SHIFT

rts

;-----------------------------------------------------------------------------

;  ON_LADDER - climbing, one line every other frame. MOVE_A = lines to the top,

;  MOVE_B = lines to the bottom; reaching either end puts Bob back on his feet (frame 0).

;  Frames: 7/8 alternate while climbing, 9 and 10 near the top as he steps on and off.

;-----------------------------------------------------------------------------

ON_LADDER:      lda FRAMES

and #$01                ; test bits $01

beq OL_MOVE

OL_RTS:         rts

OL_MOVE:        jsr READ_STICK          ; joystick in the PORTA format (bit 0 up, 1 down, 2 left, 3 right; 0 mea

lsr a                   ; up?

bcc OL_UP

lsr a                   ; down?

bcs OL_RTS

inc BOB_Y               ; down a line

inc MOVE_A

dec MOVE_B              ; MOVE_B -= 1, goto OL_OFF if 0

beq OL_OFF

OL_FRAME:       lda MOVE_A              ; near the top?

cmp #$05                ; if A >= 5 goto OL_CLIMB

bcs OL_CLIMB

ldx #$09                ; 3-4 lines from the top: frame 9; 1-2: frame 10

cmp #$03                ; if A >= 3 goto OL_SET

bcs OL_SET

inx

bne OL_SET

OL_CLIMB:       lda FRAMES              ; alternate frames 7 and 8 every 2 frames

and #$02                ; test bits $02

bne OL_SHOW

lda BOB_FRAME

ldx #$07

cmp #$08                ; if A == 8 goto OL_SET

beq OL_SET

inx

OL_SET:         stx BOB_FRAME

OL_SHOW:        jmp SHOW_BOB            ; draw Bob (upside down once he has been fired from the cannon), and on

OL_UP:          dec BOB_Y               ; up a line

inc MOVE_B

dec MOVE_A              ; MOVE_A -= 1, until 0

bne OL_FRAME

OL_OFF:         ldx #$00                ; off the ladder

beq OL_SET

;-----------------------------------------------------------------------------

;  CHECK_LEFT / CHECK_RIGHT - may Bob move to x = X?  Out: A = 0 (Z set) if blocked.

;  The screen edges ($30 and $C9), the walls of station 6's waste box (x $43/$BD and

;  $3B/$B5 below line $8A), the TNT store on station 10 (x $31/$5D, lines $4B-$A8), and

;  on station 10 inside the cannon (x $71-$A5).

;-----------------------------------------------------------------------------

CHECK_LEFT:     cpx #$30                ; left edge

bcc CK_BLOCKED

lda CANNON_ST           ; inside the cannon?

bne CK_CANNON_L

lda STATION

cmp #$0A                ; if A == $0A goto CK_TNT_L

beq CK_TNT_L

cmp #$06                ; if A <> 6 goto CK_RTS

bne CK_RTS

cpx #$43                ; station 6: the waste box's walls

bcc CK_RTS

beq CK_WASTE_Y

cpx #$BE                ; if X >= $BE goto CK_OK

bcs CK_OK

cpx #$BD                ; if X <> $BD goto CK_RTS

bne CK_RTS

CK_WASTE_Y:     lda BOB_Y               ; only at the box's height

cmp #$8A                ; if A < $8A goto CK_RTS

bcc CK_RTS

cmp #$C0                ; if A >= $C0 goto CK_RTS

bcs CK_RTS

CK_BLOCKED:     lda #$00

rts

CK_CANNON_L:    cpx #$71                ; if X >= $71 goto CK_OK

bcs CK_OK

bcc CK_BLOCKED

CK_TNT_L:       lda BOB_Y               ; station 10: the TNT store's walls

cpx #$31                ; if X == $31 goto CK_TNT_Y

beq CK_TNT_Y

cpx #$65                ; if X <> $65 goto CK_RTS

bne CK_RTS

cmp #$A9                ; if A >= $A9 goto CK_OK

bcs CK_OK

CK_TNT_Y:       cmp #$4B                ; if A < $4B goto CK_OK

bcc CK_OK

bcs CK_TNT_WALL

CHECK_RIGHT:    cpx #$CA                ; right edge

bcs CK_BLOCKED

lda CANNON_ST           ; if CANNON_ST <> 0 goto CK_CANNON_R

bne CK_CANNON_R

lda STATION

cmp #$0A                ; if A == $0A goto CK_TNT_R

beq CK_TNT_R

cmp #$06                ; if A <> 6 goto CK_RTS

bne CK_RTS

cpx #$B6                ; if X >= $B6 goto CK_OK

bcs CK_OK

cpx #$3B                ; if X < $3B goto CK_RTS

bcc CK_RTS

beq CK_WASTE_Y

cpx #$B5                ; if X == $B5 goto CK_WASTE_Y

beq CK_WASTE_Y

CK_OK:          lda #$01

CK_RTS:         rts

CK_TNT_R:       cpx #$5D                ; if X <> $5D goto CK_RTS

bne CK_RTS

lda BOB_Y

cmp #$A9                ; if A >= $A9 goto CK_OK

bcs CK_OK

cmp #$4B                ; if A < $4B goto CK_OK

bcc CK_OK

CK_TNT_WALL:    lda #$00                ; blocked, and a jump stops moving sideways

sta MOVE_B

rts

CK_CANNON_R:    cpx #$A6                ; if X >= $A6 goto CK_BLOCKED

bcs CK_BLOCKED

bcc CK_OK

;-----------------------------------------------------------------------------

;  START_JUMP - button pressed: begin a jump. MOVE_A = 0 (position in JUMP_ARC), MOVE_B =

;  -1/0/+1 from the stick, frame 11 or 12 depending on the way he faces.

;-----------------------------------------------------------------------------

START_JUMP:     ldx #$00

stx FALL_LEN

stx MOVE_A

lda #$5A                ; jump sound

sta SND3_F

sta (pAUDF3,x)

lda #$AA

sta (pAUDC3,x)

jsr READ_STICK          ; direction from the stick: left -1, right +1, else 0

lsr a

lsr a

ldx #$FF

lsr a

bcc SJ_DIR

inx

lsr a

bcs SJ_DIR

inx

SJ_DIR:         stx MOVE_B

ldx #$0B                ; frame 11, or 12 if facing left

lda BOB_FRAME

cmp #$04                ; if A < 4 goto SJ_FRAME

bcc SJ_FRAME

inx

SJ_FRAME:       stx BOB_FRAME

;-----------------------------------------------------------------------------

;  JUMPING - every other frame move MOVE_B sideways (unless blocked); every frame move

;  vertically by JUMP_ARC[MOVE_A]: 16 lines up with two short hovers, then down. Past

;  the end of the table Bob falls one line per frame until he lands.

;  On station 10 two spots jump straight to the falling part (index $29): the top of

;  the TNT store and the cannon's platform, so Bob cannot jump out of them.

;-----------------------------------------------------------------------------

JUMPING:        lda FRAMES

and #$01                ; test bits $01

bne JP_VERTICAL

lda BOB_X

clc

adc MOVE_B

tax

lda MOVE_B              ; if MOVE_B bit 7 clear goto JP_RIGHT

bpl JP_RIGHT

jsr CHECK_LEFT          ; may Bob move left to x = X?

jmp JP_MOVE

JP_RIGHT:       jsr CHECK_RIGHT         ; may Bob move right to x = X?

JP_MOVE:        beq JP_VERTICAL

stx BOB_X

JP_VERTICAL:    ldx MOVE_A

lda STATION

cmp #$0A                ; if A <> $0A goto JP_ARC

bne JP_ARC

lda BOB_Y

cmp #$A8                ; if A == $A8 goto JP_TNT

beq JP_TNT

cmp #$65                ; if A <> $65 goto JP_ARC

bne JP_ARC

lda BOB_X

cmp #$5D                ; if A >= $5D goto JP_ARC

bcs JP_ARC

ldx #$29

bne JP_ARC

JP_TNT:         lda BOB_X

cmp #$5D                ; if A < $5D goto JP_ARC

bcc JP_ARC

cmp #$66                ; if A >= $66 goto JP_ARC

bcs JP_ARC

ldx #$29

JP_ARC:         lda JUMP_ARC,x

cpx #$20                ; past the table: +1

bcc JP_APPLY

lda #$01

JP_APPLY:       inx

stx MOVE_A

clc

adc BOB_Y

sta BOB_Y

ldy SND3_F              ; sound: pitch up while rising (index < $16) ...

cpx #$16                ; if X < $16 goto JP_RISING

bcc JP_RISING

cpx #$2A                ; ... falling whistle after $2A ...

bcs FA_SOUND

iny                     ; ... else pitch down

iny

iny

tya

ldy #$00

sta SND3_F

sta (pAUDF3),y          ; AUDF3 = A

lda #$AA                ; AUDC3 = $AA

sta (pAUDC3),y          ; AUDC3 = A

bcc LAND_CHECK

JP_RISING:      dey

dey

dey

tya

sta SND3_F

ldy #$00

sta (pAUDF3),y          ; AUDF3 = A

bcc LC_SHOW

;-----------------------------------------------------------------------------

;  FALLING - one line down per frame with a falling whistle; then the landing test.

;-----------------------------------------------------------------------------

FALLING:        inc BOB_Y

FA_SOUND:       lda #$AA

ldy #$00

sta (pAUDC3),y          ; AUDC3 = A

lda FRAMES

and #$01                ; test bits $01

bne LAND_CHECK

inc SND3_F

lda SND3_F              ; AUDF3 = SND3_F

sta (pAUDF3),y          ; AUDF3 = A

;-----------------------------------------------------------------------------

;  LAND_CHECK - count the fall; if there is girder under x+3 or x+4 (and not 2 lines

;  higher, which would mean his feet are already inside it), land, lifting Bob one line

;  if needed. Forty or more frames of falling (FALL_LEN) kill him.

;-----------------------------------------------------------------------------

LAND_CHECK:     inc FALL_LEN

jsr PROBE_FEET3         ; is there girder under Bob's feet at x = BOB_X + 3?

beq LC_SOLID

ldx PROBE_Y

ldy PROBE_X

iny

jsr PROBE_AT_Y          ; girder at x = Y, line PROBE_Y?

bne LC_SHOW

LC_SOLID:       ldx PROBE_Y             ; solid 2 lines up as well? still going through: carry on

ldy PROBE_X

dex

dex

jsr PROBE_AT            ; girder at x = Y, line X?

beq LC_SHOW

ldx PROBE_Y             ; solid 1 line below the feet as well? then he is a line too low: up one

ldy PROBE_X

inx

jsr PROBE_AT            ; girder at x = Y, line X?

bne LC_LANDED

dec BOB_Y

LC_LANDED:      ldx #$00

stx BOB_FALL

lda FALL_LEN            ; fallen for 40 frames or more?

cmp #$28                ; if A < $28 goto LC_SAFE

bcc LC_SAFE

jsr SHOW_BOB            ; draw Bob (upside down once he has been fired from the cannon), and on

;-----------------------------------------------------------------------------

;  START_DEATH - Bob dies: SPECIAL = 1 (MELT_BOB), melt starts at his top line, with a

;  decaying tone on voice 3.

;-----------------------------------------------------------------------------

START_DEATH:    ldx #$01                ; X = 1 for the (pointer,x) tricks below

stx SPECIAL

lda BOB_Y               ; MOVE_B = BOB_Y

sta MOVE_B

lda #$8F                ; (pAUDF3+1,x) = (pAUDC3): pure tone, volume 15

sta (pAUDF3+1,x)

lda #$0F                ; decays via SND3_VOL

sta SND3_VOL

lda #$96                ; (pAUDC2+1,x) = (pAUDF3)

sta (pAUDC2+1,x)

SD_RTS:         rts

LC_SAFE:        lda #$00                ; meant to silence voice 3 like START_DEATH, but X = 0 here, so (pAUDF3+1,x)

tax                     ; takes bytes $59/$5A (AUDF3's high byte, AUDC3's low byte) as the pointer:

;   $05D2

sta (pAUDF3+1,x)        ; ($05E8 on the 5200), a line of player 1 - harmless, Bob is redrawn every

;   frame

lda BOB_FRAME           ; standing frame facing the same way

cmp #$05                ; if A < 5 goto LC_SHOW

bcc LC_SHOW

ldx #$01

cmp #$0B                ; if A == $0B goto LC_FRAME

beq LC_FRAME

ldx #$04

LC_FRAME:       stx BOB_FRAME

LC_SHOW:        jmp SHOW_BOB            ; draw Bob (upside down once he has been fired from the cannon), and on

;-----------------------------------------------------------------------------

;  UPDATE_LIFTS - move the moving platforms (LIFT_TAB records, see INIT_LIFTS).

;  Each platform moves one pixel each time its timer runs out: the column at the leading

;  edge is drawn and the one at the trailing edge erased, so only 6 pixels change. After

;  "travel" steps it turns round. A platform carries Bob with it when his feet are on it.

;  Station 9's platforms wait at each end until Bob steps onto them.

;  On station 9 the column erased behind a platform between x $1E-$25 is redrawn in colour 2:

;  that is where a ladder passes behind it.

;-----------------------------------------------------------------------------

UPDATE_LIFTS:   ldy #$00

UL_LOOP:        sty CNT1

lda LIFT_TAB,y          ; timer (0 = end of list)

beq SD_RTS

tya

tax

dec LIFT_TAB,x          ; count down

beq UL_MOVE             ; time to move

UL_NEXT_X:      txa

UL_NEXT:        clc                     ; next record

adc #$06

tay

jmp UL_LOOP

UL_MOVE:        lda (LIFTS),y           ; reload the timer from the station data

sta LIFT_TAB,y

lda STATION             ; station 8's lift is handled elsewhere

cmp #$08                ; if A == 8 goto UL_STEP

beq UL_STEP

lda BOB_Y               ; Bob's feet on the platform's line ...

clc

adc #$15

cmp LIFT_TAB+3,y        ; if A <> LIFT_TAB+3[Y] goto UL_STATION9

bne UL_STATION9

lda BOB_FALL            ; ... not falling ...

bne UL_STATION9

lda SPECIAL             ; ... walking or standing (or in a special state) ...

bne UL_ON_X

lda BOB_FRAME

cmp #$07                ; if A >= 7 goto UL_STATION9

bcs UL_STATION9

UL_ON_X:        lda LIFT_TAB+1,y        ; ... and within its width?

clc

adc #$05

sta PLOT_X

lda LIFT_TAB+2,y

adc #$2E

adc LIFT_TAB+1,y

sec

sbc BOB_X

cmp PLOT_X              ; if A >= PLOT_X goto UL_STATION9

bcs UL_STATION9

lda BOB_X               ; yes: carry Bob along

adc LIFT_TAB+5,y

sta BOB_X

jmp UL_STEP

UL_STATION9:    lda STATION             ; station 9: a platform at the start of its run waits for Bob

cmp #$09                ; if A <> 9 goto UL_STEP

bne UL_STEP

tya

clc

adc #$04

tay

lda (LIFTS),y

cmp LIFT_TAB+4,x        ; if A == LIFT_TAB+4[X] goto UL_NEXT_X

beq UL_NEXT_X

txa

tay

UL_STEP:        lda LIFT_TAB+5,y        ; moving left or right?

bpl UL_RIGHT

dec LIFT_TAB+2,x        ; left: x - 1; draw at x, erase at x + width

lda LIFT_TAB+2,y        ; PLOT_X = LIFT_TAB+2[Y]

sta PLOT_X

clc

adc LIFT_TAB+1,y

sta CNT2

jmp UL_DRAW

UL_RIGHT:       lda LIFT_TAB+2,y        ; right: draw at x + width, erase at x; x + 1

sta CNT2

clc

adc LIFT_TAB+1,y

sta PLOT_X

inc LIFT_TAB+2,x

UL_DRAW:        ldx #$08                ; leading column in colour 3 (colour 1 on station 8)

lda STATION

cmp #$08                ; if A <> 8 goto UL_DRAW2

bne UL_DRAW2

ldx #$00

UL_DRAW2:       jsr LIFT_COLUMN         ; 3 pixels down from the platform's line at x = PLOT_X, colour X

lda CNT2                ; trailing column in the background colour ...

sta PLOT_X

ldx #$0C

cmp #$1E                ; if A < $1E goto UL_ERASE

bcc UL_ERASE

cmp #$26                ; if A >= $26 goto UL_ERASE

bcs UL_ERASE

lda STATION             ; ... or colour 2 where station 9's ladder is

cmp #$09                ; if A <> 9 goto UL_ERASE

bne UL_ERASE

ldx #$04

UL_ERASE:       jsr LIFT_COLUMN         ; 3 pixels down from the platform's line at x = PLOT_X, colour X

tya                     ; end of the run?

tax

dec LIFT_TAB+4,x        ; LIFT_TAB+4[X] -= 1, goto UL_TURN if 0

beq UL_TURN

jmp UL_NEXT

UL_TURN:        lda #$00                ; reverse the direction and reload the run length

sec

sbc LIFT_TAB+5,y

sta LIFT_TAB+5,y

tya

clc

adc #$04

tay

lda (LIFTS),y           ; LIFT_TAB[Y] = (LIFTS)[Y]

sta LIFT_TAB,y

iny

iny

jmp UL_LOOP

;-----------------------------------------------------------------------------

;  LIFT_COLUMN - 3 pixels down from the platform's line at x = PLOT_X, colour X.

;-----------------------------------------------------------------------------

LIFT_COLUMN:    stx PLOT_COLOR

ldx #$03

stx CNT3

lda LIFT_TAB+3,y        ; PLOT_Y = LIFT_TAB+3[Y]

sta PLOT_Y

LC_LOOP:        ldy PLOT_X

ldx PLOT_Y

jsr PLOT_XY             ; set pixel (x = Y, line = X) to PLOT_COLOR

inc PLOT_Y

dec CNT3                ; CNT3 -= 1, until 0

bne LC_LOOP

ldy CNT1

CF_RTS:         rts

;-----------------------------------------------------------------------------

;  CLAIM_FLOOR - the heart of the game: the girder section under Bob's feet becomes his.

;

;  Runs every frame while Bob walks or stands (not on the bottom floor, not during the

;  start-up second, not dying, falling, sliding or in a special state). For the current

;  animation frame, FEET_OFFSETS gives the x of his two feet. Under each foot:

;    * the feet line must be girder (colour 3);

;    * an unclaimed section is striped, so one of the next two lines is background:

;      if the line below the feet is background, the gaps are at +1, +3, +5 (3 rows),

;      otherwise at +2, +4 (2 rows); if both lines are solid it is already claimed.

;    * claim it: SECTIONS - 1, score + 5, find the section's left edge (the stripe

;      pattern changes between neighbouring sections), and fill its gap rows with

;      colour 3, so the 4-pixel section turns solid.

;  The station is complete when SECTIONS reaches 0.

;-----------------------------------------------------------------------------

CLAIM_FLOOR:    ldy BOB_Y               ; not on the bottom floor

cpy #$AA                ; if Y == $AA goto CF_RTS

beq CF_RTS

lda MOVE_OK             ; not before Bob may move

beq CF_RTS

lda KILLED              ; not when killed

bne CF_RTS

lda DYING               ; not when dying

bne CF_RTS

lda BOB_FALL            ; not when falling

bne CF_RTS

lda SLIDE_LEN           ; not when sliding

bne CF_RTS

lda SPECIAL             ; not in a special state

bne CF_RTS

lda BOB_FRAME           ; walking or standing only

cmp #$07                ; if A >= 7 goto CF_RTS

bcs CF_RTS

asl a                   ; the two foot positions for this frame

tax

lda FEET_OFFSETS,x

pha

inx

lda FEET_OFFSETS,x

jsr CLAIM_AT            ; second foot

pla                     ; first foot

jmp CLAIM_AT

CLAIM_AT:       clc                     ; x = BOB_X + offset

adc BOB_X

sta PLOT_X

lda BOB_Y               ; y = feet line

clc

adc #$15

sta PLOT_Y

jsr READ_PIXEL          ; girder under the foot?

bne CF_RTS

inc PLOT_Y              ; line below solid?

jsr READ_PIXEL          ; colour of the pixel at P/M x = PLOT_X, line y = PLOT_Y (0 = background

bne CA_THREE

inc PLOT_Y              ; and the one after: if solid too, already claimed

jsr READ_PIXEL          ; colour of the pixel at P/M x = PLOT_X, line y = PLOT_Y (0 = background

beq CF_RTS

ldx #$02                ; 2 gap rows (at +2, +4)

bne CA_CLAIM

CA_THREE:       ldx #$03                ; 3 gap rows (at +1, +3, +5)

CA_CLAIM:       stx CNT0

dec SECTIONS            ; one section fewer to go

ldx #$06                ; 5 points (digit 6 = units)

lda #$05

jsr ADD_SCORE           ; add A to score digit X (1 = 100,000s

dec PLOT_X              ; find the left edge of the section: solid just left of here (in a gap row)?

jsr READ_PIXEL          ; colour of the pixel at P/M x = PLOT_X, line y = PLOT_Y (0 = background

beq CA_FILL

ldy #$03                ; no: look up to 3 more pixels left

sty CNT1

CA_LEFT:        dec PLOT_X

ldy PLOT_X

cpy #$2F                ; left edge of the screen

beq CA_EDGE

jsr READ_PIXEL_PTR      ; colour of pixel x = Y on the line in PIX_PTR

beq CA_FILL

dec CNT1                ; CNT1 -= 1, until 0

bne CA_LEFT

inc PLOT_X              ; none: the section starts further right; step 4 right, then scan right

inc PLOT_X

inc PLOT_X

inc PLOT_X

CA_RIGHT:       inc PLOT_X

CA_RIGHT_CHK:   ldy PLOT_X

jsr READ_PIXEL_PTR      ; colour of pixel x = Y on the line in PIX_PTR

bne CA_RIGHT

lda PLOT_X              ; PLOT_X = PLOT_X - 5

sec

sbc #$05

sta PLOT_X

CA_FILL:        inc PLOT_X              ; first pixel of the section

ldy #$08                ; fill in colour 3

sty PLOT_COLOR

lda PLOT_X              ; P/M x -> pixel x

sec

sbc #$30

sta PLOT_X

CA_ROW:         ldy #$03                ; 4 pixels in this gap row

sty CNT1

ldx PLOT_Y

ldy PLOT_X

jsr PLOT_XY             ; set pixel (x = Y, line = X) to PLOT_COLOR

CA_PIX:         inc PLOT_X

ldy PLOT_X

jsr PLOT_PTR            ; set pixel x = Y on the line already in PTR

dec CNT1                ; CNT1 -= 1, until 0

bne CA_PIX

inc PLOT_Y              ; next gap row (2 lines down)

inc PLOT_Y

dec PLOT_X

dec PLOT_X

dec PLOT_X

dec CNT0                ; CNT0 -= 1, until 0

bne CA_ROW

rts

CA_EDGE:        lda #$05                ; reached the left edge of the screen

sec

sbc CNT1

clc

adc PLOT_X

sta PLOT_X

jmp CA_RIGHT_CHK

;-----------------------------------------------------------------------------

;  READ_HITS - save the collision registers for band X: any missile touching a player

;  (M0PL..M3PL) in HIT_M, player 3 touching a player (P3PL) in HIT_P3. Bob is players

;  0-2, so bits 0-2 mean "touched Bob".

;-----------------------------------------------------------------------------

READ_HITS:      ldy #$00

lda (pSIZEP0),y         ; A = M0PL

ora (pSIZEP1),y         ; A |= M1PL

ora (pSIZEP2),y         ; A |= M2PL

ora (pSIZEP3),y         ; A |= M3PL

sta MUT_SCORE,x

lda (pGRAFP2),y         ; A = P3PL

sta HIT_P3-1,x

rts

;-----------------------------------------------------------------------------

;  DLI_MUTANTS - one DLI at the end of the status line and three more down the playfield

;  (bitmap lines 46, 93 and 140) divide the screen into four bands. Eight mutants share

;  two sprites: in each band one is drawn with player 3 and one with the four missiles

;  side by side (fifth-player mode, colour COLPF3). Each DLI:

;    * saves and clears the collisions of the band just finished (READ_HITS);

;    * re-positions player 3 and the missiles for this band's two mutants;

;    * sets band-specific things: the real playfield colours below the status line, the

;      transporter booth colour (green, or red while it recharges), the lift (station 8,

;      band 3) or the cannon (station 10, bottom band) which also use player 3 and the

;      missiles, and a flickering colour 1 in the bottom band on stations 5, 6, 8 and 9

;      (the radioactive waste glows);

;    * moves two mutants and draws their shapes (the work is spread over the four DLIs).

;-----------------------------------------------------------------------------

DLI_MUTANTS:    pha                     ; save A, X, Y

txa

pha

tya

pha

lda DLI_BAND            ; first DLI of the frame (status line): no band finished yet

cmp #$04                ; if A == 4 goto DM_HITCLR

beq DM_HITCLR

lda #$04                ; band index 1-4

sec

sbc DLI_BAND

tax

jsr READ_HITS           ; save the collision registers for band X

DM_HITCLR:      ldy #$00

sta (pHITCLR),y         ; clear the collision registers

lda SPECIAL             ; transporter state 5: Bob's colours flash in the booths involved

cmp #$05                ; if A <> 5 goto DM_BAND

bne DM_BAND

ldx DLI_BAND

cpx TRANS_SRC           ; if X == TRANS_SRC goto DM_FLASH

beq DM_FLASH

cpx TRANS_DEST          ; if X <> TRANS_DEST goto DM_BAND

bne DM_BAND

lda #$0E

sec

sbc MOVE_A

bcs DM_FLASH_SET

DM_FLASH:       lda MOVE_A

DM_FLASH_SET:   sta (pCOLPM2),y         ; COLPM2 = A

cmp #$0B                ; if A < $0B goto DM_FLASH2

bcc DM_FLASH2

lda #$0A

DM_FLASH2:      ora #$30

sta (pCOLPM1),y         ; COLPM1 = A

adc #$50

sta (pCOLPM0),y         ; COLPM0 = A

DM_BAND:        lda DLI_BAND            ; which band?

cmp #$04                ; if A == 4 goto DM_TOP

beq DM_TOP

cmp #$03                ; if A <> 3 goto DM_BAND1

bne DM_BAND1

lda STATION             ; band 3 on station 8: player 3 is the lift

cmp #$08                ; if A <> 8 goto DM_BAND_J

bne DM_BAND_J

lda LIFT_X              ; HPOSP3 = LIFT_X

sta (pHPOSP3),y         ; HPOSP3 = A

lda #$0E                ; COLPM3 = $0E

sta (pCOLPM3),y         ; COLPM3 = A

lda #$01                ; double width

sta (pSIZEP3),y         ; SIZEP3 = A

DM_BAND_J:      jmp DM_POSITION

DM_BAND1:       cmp #$01                ; if A <> 1 goto DM_POSITION

bne DM_POSITION

lda STATION             ; bottom band on station 10: the cannon (missiles + player 3)

cmp #$0A                ; if A <> $0A goto DM_GLOW

bne DM_GLOW

lda CANNON_X

ldx #$06

clc

DM_CANNON:      sta (pHPOSM0,x)         ; HPOSM3 = x, M2 = x+2, M1 = x+4, M0 = x+6

adc #$02

dex

dex

bpl DM_CANNON

sta (pHPOSP3),y         ; player 3 to the right of them

lda #$04                ; COLPF3 = 4

sta (pCOLPF3),y         ; COLPF3 = A

sta (pCOLPM3),y         ; COLPM3 = A

bne DM_POSITION

DM_GLOW:        cmp #$08                ; stations 5, 6, 8, 9: colour 1 flickers in the bottom band

beq DM_GLOW_SET

cmp #$09                ; if A == 9 goto DM_GLOW_SET

beq DM_GLOW_SET

cmp #$05                ; if A == 5 goto DM_GLOW_SET

beq DM_GLOW_SET

cmp #$06                ; if A <> 6 goto DM_POSITION

bne DM_POSITION

DM_GLOW_SET:    ldx TRANS_LOCK

lda FRAMES              ; a new random colour every 4 frames

and #$03                ; test bits $03

bne DM_COLPF0

lda (pSKRES),y          ; A = RANDOM

and #$F6

clc

adc #$06

sta TRANS_LOCK

tax

bne DM_COLPF0

DM_TOP:         lda COLPF1_V            ; below the status line: the station's own colours 2 and 3

sta (pCOLPF1),y         ; COLPF1 = A

lda COLPF2_V            ; COLPF2 = COLPF2_V

sta (pCOLPF2),y         ; COLPF2 = A

lda #$00                ; missiles normal width (they are double width for the bonus box)

sta (pSIZEM),y          ; SIZEM = A

lda SPECIAL_X           ; transporter booths?

beq DM_POSITION

ldx #$C6                ; green ...

lda TRANS_LOCK          ; if TRANS_LOCK == 0 goto DM_COLPF0

beq DM_COLPF0

lda FRAMES              ; ... red while it recharges: one step per 64 frames (128 from zone 5)

and #$3F

ora PAUSED

bne DM_RED

lda ZONE

cmp #$05                ; if A < 5 goto DM_RECHARGE

bcc DM_RECHARGE

lda FRAMES

and #$7F                ; test bits $7F

bne DM_RED

DM_RECHARGE:    dec TRANS_LOCK          ; TRANS_LOCK -= 1, goto DM_COLPF0 if 0

beq DM_COLPF0

DM_RED:         ldx #$42

DM_COLPF0:      txa

sta (pCOLPF0),y         ; COLPF0 = A

DM_POSITION:    ldx MUT_IDX             ; the missile mutant of this band (the record after MUT_IDX)

lda MUT_TAB+11,x        ; if MUT_TAB+11[X] == 0 goto DM_PLAYER3

beq DM_PLAYER3

ldx #$06                ; four missiles side by side, 2 apart

clc

DM_MISSILES:    sta (pHPOSM0,x)

adc #$02

dex

dex

bpl DM_MISSILES

ldx MUT_IDX             ; its colour

lda MUT_TAB+14,x        ; COLPF3 = MUT_TAB+14[X]

sta (pCOLPF3),y         ; COLPF3 = A

DM_PLAYER3:     lda MUT_TAB,x           ; the player-3 mutant of this band

beq DM_LOGIC

sta (pHPOSP3),y         ; HPOSP3 = A

lda MUT_TAB+3,x         ; COLPM3 = MUT_TAB+3[X]

sta (pCOLPM3),y         ; COLPM3 = A

DM_LOGIC:       lda #$02                ; update two mutants: the first drawn into the missiles ($0300) ...

sta MUT_LOOP

lda #$00                ; MUT_PM = 0

sta MUT_PM

lda #$03

;-----------------------------------------------------------------------------

;  MUT_UPDATE - move and animate one mutant; its record is at MUT_TAB + $4D - MUT_IDX.

;  Record (11 bytes, set up from the station's mutant list):

;    0 x (0 = none, 1 = eaten)    1 normal colour      2 state: bit 0 step, bit 1 facing,

;    3 colour now                 4 colour timer         bit 2 edible, 8/9 = eaten (the

;                                                        shape becomes "80" or "90")

;    5 brightness step (+/-2)     6 steps left in this direction

;    7 dx (+/-1)                  8 frames per step      9 animation timer

;   10 edible time

;  The station's list gives y, the step timer, the patrol length and the colour timer.

;-----------------------------------------------------------------------------

MUT_UPDATE:     sta MUT_PM+1

ldy #$00

lda #$4D

sec

sbc MUT_IDX

tax

lda MUT_TAB,x           ; nobody there (0) or eaten (1)?

cmp #$02                ; if A < 2 goto MU_SKIP

bcc MU_SKIP

lda PAUSED              ; paused?

bne MU_SKIP

lda ALIVE               ; turn over?

beq MU_SKIP

lda SECTIONS            ; station cleared?

bne MU_ACTIVE

MU_SKIP:        jmp MU_NEXT

MU_ACTIVE:      lda MUT_TAB+2,x         ; being eaten?

cmp #$08                ; if A < 8 goto MU_HIT

bcc MU_HIT

lda (pSKRES),y          ; flashing random colour

ora #$08

sta MUT_TAB+3,x

lda MUT_TAB+9,x         ; rising tone on voice 2 (AUDF2 counts down)

sta (pAUDF2),y          ; AUDF2 = A

lda #$8A                ; AUDC2 = $8A

sta (pAUDC2),y          ; AUDC2 = A

dec MUT_TAB+9,x

jmp MU_ANIM

MU_HIT:         lda MUT_PM+1            ; did it touch Bob in its band? (collision bits 0-2 = players 0-2)

clc

adc DLI_BAND

tay

lda a:$00E2,y

and MOVE_OK

beq MU_COLOUR

lda MUT_TAB+10,x        ; edible?

beq MU_KILL

lda #$FA                ; yes: Bob eats it: 80 or 90 points (added by GAME_VBI), then it flashes and

;   disappears

sta MUT_TAB+9,x

ldy #$00

lda (pSKRES),y          ; A = RANDOM

and #$01

ora #$08

sta MUT_SCORE

jmp MU_SHAPE

MU_KILL:        sty KILLED              ; no: Bob is killed

MU_COLOUR:      dec MUT_TAB+4,x         ; colour timer

bne MU_MOVE

lda MUT_TAB+10,x        ; edible: blink green ($C6) / purple ($5A) ...

beq MU_SHIMMER

lda MUT_TAB+3,x

eor #$9C

sta MUT_TAB+3,x

cmp #$C6                ; if A == $C6 goto MU_EDIBLE_TICK

beq MU_EDIBLE_TICK

lda #$05

bne MU_BLINK

MU_EDIBLE_TICK: lsr MUT_TAB+10,x        ; ... and each green blink halves the time left

lda MUT_TAB+10,x        ; if MUT_TAB+10[X] <> 0 goto MU_BLINK

bne MU_BLINK

lda MUT_TAB+1,x         ; edible time over: back to its own colour

sta MUT_TAB+3,x

lda MUT_TAB+2,x

and #$03

sta MUT_TAB+2,x

lda #$02

MU_BLINK:       sta MUT_TAB+4,x

bne MU_MOVE

MU_SHIMMER:     txa                     ; normal: brightness pulses up and down by 2

clc

adc #$04

tay

lda (MUTANTS),y         ; MUT_TAB+4[X] = (MUTANTS)[Y]

sta MUT_TAB+4,x

lda MUT_TAB+5,x         ; MUT_TAB+3[X] = MUT_TAB+5[X] + MUT_TAB+3[X]

clc

adc MUT_TAB+3,x

sta MUT_TAB+3,x

and #$0F                ; test bits $0F

beq MU_STEP_UP

cmp #$0E                ; if A <> $0E goto MU_MOVE

bne MU_MOVE

lda #$FE

bne MU_STEP_SET

MU_STEP_UP:     lda #$02

MU_STEP_SET:    sta MUT_TAB+5,x

MU_MOVE:        dec MUT_TAB+8,x         ; time for a step?

bne MU_ANIM

txa                     ; next step after (list value + 1 - zone) frames, at least 1

clc

adc #$08

tay

lda (MUTANTS),y

clc

adc #$01

sec

sbc ZONE

beq MU_FASTEST

bcs MU_DELAY

MU_FASTEST:     lda #$01

MU_DELAY:       sta MUT_TAB+8,x

lda MUT_TAB+7,x         ; x = x + dx

clc

adc MUT_TAB,x

sta MUT_TAB,x

dec MUT_TAB+6,x         ; end of the patrol?

bne MU_ANIM

dey                     ; turn round: reload the patrol length, negate dx, face the other way

dey

lda (MUTANTS),y         ; MUT_TAB+6[X] = (MUTANTS)[Y]

sta MUT_TAB+6,x

lda #$00                ; MUT_TAB+7[X] = 0 - MUT_TAB+7[X]

sec

sbc MUT_TAB+7,x

sta MUT_TAB+7,x

lda MUT_TAB+2,x

eor #$02

sta MUT_TAB+2,x

jmp MU_ANIM_NOW

MU_ANIM:        dec MUT_TAB+9,x         ; animation timer

bne MU_NEXT

MU_ANIM_NOW:    lda #$0A                ; MUT_TAB+9[X] = $0A

sta MUT_TAB+9,x

lda MUT_TAB+2,x         ; eaten and finished?

cmp #$08                ; if A < 8 goto MU_NEXT_STEP

bcc MU_NEXT_STEP

lda #$00                ; voice 2 off, mutant gone

tay

sta (pAUDC2),y          ; AUDC2 = A

lda #$01                ; MUT_TAB[X] = 1

sta MUT_TAB,x

bne MU_NEXT

MU_NEXT_STEP:   eor #$01                ; toggle the walking step

MU_SHAPE:       sta MUT_TAB+2,x         ; draw shape (state * 12) at the mutant's y + $20 in its P/M memory

asl a

asl a

sta TMPEF

asl a

adc TMPEF

pha

txa

tay

iny

lda (MUTANTS),y

adc #$20

tay

pla

tax

lda #$0A                ; 10 lines

sta TMPEF

MU_COPY:        lda MUTANT_SHAPES-2,x   ; (MUT_PM)[Y] = MUTANT_SHAPES-2[X]

sta (MUT_PM),y

inx

iny

dec TMPEF               ; TMPEF -= 1, until 0

bne MU_COPY

MU_NEXT:        lda MUT_IDX             ; next record; the second one goes into player 3 ($0700)

clc

adc #$0B

sta MUT_IDX

lda #$07

dec MUT_LOOP            ; MUT_LOOP -= 1, goto MU_DONE if 0

beq MU_DONE

jmp MUT_UPDATE          ; move and animate one mutant; its record is at MUT_TAB + $4D - MUT_IDX

MU_DONE:        dec DLI_BAND            ; next band

jmp NMI_EXIT            ; common tail of every interrupt handler

;-----------------------------------------------------------------------------

;  READ_BUTTON - fire button into bit 0 of A (0 = pressed): TRIG0, or bit 4 of the

;  current demo input.

;-----------------------------------------------------------------------------

READ_BUTTON:    lda DEMO_PTR+1          ; demo?

bne RB_DEMO

sty SAVE_Y2

ldy #$00

lda (pTRIG0),y          ; A = TRIG0

ldy SAVE_Y2

rts

RB_DEMO:        jsr DEMO_INPUT          ; current byte of the demo recording

and #$10

lsr a

lsr a

lsr a

lsr a

rts

;-----------------------------------------------------------------------------

;  READ_STICK - joystick in the PORTA format (bit 0 up, 1 down, 2 left, 3 right; 0 means

;  pushed), from the demo recording, PORTA (400/800) or the 5200's analog stick.

;  The 5200 stick is turned into the same four bits: below $30 or above $B4 on either

;  pot counts as pushed.

;-----------------------------------------------------------------------------

READ_STICK:     lda DEMO_PTR+1          ; if DEMO_PTR+1 <> 0 goto RS_DEMO

bne RS_DEMO

lda IS5200              ; if IS5200 <> 0 goto RS_5200

bne RS_5200

lda PORTA               ; 400/800: stick 0 is the low nibble of PORTA

rts

RS_DEMO:        jsr DEMO_INPUT          ; current byte of the demo recording

and #$0F

rts

RS_5200:        sty SAVE_Y2

ldy POT_X

lda #$0F

cpy #$B4                ; far right

bcc RS_LEFT

eor #$08

RS_LEFT:        cpy #$30                ; far left

bcs RS_Y

eor #$04

RS_Y:           ldy POT_Y

cpy #$B4                ; far down

bcc RS_UP

eor #$02

RS_UP:          cpy #$30                ; far up

bcs RS_DONE

eor #$01

jmp RS_DONE

DEMO_INPUT:     sty SAVE_Y2             ; current byte of the recording

ldy #$00

lda (DEMO_PTR),y

RS_DONE:        ldy SAVE_Y2

rts

;-----------------------------------------------------------------------------

;  CANNON - station 10, every frame.

;    state 0: the cannon rolls back to its rest position ($A1). If Bob, carrying TNT, steps

;             into its mouth he gets in (state 1).

;    state 1: the cannon follows Bob as he walks; the button fires it (state 2).

;    state 2: Bob flies up two lines a frame to a height set by the charge:

;             1 ton $5D, 2 tons $33, 3 tons $09 (the three upper platforms), 4 or more

;             and he is blown off the top of the screen (to $DF), turning upside down -

;             hence the "DANGER" warning.

;-----------------------------------------------------------------------------

CANNON:         ldx CANNON_ST

dex

beq CN_AIM

dex

beq CN_FLY

lda CANNON_X            ; state 0: roll back one pixel every 4 frames

cmp #$A1                ; if A == $A1 goto CN_REST

beq CN_REST

lda FRAMES

and #$03                ; test bits $03

bne CN_REST

inc CANNON_X

CN_REST:        lda CANNON_X            ; Bob in the mouth (x = cannon + 4, y = $8F, on his feet)?

clc

adc #$04

cmp BOB_X               ; if A <> BOB_X goto CN_EXIT

bne CN_EXIT

lda BOB_Y

cmp #$8F                ; if A <> $8F goto CN_EXIT

bne CN_EXIT

lda BOB_FRAME

cmp #$07                ; if A >= 7 goto CN_EXIT

bcs CN_EXIT

lda #$02                ; FALL_LEN = 2

sta FALL_LEN

lda TNT                 ; only with TNT on board

beq CN_EXIT

lda #$00                ; in he goes: state 1

sta BOB_FALL

inc CANNON_ST

CN_EXIT:        jmp NMI_EXIT            ; common tail of every interrupt handler

CN_AIM:         lda BOB_X               ; state 1: the cannon moves with Bob

sec

sbc #$04

sta CANNON_X

lda DYING               ; if DYING <> 0 goto CN_EXIT

bne CN_EXIT

lda SPECIAL             ; if SPECIAL <> 0 goto CN_EXIT

bne CN_EXIT

lda (pTRIG0,x)          ; fire?

and #$01                ; test bits $01

bne CN_EXIT

ldx #$FF                ; fire: state 2, jump frame

stx SND3_F

inx

stx MOVE_A

stx FALL_LEN

stx MOVE_B

lda #$02                ; CANNON_ST = 2

sta CANNON_ST

ldx #$0B

lda BOB_FRAME

cmp #$04                ; if A < 4 goto CN_FRAME

bcc CN_FRAME

inx

CN_FRAME:       stx BOB_FRAME

ldy #$00

CN_SOUND:       tya

clc

adc SND3_F

sta SND3_F

ldy #$00

sta (pAUDF3),y          ; AUDF3 = A

lda #$8F                ; AUDC3 = $8F

sta (pAUDC3),y          ; AUDC3 = A

jmp NMI_EXIT            ; common tail of every interrupt handler

CN_FLY:         dec BOB_Y               ; state 2: up 2 lines per frame

dec BOB_Y

ldy #$FE                ; target height from the charge

lda #$5D

ldx TNT

dex

beq CN_TARGET

lda #$33

dex

beq CN_TARGET

lda #$09

dex

beq CN_TARGET

ldy #$FC                ; too much TNT: 4 lines per frame, to $DF (off the top)

lda #$DF

dec BOB_Y

dec BOB_Y

CN_TARGET:      cmp BOB_Y               ; if A <> BOB_Y goto CN_SOUND

bne CN_SOUND

cmp #$DF                ; if A <> $DF goto CN_LAND

bne CN_LAND

sta CANNON_DONE         ; upside down from now on

CN_LAND:        inc BOB_Y

inc BOB_Y

lda #$00                ; cannon empty and unloaded

sta CANNON_ST

sta TNT

jmp NMI_EXIT            ; common tail of every interrupt handler

;-----------------------------------------------------------------------------

;  VBI_TAIL - end of GAME_VBI: advance the demo recording, then station 10's cannon or

;  station 8's lift, then return from the interrupt.

;-----------------------------------------------------------------------------

VBI_TAIL:       lda DEMO_PTR+1          ; demo: count down the current input ...

beq VT_STATION

dec DEMO_CNT            ; DEMO_CNT -= 1, until 0

bne VT_STATION

lda DEMO_PTR            ; ... and move to the next (input, frames) pair

clc

adc #$02

sta DEMO_PTR

bcc VT_COUNT

inc DEMO_PTR+1

VT_COUNT:       ldy #$01

lda (DEMO_PTR),y        ; DEMO_CNT = (DEMO_PTR)[Y]

sta DEMO_CNT

VT_STATION:     lda STATION

cmp #$0A                ; if A <> $0A goto LIFT_CONTROL

bne LIFT_CONTROL

jmp CANNON              ; station 10, every frame

;-----------------------------------------------------------------------------

;  LIFT_CONTROL - station 8. Standing on the lift, SPACE (or the 5200's # key) hands the

;  joystick to the lift; SPACE or the button hands it back. While in control the stick

;  drives the lift (and Bob on it): left/right within x $67-$9B, up/down within lines

;  $59-$B3. It rises at half the speed it falls (a line every 4

;  frames up, every 2 down). The scissor arms are extended or

;  folded by one line each time the platform moves, and a motor hum plays on voice 3.

;-----------------------------------------------------------------------------

LIFT_CONTROL:   ldx LIFT_X              ; no lift?

beq LC_EXIT

ldx ON_LIFT             ; Bob not on it?

bne LC_EXIT

ldx LIFT_DLY

lda LIFT_ON             ; already driving?

bne LC_DRIVE

lda BOB_FRAME           ; only when walking or standing ...

cmp #$07                ; if A >= 7 goto LC_EXIT

bcs LC_EXIT

lda SPECIAL             ; if SPECIAL <> 0 goto LC_EXIT

bne LC_EXIT

lda DYING               ; if DYING <> 0 goto LC_EXIT

bne LC_EXIT

jsr LIFT_SPACE          ; ... and SPACE pressed

bne LC_EXIT

ldx #$05                ; take control

stx JUMP_LOCK

stx LIFT_DLY

LC_ON:          stx LIFT_ON

LC_EXIT:        jmp NMI_EXIT            ; common tail of every interrupt handler

LC_DRIVE:       jsr LIFT_KEY            ; SPACE or button again?

bne LC_STICK

ldx #$05                ; give control back, motor off

stx LIFT_DLY

lda #$00

tax

sta (pAUDC3,x)

beq LC_ON

LC_STICK:       ldy #$00                ; motor hum

lda #$4F                ; AUDC3 = $4F

sta (pAUDC3),y          ; AUDC3 = A

ldx #$00                ; direction from the stick

jsr READ_STICK          ; joystick in the PORTA format (bit 0 up, 1 down, 2 left, 3 right; 0 mea

lsr a

bcs LC_DOWN

ldy #$FF

LC_DOWN:        lsr a

bcs LC_LEFT

ldy #$01

LC_LEFT:        lsr a

bcs LC_RIGHT

ldx #$FF

LC_RIGHT:       lsr a

bcs LC_PITCH

ldx #$01

LC_PITCH:       lda #$5F                ; motor note: idle, or higher when moving sideways

dex

inx

beq LC_UP

lda #$17

LC_UP:          cpy #$FF                ; going up: only one frame in four

bne LC_DOWN_NOTE

lda FRAMES

and #$03                ; test bits $03

beq LC_UP_NOTE

ldy #$00

LC_UP_NOTE:     lda #$1A

LC_DOWN_NOTE:   cpy #$01                ; if Y <> 1 goto LC_SET_NOTE

bne LC_SET_NOTE

lda #$17

LC_SET_NOTE:    sty SAVE_Y2

ldy #$00

sta (pAUDF3),y          ; AUDF3 = A

lda FRAMES              ; move on even frames only

and #$01                ; test bits $01

bne LC_DONE

txa                     ; sideways, within limits

clc

adc LIFT_X

cmp #$67                ; if A == $67 goto LC_VERT

beq LC_VERT

cmp #$9B                ; if A == $9B goto LC_VERT

beq LC_VERT

sta LIFT_X

txa                     ; Bob moves with it

clc

adc BOB_X

sta BOB_X

sta (pHPOSP0),y         ; HPOSP0 = A

sta (pHPOSP1),y         ; HPOSP1 = A

sta (pHPOSP2),y         ; HPOSP2 = A

LC_VERT:        ldy SAVE_Y2             ; up or down, within limits

beq LC_DONE

tya

clc

adc LIFT_Y

cmp #$59                ; if A == $59 goto LC_DONE

beq LC_DONE

cmp #$B3                ; if A == $B3 goto LC_DONE

beq LC_DONE

sta LIFT_Y

lda SAVE_Y2             ; Bob too

clc

adc BOB_Y

sta BOB_Y

ldx LIFT_ANIM           ; scissor phase +1 going down, -1 going up (cycle of 7)

dey                     ; loop until Y = 0

bne LC_PHASE_UP

inx

cpx #$07                ; if X <> 7 goto LC_PHASE

bne LC_PHASE

ldx #$00

beq LC_PHASE

LC_PHASE_UP:    dex

bpl LC_PHASE

ldx #$06

LC_PHASE:       stx LIFT_ANIM

ldy LIFT_Y              ; redraw the top of the lift in player 3: clear above, platform, scissor

;   line

lda #$00                ; PM_P3+31[Y] = 0

sta PM_P3+31,y

lda #$FF                ; PM_P3+32[Y] = $FF

sta PM_P3+32,y

sta PM_P3+33,y

lda LIFT_SCISSOR,x      ; PM_P3+34[Y] = LIFT_SCISSOR[X]

sta PM_P3+34,y

jsr SHOW_BOB            ; draw Bob (upside down once he has been fired from the cannon), and on

LC_DONE:        jmp NMI_EXIT            ; common tail of every interrupt handler

;-----------------------------------------------------------------------------

;  LIFT_KEY - Z set if the button (LIFT_KEY) or SPACE (LIFT_SPACE) has just been pressed:

;  LIFT_DLY must have counted down to 0 while nothing was pressed.

;-----------------------------------------------------------------------------

LIFT_KEY:       ldy #$00

lda (pTRIG0),y          ; A = TRIG0

and #$01                ; test bits $01

beq LK_PRESSED

LIFT_SPACE:     lda KEYCODE

cmp #$21                ; if A <> $21 goto LK_UP

bne LK_UP

lda #$FF                ; consume the key

sta KEYCODE

LK_PRESSED:     dex                     ; pressed: Z set only if the delay had run out

inx

bne LK_RESTART

rts

LK_UP:          dex                     ; not pressed: count the delay down

bpl LK_SET

rts

LK_RESTART:     ldx #$05

LK_SET:         stx LIFT_DLY

inx

rts

;-----------------------------------------------------------------------------

;  READ_POTS - 5200 analog stick: POT0 -> POT_X, POT1 -> POT_Y, start the next scan.

;-----------------------------------------------------------------------------

READ_POTS:      ldy #$00

lda (pAUDF1),y          ; A = POT0

sta POT_X

lda (pAUDC1),y          ; A = POT1

sta POT_Y

sta (pPOTGO),y          ; POTGO = A

rts

;-----------------------------------------------------------------------------

;  GAME_VBI - the game. Everything that moves runs here, once per frame:

;    frame counter, display list, the bonus box (missiles), P/M DMA, mutant band setup,

;    colours, 5200 pots, mutants turning edible, "time running out" warnings, pause,

;    status-line blink, Bob touching deadly scenery, dying, the extra-life jingle,

;    MOVE_BOB, moving platforms, Bob's sprite position and size, CLAIM_FLOOR,

;    pulverizers, end-of-turn timing, the credits reveal, status line, items, the bonus

;    timer, the end-of-station bonus count and screen wipe, mutant points, the phone-number

;    code, and finally VBI_TAIL (demo, cannon, lift).

;-----------------------------------------------------------------------------

GAME_VBI:       jsr POLL_KEYS           ; called at the start of every VBI handler

inc FRAMES

lda #$20                ; display list at $2000 (Y = 0 after POLL_KEYS)

sta DLISTH

sty DLISTL

lda #$55                ; missiles double width: the box behind the bonus

sta (pSIZEM),y          ; SIZEM = A

lda #$84                ; HPOSM3 = $84, M2 = $80, M1 = $7C (M0 = $78, or $7C below 1000)

ldx #$06

sec

GV_BOX:         sta (pHPOSM0,x)

sbc #$04

dex

dex                     ; loop until X = 0

bne GV_BOX

ldx STATUS+18           ; narrower box when the bonus is under 1000

bne GV_BOX0

lda #$7C

GV_BOX0:        sta (pHPOSM0),y         ; HPOSM0 = A

ldx #$3E                ; DMACTL: normal width, players + missiles, single-line, display list

stx DMACTL

lda #$03                ; X = $3E: ($0C,x) = (pGRACTL): players and missiles on

sta (pGRACTL-$3E,x)

ldx #$04                ; the first DLI handles band 4

stx DLI_BAND

jsr READ_HITS           ; collisions of the last band of the previous frame

tya

sta (pSIZEP3),y         ; player 3 normal width, first mutant record

sta MUT_IDX

lda COLPF0_V            ; status line colours

sta (pCOLPF0),y         ; COLPF0 = A

lda COLBK_V             ; COLBK = COLBK_V

sta (pCOLBK),y          ; COLBK = A

sta (pCOLPF2),y         ; COLPF2 = A

lda #$0C                ; COLPF1 = $0C

sta (pCOLPF1),y         ; COLPF1 = A

jsr READ_POTS           ; 5200 stick

ldx ITEM_GOT            ; an item was picked up: every mutant still alive turns edible

beq GV_WARN

lda ZONE                ; edible time = $AF - 16 * min(zone, 9) (no SEC: carry is clear)

cmp #$0A                ; if A < $0A goto GV_EDIBLE_TIME

bcc GV_EDIBLE_TIME

lda #$09

GV_EDIBLE_TIME: asl a

asl a

asl a

asl a

sta ITEM_GOT

lda #$B0

sbc ITEM_GOT

sta ITEM_GOT

ldx #$00

ldy #$08

GV_EDIBLE:      lda MUT_TAB+2,x

cmp #$08                ; if A >= 8 goto GV_EDIBLE_NEXT

bcs GV_EDIBLE_NEXT

ora #$04                ; edible, green, start blinking

sta MUT_TAB+2,x

lda #$01                ; MUT_TAB+9[X] = 1

sta MUT_TAB+9,x

lda #$C6                ; MUT_TAB+3[X] = $C6

sta MUT_TAB+3,x

lda ITEM_GOT            ; MUT_TAB+10[X] = ITEM_GOT

sta MUT_TAB+10,x

sta MUT_TAB+4,x

GV_EDIBLE_NEXT: txa

clc

adc #$0B

tax

dey                     ; loop until Y = 0

bne GV_EDIBLE

sty ITEM_GOT

GV_WARN:        ldy #$84                ; bonus box colour

lda DYING               ; time running out (bonus under 700, Bob alive, station not done)?

ora PAUSED

bne GV_BOX_COL

lda STATION

cmp #$0B                ; if A == $0B goto GV_BOX_COL

beq GV_BOX_COL

lda ALIVE               ; if ALIVE == 0 goto GV_BOX_COL

beq GV_BOX_COL

ldx SPECIAL

dex

cpx #$02                ; if X < 2 goto GV_BOX_COL

bcc GV_BOX_COL

lda SECTIONS            ; if SECTIONS == 0 goto GV_BOX_COL

beq GV_BOX_COL

lda STATUS+18

cmp #$11                ; if A >= $11 goto GV_BOX_COL

bcs GV_BOX_COL

ldx STATUS+19

cpx #$17                ; if X >= $17 goto GV_BOX_COL

bcs GV_BOX_COL

ldx #$00                ; Bob's face colour flickers

lda (pSKRES,x)

and #$0A

ora #$30

sta (pCOLPM1,x)

ldx STATUS+19           ; the box blinks (8 frames on, 8 off)

lda FRAMES

and #$0F

cmp #$08                ; if A < 8 goto GV_TICK

bcc GV_TICK

ldy #$00

GV_TICK:        and #$03                ; a tick every 4 frames ...

bne GV_BOX_COL

lda SECTIONS            ; if SECTIONS == 0 goto GV_BOX_COL

beq GV_BOX_COL

cpx #$10                ; if X == $10 goto GV_BOX_COL

beq GV_BOX_COL

txa

ldx #$00

dec TICK_SND            ; ... with gaps that shrink as the bonus falls

bne GV_TICK_VOL

sec

sbc #$11

asl a

adc #$02

sta TICK_SND

lda #$DC

sta (pAUDF1,x)

ldx #$AD

GV_TICK_VOL:    txa

ldx #$00

sta (pAUDC1,x)

GV_BOX_COL:     tya

ldy #$00

sta (pCOLPF3),y         ; COLPF3 = A

lda ALIVE               ; turn over: nothing more

beq GV_EXIT

lda PAUSED              ; paused?

beq GV_RUN

jsr CHECK_PAUSE         ; ESC (5200

lda PAUSED              ; if PAUSED == 0 goto GV_RUN

beq GV_RUN

jsr SILENCE             ; still paused: keep quiet

GV_EXIT:        jmp NMI_EXIT            ; common tail of every interrupt handler

GV_RUN:         dec BLINK_TMR           ; every 30 frames toggle CHACTL bit 0 (blank bit-7 characters): the score

;   blinks

bne GV_TOUCH

lda #$1E                ; BLINK_TMR = $1E

sta BLINK_TMR

lda #$01                ; BLINK_ON = 1 - BLINK_ON

sec

sbc BLINK_ON

sta BLINK_ON

sta CHACTL

GV_TOUCH:       lda DYING               ; already dying?

bne GV_DYING

lda #$00                ; Bob (players 0-2) touching pixel colour 1?

ldx #$04

GV_TOUCH_LOOP:  ora (pHPOSM0,x)         ; (pHPOSM2,x) .. (pHPOSM0,x) read P2PF, P1PF, P0PF

dex

dex

bpl GV_TOUCH_LOOP

ldy STATION

and #$01                ; test bits $01

beq GV_KILLED

cpy #$06                ; deadly on station 6 (the waste), 8 (its moving bar) ...

beq GV_DIE

cpy #$08                ; if Y == 8 goto GV_DIE

beq GV_DIE

cpy #$09                ; ... and 9 (a pulverizer: a jump is stopped dead)

bne GV_KILLED

lda BOB_FRAME

cmp #$0B                ; if A < $0B goto GV_DIE

bcc GV_DIE

lda #$20                ; MOVE_A = $20

sta MOVE_A

lda #$00                ; MOVE_B = 0

sta MOVE_B

beq GV_DIE

GV_KILLED:      lda KILLED              ; touched by a mutant or out of time?

beq GV_XLIFE

GV_DIE:         ldx #$01                ; Bob starts dying

stx DYING

dex

stx LIFT_ON

lda STATION             ; station 10: inside or beside the cannon he melts at once

cmp #$0A                ; if A <> $0A goto GV_DIE_POSE

bne GV_DIE_POSE

ldx CANNON_ST

dex

beq GV_MELT

dex

beq GV_DIE_POSE

lda TNT                 ; if TNT == 0 goto GV_DIE_POSE

beq GV_DIE_POSE

lda BOB_FRAME

cmp #$07                ; if A >= 7 goto GV_DIE_POSE

bcs GV_DIE_POSE

lda CANNON_X

clc

adc #$04

cmp BOB_X               ; if A <> BOB_X goto GV_DIE_POSE

bne GV_DIE_POSE

lda BOB_Y

cmp #$7C                ; if A < $7C goto GV_DIE_POSE

bcc GV_DIE_POSE

cmp #$92                ; if A < $92 goto GV_MELT

bcc GV_MELT

GV_DIE_POSE:    lda BOB_FRAME           ; in the air: wait until he lands

cmp #$0B                ; if A >= $0B goto GV_THROB

bcs GV_THROB

cmp #$07                ; on a ladder: he lets go and falls (a fatal fall)

bcc GV_DYING2

sta BOB_FALL

dec BOB_Y

ldx #$1E

stx SND3_F

ldx #$32

stx FALL_LEN

bne GV_THROB

GV_DYING:       lda BOB_FRAME           ; dying and in the air: wait

cmp #$0B                ; if A >= $0B goto GV_THROB

bcs GV_THROB

GV_DYING2:      lda SPECIAL             ; on the ground and not yet melting: melt

bne GV_THROB

lda BOB_FALL            ; if BOB_FALL <> 0 goto GV_THROB

bne GV_THROB

GV_MELT:        jsr START_DEATH         ; Bob dies

GV_THROB:       lda FRAMES              ; every 4 frames: noise, and Bob's size pulses (DYING 4,3,2,1,4 ...)

and #$03                ; test bits $03

bne GV_XLIFE

ldx #$00

lda (pSKRES,x)

and #$0F

sta (pAUDF1,x)

lda #$8E

sta (pAUDC1,x)

ldx DYING

dex                     ; loop until X = 0

bne GV_THROB_SET

ldx #$04

GV_THROB_SET:   stx DYING

GV_XLIFE:       lda XLIFE               ; extra-life jingle playing (XLIFE 1-9)?

beq GV_XLIFE_OFF

cmp #$0A                ; if A == $0A goto GV_XLIFE_OFF

beq GV_XLIFE_OFF

lda FRAMES              ; every 16 frames one note

and #$0F                ; test bits $0F

bne GV_XLIFE_NOTE

dec XLIFE               ; XLIFE -= 1, until 0

bne GV_XLIFE_NOTE

ldx #$00                ; finished: voice 4 off

txa

sta (pAUDC4,x)

beq GV_XLIFE_OFF

GV_XLIFE_NOTE:  cmp #$08                ; each 16 frames: 8 frames of note $8C on a blue background, 8 of note $64

;   on black

bcc GV_XLIFE_ON

lda #$00

ldx #$64

bne GV_XLIFE_SET

GV_XLIFE_ON:    lda #$88

ldx #$8C

GV_XLIFE_SET:   ldy #$00

pha

txa

sta (pAUDF4),y          ; AUDF4 = A

lda #$AA                ; AUDC4 = $AA

sta (pAUDC4),y          ; AUDC4 = A

pla

jmp GV_BACKGROUND

GV_XLIFE_OFF:   lda #$00

GV_BACKGROUND:  sta COLBK_V

lda MOVE_OK             ; Bob may move, the station is not finished and he is not driving the lift?

beq GV_POSITION

lda SECTIONS            ; if SECTIONS == 0 goto GV_POSITION

beq GV_POSITION

lda LIFT_ON             ; if LIFT_ON <> 0 goto GV_POSITION

bne GV_POSITION

lda CANNON_ST           ; flying from the cannon: only redraw him

cmp #$02                ; if A <> 2 goto GV_MOVE

bne GV_MOVE

jsr SHOW_BOB            ; draw Bob (upside down once he has been fired from the cannon), and on

jmp GV_TRAIL

GV_MOVE:        jsr MOVE_BOB            ; move Bob

lda STATION

cmp #$0A                ; if A <> $0A goto GV_POSITION

bne GV_POSITION

GV_TRAIL:       lda CANNON_DONE         ; station 10: wipe the 5 lines under Bob's sprite (he can move 2 lines a

;   frame)

bne GV_POSITION

ldx SPECIAL             ; if SPECIAL <> 0 goto GV_POSITION

bne GV_POSITION

lda BOB_Y

clc

adc #$30

tax

ldy #$05

lda #$00

GV_TRAIL_LOOP:  sta KERNEL_DELAY1,x

sta PM_P1,x

sta PM_P2+6,x

inx

dey                     ; loop until Y = 0

bne GV_TRAIL_LOOP

GV_POSITION:    jsr UPDATE_LIFTS        ; moving platforms

lda SECTIONS            ; Bob's horizontal position (0 = off screen once the station is cleared) ...

beq GV_HPOS

ldy DYING

lda BOB_X               ; ... offset to keep him centred while he throbs

clc

adc BOB_SIZE_X,y

GV_HPOS:        ldx #$00

sta (pHPOSP0,x)

sta (pHPOSP1,x)

sta (pHPOSP2,x)

lda BOB_SIZE,y          ; width of players 0-2 (normal, double, quadruple)

sta (pSIZEP0,x)

sta (pSIZEP1,x)

sta (pSIZEP2,x)

jsr CLAIM_FLOOR         ; claim girder sections

jsr MOVE_PULVERIZERS    ; station 9's pulverizers

lda FRAMES              ; every 4 frames voice 3 fades one step

and #$03                ; test bits $03

bne GV_DEAD

tay

ldx SND3_VOL            ; if SND3_VOL == 0 goto GV_DEAD

beq GV_DEAD

dex

stx SND3_VOL

txa

ora #$80

sta (pAUDC3),y          ; AUDC3 = A

GV_DEAD:        lda SPECIAL             ; dead (and not during the extra-life jingle): end the turn after two

;   seconds

cmp #$02                ; if A <> 2 goto GV_CREDITS

bne GV_CREDITS

ldx XLIFE

dex

cpx #$09                ; if X < 9 goto GV_CREDITS

bcc GV_CREDITS

lda ALIVE               ; if ALIVE == 0 goto GV_CREDITS

beq GV_CREDITS

dec SEC_TMR             ; SEC_TMR -= 1, until 0

bne GV_CREDITS

ldx #$3C

stx SEC_TMR

dec ALIVE

GV_CREDITS:     lda STATION             ; credits screen: reveal the hidden text around Bob

cmp #$0B                ; if A <> $0B goto GV_PLAY

bne GV_PLAY

lda BOB_Y               ; 30 lines from 12 above Bob's top ...

sec

sbc #$0C

sta PLOT_Y

lda #$1E                ; CNT3 = $1E

sta CNT3

lda BOB_X               ; ... at x = BOB_X + 3

clc

adc #$03

sta PLOT_X

lda #$04                ; recolour in colour 2

sta PLOT_COLOR

GV_REVEAL:      ldy PLOT_Y

cpy #$08                ; if Y < 8 goto GV_REVEAL_NEXT

bcc GV_REVEAL_NEXT

cpy #$BF                ; if Y >= $BF goto GV_REVEAL_NEXT

bcs GV_REVEAL_NEXT

jsr READ_PIXEL          ; a hidden (colour 1) pixel?

cmp #$01                ; if A <> 1 goto GV_REVEAL_NEXT

bne GV_REVEAL_NEXT

lda PLOT_X

sec

sbc #$30

tay

jsr PLOT_YX_Y           ; set pixel (x = Y, line = PLOT_Y) to PLOT_COLOR

GV_REVEAL_NEXT: inc PLOT_Y

dec CNT3                ; CNT3 -= 1, until 0

bne GV_REVEAL

jmp VBI_TAIL            ; end of GAME_VBI

GV_PLAY:        jsr DRAW_STATUS         ; status line

jsr UPDATE_ITEMS        ; items

lda DYING               ; dying or dead: no timer

bne BD_DONE

ldx SPECIAL

dex

cpx #$02                ; if X < 2 goto BD_DONE

bcc BD_DONE

lda SECTIONS            ; station cleared: count the bonus into the score

beq TALLY

dec BONUS_TMR           ; bonus timer: -100 every 180 frames (3 seconds)

bne BD_DONE

lda #$B4                ; BONUS_TMR = $B4

sta BONUS_TMR

BONUS_DOWN:     dec STATUS+19           ; hundreds digit - 1

lda STATUS+19

cmp #$10                ; if A == $10 goto BD_ZERO

beq BD_ZERO

bcs BD_DONE

lda #$19                ; borrow from the thousands

sta STATUS+19

dec STATUS+18

lda STATUS+18

cmp #$10                ; if A <> $10 goto GV_MUTSCORE

bne GV_MUTSCORE

lda #$00                ; (a leading zero is shown as a blank)

sta STATUS+18

beq GV_MUTSCORE

BD_ZERO:        ldx STATUS+18           ; bonus 0 while sections remain: out of time

bne GV_MUTSCORE

lda SECTIONS            ; if SECTIONS == 0 goto GV_MUTSCORE

beq GV_MUTSCORE

inx

stx KILLED

bne GV_MUTSCORE

BD_DONE:        jmp GV_MUTSCORE

TALLY:          tay                     ; station cleared: voices 2 and 3 off

sta (pAUDC3),y          ; AUDC3 = A

sta (pAUDC2),y          ; AUDC2 = A

lda STATUS+18           ; bonus left?

ora STATUS+19

cmp #$10                ; if A == $10 goto WIPE

beq WIPE

lda FRAMES              ; every 4 frames: a click, +100 to the score, -100 from the bonus

and #$03                ; test bits $03

beq TALLY_STEP

cmp #$02                ; if A <> 2 goto GV_MUTSCORE

bne GV_MUTSCORE

tya

sta (pAUDC4),y          ; AUDC4 = A

beq GV_MUTSCORE

TALLY_STEP:     lda #$AA

ldx #$04

sta (pAUDC3,x)

lda #$01

jsr ADD_SCORE           ; add A to score digit X (1 = 100,000s

lda #$3F                ; X = $3F: ($1D,x) = (pAUDF4): AUDF4 = $3F

tax

sta (pHPOSM2+1,x)

bne BONUS_DOWN

WIPE:           ldx XLIFE               ; bonus all counted (and no jingle playing): wipe the screen from the bottom

dex

cpx #$09                ; if X < 9 goto GV_MUTSCORE

bcc GV_MUTSCORE

ldx WIPE_LINE           ; remove the mutants' P/M data on this line

lda #$00                ; PM_MISSILES+39[X] = 0

sta PM_MISSILES+39,x

sta PM_P3+39,x

lda #$41                ; replace this display-list line by a JVB: the picture ends here

cpx #$62                ; if X >= $62 goto WIPE_LMS

bcs WIPE_LMS

sta DLIST+5,x

bcc WIPE_NEXT

WIPE_LMS:       sta DLIST+7,x           ; (lines after the LMS sit 2 bytes further on)

WIPE_NEXT:      dex

stx WIPE_LINE

txa                     ; rising tone

ldx #$00

sta (pAUDF4,x)

lda #$AA

sta (pAUDC4,x)

GV_MUTSCORE:    lda MUT_SCORE           ; a mutant was eaten: 80 or 90 points (digit 5 = tens)

beq GV_PAUSE

ldx #$05

jsr ADD_SCORE           ; add A to score digit X (1 = 100,000s

sta MUT_SCORE

GV_PAUSE:       jsr CHECK_PAUSE         ; ESC pauses

ldx PAUSED              ; if PAUSED == 0 goto PHONE_CODE

beq PHONE_CODE

jsr SILENCE             ; all four voices off (SILENCE_234 leaves voice 1 alone; A must be 0)

jmp NMI_EXIT            ; common tail of every interrupt handler

;-----------------------------------------------------------------------------

;  PHONE_CODE - the famous cheat: type Big Five's phone number, 213-782-6861, during a

;  game, then a station number with SHIFT, and that station starts at once. The code is

;  checked against the number as printed in the credits text, "(213) 782-6861", skipping

;  ")", the space and "-". A wrong digit starts the count again; holding a key (the same

;  digit repeating) is ignored. Once all 13 positions are matched, SHIFT + digit warps.

;-----------------------------------------------------------------------------

PHONE_CODE:     jsr READ_DIGIT          ; a digit key?

beq PC_EXIT

ldx CHEAT_IDX           ; whole number typed?

cpx #$0D                ; if X >= $0D goto PC_WARP

bcs PC_WARP

tya                     ; digit -> screen code

clc

adc #$10

cmp CHEAT_CODE,x        ; the next digit of the number?

beq PC_MATCH

cmp LASTCHAR            ; the same key still repeating?

beq PC_EXIT

ldx #$FF                ; wrong: start again

PC_MATCH:       sta LASTCHAR

inx

lda CHEAT_CODE,x        ; skip ") " and "-"

cmp #$09                ; if A <> 9 goto PC_DASH

bne PC_DASH

inx

inx

PC_DASH:        cmp #$0D                ; if A <> $0D goto PC_SET

bne PC_SET

inx

PC_SET:         stx CHEAT_IDX

PC_EXIT:        jmp VBI_TAIL            ; end of GAME_VBI

PC_WARP:        ldx #$00                ; code complete: SHIFT held (SKSTAT bit 3 = 0)?

lda (pSKCTL,x)

and #$08                ; test bits $08

bne PC_EXIT

sty STATION             ; station = the digit (0 means 10), and start it

jmp PLAY_STATION        ; build STATION and run it until the turn ends

;-----------------------------------------------------------------------------

;  SILENCE - all four voices off (SILENCE_234 leaves voice 1 alone; A must be 0).

;-----------------------------------------------------------------------------

SILENCE:        lda #$00

tay

sta (pAUDC1),y          ; AUDC1 = A

SILENCE_234:    tay

sta (pAUDC2),y          ; AUDC2 = A

sta (pAUDC3),y          ; AUDC3 = A

sta (pAUDC4),y          ; AUDC4 = A

rts

;-----------------------------------------------------------------------------

;  CHECK_PAUSE - ESC (5200: PAUSE) toggles PAUSED, with a debounce. Not in the demo.

;-----------------------------------------------------------------------------

CHECK_PAUSE:    lda DEMO_PTR+1          ; if DEMO_PTR+1 <> 0 goto PAUSE_RTS

bne PAUSE_RTS

lda KEYCODE

cmp #$1C                ; if A == $1C goto PAUSE_KEY

beq PAUSE_KEY

ldx PAUSE_DLY           ; if PAUSE_DLY == 0 goto PAUSE_RTS

beq PAUSE_RTS

dec PAUSE_DLY

PAUSE_RTS:      rts

PAUSE_KEY:      lda #$FF                ; KEYCODE = $FF

sta KEYCODE

ldy #$1E

ldx PAUSE_DLY

sty PAUSE_DLY

bne PAUSE_RTS

lda #$01                ; toggle

sec

sbc PAUSED

sta PAUSED

rts

;-----------------------------------------------------------------------------

;  JUMP_ARC - vertical movement per frame of a jump: 11 lines up, a pause, 3 up, pause,

;  2 up, pause, then down again; after index $1F Bob simply falls 1 line per frame.

;  (Indexes 30 and 31 read the zeros at the start of BOB_SIZE.)

;-----------------------------------------------------------------------------

JUMP_ARC:       .byte $FF,$FF,$FF,$FF,$FF,$FF,$FF,$FF,$FF,$FF

.byte $FF,$00,$00,$FF,$FF,$FF,$00,$00,$FF,$FF

.byte $00,$00,$00,$01,$01,$00,$00,$01,$01,$01

;-----------------------------------------------------------------------------

;  BOB_SIZE / BOB_SIZE_X - while dying, DYING (1-4) picks the players' width (SIZEP:

;  0 normal, 1 double, 3 quadruple) and an x offset that keeps Bob centred: he throbs.

;-----------------------------------------------------------------------------

BOB_SIZE:       .byte $00,$00,$01,$03,$01

BOB_SIZE_X:     .byte $00,$00,$FC,$F8,$FC

;-----------------------------------------------------------------------------

;  FEET_OFFSETS - for walking/standing frames 0-6, the x offsets of Bob's two feet

;  (the pixels CLAIM_FLOOR looks under).

;-----------------------------------------------------------------------------

FEET_OFFSETS:   .byte $01,$06

.byte $03,$05

.byte $03,$06

.byte $02,$06

.byte $02,$04

.byte $01,$04

.byte $01,$05

;-----------------------------------------------------------------------------

;  Unused.

;-----------------------------------------------------------------------------

UNUSED_END:     .byte $05,$00,$00,$00,$00,$00,$00

;-----------------------------------------------------------------------------

;  The cartridge trailer. On the 400/800:

;    $BFFA start address (RESTART)         $BFFC 0 = a cartridge is present

;    $BFFD option byte $FF: bit 7 "diagnostic cartridge" = jump to the init address

;          straight after RESET, before the OS initialises anything

;    $BFFE init address (COLD_START)

;  On the 5200 the BIOS reads $BFFD = $FF as "skip the title screen" and starts the game

;  through $BFFE: the same bytes serve both machines.

;-----------------------------------------------------------------------------

CART_TRAILER:   .word RESTART

.byte $00,$FF

.word COLD_START